| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0179-v2 |
| --- | --- |

| Title: | CE10.2.2: Integer-MV-based CU-boundary-only OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Chun-Chia Chen, Yu-Cheng Lin, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Xiaoyu Xiu, Yuwen He 9276 Scranton Road, Suite 300, San Diego, CA 92121, USA | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, chunchia.chen, yucheng.lin, yuwen.huang, shawmin.lei} @mediatek.com Yuwen.He@InterDigital.com |
| Source: | MediaTek Inc., InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes complexity reduction methods for overlapped block motion compensation (OBMC). In the proposed Core Experiment 10.2.2 (CE10.2.2), the generation of the prediction blocks from neighbouring blocks is restricted to use integer motion vectors (MVs), resulting in interpolation-free OBMC. Besides, encoder-only lossless complexity reduction methods are developed. To further reduce the worst case complexity, OBMC is disabled for coding units (CUs) smaller than 64 luma samples. As for the syntax, an OBMC flag is signalled at CU level for inter mode. It is reported that OBMC using integer MVs (CE10.2.2) results in -0.37% and -0.39% luma BD-rates with 5% and 6% encoding time increases and 4% and 6% decoding time increases for VTM3.0-RA and VTM3.0-LB, respectively, and the worst case MC complexity of OBMC does not exceed the worst case in VTM3.0 (8x4 or 4x8 bi-prediction).

# Introduction

Based on JVET-L0101 [1], this contribution proposes to further reduce the memory bandwidth and interpolation operations of OBMC. More details are described in the following sections.

# Proposed methods

## Integer-MV-based OBMC

To reduce the memory bandwidth and interpolation operations, the prediction blocks from neighbouring blocks are generated using integer MVs. If the MVs in neighbouring CUs are fractional, they will be converted to integer MVs by rounding them to the nearest integers. Also, to further reduce the complexity of OBMC, the prediction block from a neighbouring bi-prediction block always applies equal weights for weighted averaging regardless of the gbi_index from the neighbouring block.

The number of luma lines and the number of chroma lines for weighted averaging are three and one, respectively. If the CU width/height is less than 8 luma samples, the number of luma lines for weighted averaging at left/top CU boundary is reduced to two.

## CU-boundary-only OBMC

OBMC is only applied at top and left CU boundaries of the current CU.

## Disabling OBMC for small CUs

OBMC is disabled for CUs with sizes smaller than 64 luma samples.

## Disabling OBMC at CTU row boundaries

To save the line buffer for storing prediction blocks from top neighbouring blocks of the above coding tree unit (CTU) row, OBMC at the top CTU row boundaries of the current CTU is disabled.

## OBMC syntax

In sequence parameter set (SPS), sps_obmc_enabled_flag is signalled. When sps_obmc_enabled_flag is true, at the CU level obmc_flag is signalled when the current CU is smaller than or equal to 256 luma samples in area.

## Lossless data reuse

In this proposal, redundant MC operations in JEM OBMC are removed. Two data reuse methods are proposed and described in Sections 2.6.1 and 2.6.2. The first method can reduce encoding time and does not change encoder outputs. The second method can reduce both encoding time and decoding time and does not change encoder and decoder outputs.

### Reuse of prediction blocks from neighbouring blocks at CU boundaries

At the encoder, we can use one buffer to store prediction blocks from neighbouring blocks at top and left CU boundaries. The data stored in the buffer can be shared for different rate-distortion optimization (RDO) processes of CU mode candidates.

### MC merging

When we generate prediction blocks from neighbouring blocks at top and left CU boundaries, if adjacent neighbouring blocks have the same motion information, they can be generated together to save encoding time and decoding time. That is, the MC operations of adjacent neighbouring blocks of the same motion can be merged into one MC operation of a larger block for saving redundant memory access.

## Parallel weighted averaging

The idea is to allow parallel weighted averaging for each sample in the CU and is shown in equations of Figure 1, where the F(x,y) is the final weighted averaging result, the C(x,y) is the current 4x4 block predictor, and the L(x,y)/T(x,y) denotes the prediction block samples from left/top neighbouring block. If the left/top prediction block is unavailable, L(x,y)/T(x,y) is replaced with C(x,y). The w(i) is a weight table.

Figure 1. Parallel weighted averaging

## Skipping OBMC using MV similarity

OBMC may be skipped based on the similarity between the current block MV(s) and the neighbouring block MV(s). Note that this check is performed before converting neighbouring block MVs into integer MVs.

Specifically, each time before MC is invoked using the motion information from a given neighbouring block, the neighbouring block MV(s) is compared to the current block MV(s) based on the following three conditions:

- If each reference list (i.e., either L0/L1 in uni-prediction or both L0 and L1 in bi-prediction) that is used by the neighbouring block is also used for the prediction of the current block;

- The neighbouring block MV(s) and the current block MV(s) point to the same reference picture.

- MV difference(s) between the neighboring block MV(s) and the current block MV(s) is no larger than pre-defined MV difference threshold (both L0 and L1 MVs are checked if bi-pred is used).

If all the three conditions are met, OBMC is not performed. This is applicable to luma and chroma. Additionally, to achieve a better performance/complexity trade-off, two different threshold values i.e., 1.5-pel and 1-pel, are applied when the neighbouring block is uni-predicted or bi-predicted.

# Computational complexity analysis

Table 1-1 shows the ratio of the OBMC complexity to the non-OBMC complexity in the aspects of memory bandwidth and interpolation operations. The left-hand side shows the complexity of the on-the-fly implementation, which generates prediction blocks from neighbouring blocks when the current CU is decoded. The right-hand side shows the complexity of the pre-generation implementation, which generates prediction blocks from neighbouring blocks when the neighbouring block is decoded. If OBMC is implemented by the pre-generation method, additional 1.28KB buffer (0.96KB for luma component and 0.32KB for chroma components) is required for storing prediction blocks from neighbouring blocks. For each implementation method, we use the worst case pattern to calculate the memory bandwidth and interpolation operations. The worst case pattern is shown in Figure 2, where the gray and green regions are the prediction blocks from neighbouring blocks.

Table 1-1. Comparison of the memory bandwidth and interpolation operations between the worst case pattern in Figure 2 and a non-OBMC 8x4 bi-prediction block.

|  | On-the-fly method |  | Pre-generation method |  |
| --- | --- | --- | --- | --- |
|  | Memory bandwidth | Interpolation operations | Memory bandwidth | Interpolation operations |
| Compared with non-OBMC 8x4 bi-prediction block | 0.83 | 1.00 | 1.00 | 1.00 |

Figure 2. Worst case pattern of OBMC in a 16x16 region.

# Computational complexity analysis (additional)

Table 1-2. Comparison of the memory bandwidth between the worst case pattern #1 / #2 and a non-OBMC 8x4 bi-prediction block.

|  | On-the-fly method |  | Pre-generation method |  |
| --- | --- | --- | --- | --- |
|  | Memory bandwidth | Local buffer | Memory bandwidth | Local buffer |
| #1 | 0.88 (A) | 0 | 0.84 (C) | 1.28 KB (E) |
| #2 | 0.74 (B) | 0 | 0.71 (D) | 1.28 KB (E) |

Pattern #1                   Pattern #2

Detailed computation:

(A): 15x11x2x2 + 11x15x2x2 + 15x15x2x2 + 3x8x4 = 2316

2316/256/10.3125 = 0.88

(B): 15x15x2x2 + 11x11x8 + 3x8x4 = 1964

1964/256/10.3125 = 0.74

(C): 15x11x2x2 + 11x15x2x2 + 15x15x2x2 = 2220

2220/256/10.3125 = 0.84

(D): 15x15x2x2 + 11x11x8 =1868

1864/256/10.3125 = 0.71

(E): 128x2x3 + 64x2x2x1 = 1024 samples  1024x10/8 = 1.28KB

# Experiment results

The proposed modifications have been implemented on top of VTM3.0 [2]. The common test conditions [3] are used for evaluation. The CE10.2.2 results are summarized in Table 2.

Table2.  Results of CE10.2.2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.69% | -0.69% | 104% | 103% |
| Class A2 | -0.37% | -0.99% | -0.93% | 105% | 101% |
| Class B | -0.35% | -0.94% | -0.98% | 105% | 105% |
| Class C | -0.54% | -0.95% | -0.93% | 105% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.37% | -0.91% | -0.90% | 105% | 104% |
| Class D | -0.43% | -0.78% | -0.73% | 105% | 108% |
| Class F | -0.04% | -0.32% | -0.23% | 104% | 106% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.57% | -0.38% | 107% | 105% |
| Class C | -0.48% | -0.54% | -0.50% | 107% | 109% |
| Class E | -0.37% | -0.52% | -0.64% | 105% | 106% |
| Overall | -0.39% | -0.55% | -0.48% | 106% | 106% |
| Class D | -0.17% | 0.04% | 0.78% | 107% | 111% |
| Class F | 0.10% | -0.21% | 0.31% | 106% | 105% |

# Conclusions

In this proposal, integer-MV-based CU-boundary-only OBMC is proposed. The results of the proposed OBMC are -0.37% and -0.39% luma BD-rate with 5% and 6% encoding time increases and 4% and 6% decoding time increases for VTM3.0-RA and VTM3.0-LB, respectively.

# References

[1] Z.-Y. Lin, C.-C. Chen, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE10.2.1: OBMC,” Document of Joint Video Experts Team, JVET-L0101.

[2] VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Oct 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).