| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0180-v1 |
| --- | --- |

| Title: | CE10.2.3: Subblock OBMC with uni-prediction-based OBMC at CU boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Cheng Lin, Chun-Chia Chen, Chih-Wei Hsu, Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei, No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yucheng.lin, chunchia.chen, cw.hsu, zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment 10.2.3 (CE10.2.3) on subblock overlapped block motion compensation (OBMC). In CE10.2.3, except for affine mode of uni-prediction, the coding-unit-boundary-only OBMC from CE10.2.1 in JVET-M0178 is applied as it is. For affine mode of uni-prediction, OBMC is applied at top, left, and right subblock boundaries for each subblock of the coding unit (CU), and the number of OBMC lines for weighted averaging at subblock boundaries is set to one. It is reported that CE10.2.3 results in -0.39% and -0.39% luma BD-rates with 105% and 107% encoding time increases and 104% and 108% decoding time increases for VTM3.0-RA and VTM3.0-LB, respectively, and the worst case motion compensation (MC) complexity of CE10.2.3 OBMC does not exceed the worst case in VTM3.0 (i.e., 8x4 or 4x8 bi-prediction).

# Proposed OBMC

Based on JVET-L0101 [1], this contribution proposes to further apply subblock OBMC on top of CE10.2.1 [2]. The CU-boundary-only OBMC from CE10.2.1 is applied as it is except for affine mode of uni-prediction.

As shown in Figure 1, subblock OBMC is applied to affine mode of uni-prediction as follows.

For each subblock of the current CU:

- For the leftmost sample column except for the top sample of the leftmost sample column:

  - OBMC from the left neighbouring subblock is performed when the left neighbouring subblock is available. If the boundary between the left neighbouring subblock and the current subblock is a CU boundary, the CU-boundary-only OBMC in CE10.2.1 is applied; otherwise, a straightforward subblock OBMC as described later is applied.

- For the top sample row:

  - OBMC from the top neighbouring subblock OBMC is performed when the top neighbouring subblock is available. If the boundary between the top neighbouring subblock and the current subblock is a CU boundary, the CU-boundary-only OBMC in CE10.2.1 is applied; otherwise, a straightforward subblock OBMC as described later is applied.

- For the rightmost sample column except for the top sample of the rightmost sample column:

  - OBMC from the right neighbouring subblock is performed. If the right neighbouring subblock belongs to a different CU, OBMC from the right neighbouring subblock is skipped; otherwise, a straightforward subblock OBMC as described later is applied.

OBMC from left neighbouring subblock

OBMC from right neighbouring subblock

OBMC from top neighbouring subblock

Figure 1.  Subblock OBMC

Straightforward subblock OBMC:

- For each to-be-predicted sample, motion compensation (MC) is performed using both the current subblock motion vector (MV) and the neighbouring subblock MV to generate two prediction samples.

- To generate the final prediction sample, a weighted averaging process is applied, where 0.75 and 0.25 are used as weights for the prediction sample generated by using the current subblock MV and the prediction sample generated by using the neighbouring subblock MV, respectively, and a rounding (+0.5 and then truncate to integer) is used.

# Complexity comparison

In order to filter one sample, an eight-tap filter requires eight multiplications and seven additions, which is denoted as one unit of filter operation for comparing filter operation. In the worst case of VTM3.0 (8x4 or 4x8 bi-prediction), 120 units of filter operation per 4x4 block are required. When the subblock is not at the top or left CU boundary of the current CU, the worst case of the proposed method requires 102 units of filter operation per 4x4 block. As for MC memory bandwidth, the VTM3.0 worst case is 165 samples per 4x4 block. When the subblock is not at the top or left CU boundary of the current CU, the proposed method worst case is 156 samples per 4x4 block. Table 1 shows the comparison.

Table 1.  Comparison of worst case complexity

|  | VTM3.0 worst case | Proposed method worst case when the subblock is not at the top or left CU boundary of the current CU |
| --- | --- | --- |
| Filter operation per 4x4 block | 120 units of filter operation | 102 units of filter operation |
| MC mem. bandwidth per 4x4 block | 165 samples | 156 samples |

When the subblock is at the top or let CU boundary of the current CU, the worst case of this proposal does not exceed the VTM3.0 worst case either, as can be seen from CE10.2.1 in JVET-M0178.

# Experiment results

The proposed modifications have been implemented on top of VTM3.0 [3]. The common test conditions [4] are used for evaluation. The CE10.2.3 results are summarized in Table 2.

Table 2.  Results of CE10.2.3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.33% | -0.40% | 105% | 100% |
| Class A2 | -0.54% | -0.65% | -0.59% | 105% | 101% |
| Class B | -0.36% | -0.62% | -0.77% | 105% | 104% |
| Class C | -0.48% | -0.70% | -0.67% | 106% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.39% | -0.59% | -0.63% | 105% | 104% |
| Class D | -0.49% | -0.81% | -0.80% | 105% | 107% |
| Class F (optional) | -0.21% | -0.40% | -0.34% | 104% | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.75% | -0.67% | 108% | 107% |
| Class C | -0.52% | -0.74% | -0.63% | 107% | 109% |
| Class E | -0.33% | -0.52% | -1.11% | 107% | 108% |
| Overall | -0.39% | -0.69% | -0.76% | 107% | 108% |
| Class D | -0.63% | -0.57% | -0.31% | 107% | 112% |
| Class F (optional) | -0.37% | -0.45% | -0.49% | 106% | 107% |

# Conclusions

In this proposal, uni-prediction-based subblock OBMC is proposed. The proposed OBMC results in -0.39% and -0.39% luma BD-rates with 105% and 107% encoding time increases and 104% and 108% decoding time increases for VTM3.0-RA and VTM3.0-LB, respectively.

# References

- Z.-Y. Lin, C.-C. Chen, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE10.2.1: OBMC,” Document of Joint Video Experts Team, JVET-L0101.

- Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, C.-C. Chen, Y.-C. Lin, Y.-W. Huang, S.-M. Lei, “CE10.2.1: Uni-prediction-based CU-boundary-only OBMC,” Document of Joint Video Experts Team, JVET-M0178.

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).