| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0187-v1 |
| --- | --- |

| Title: | CE11-related: Long deblocking filters with reduced line buffer requirement and enhanced parallel processing accessibility |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two aspects of modifications on top of the CE11.1.3 long deblocking filters in JVET-M0186. In both VTM3.0 and CE11.1.3, deblocking can be performed at 8x8 grids and is skipped at non-8x8 grids. The maximum numbers of to-be-read luma samples on one side of an edge during deblocking are four and eight for VTM3.0 and CE11.1.3, respectively. The maximum numbers of to-be-modified luma samples on one side of an edge during deblocking are three and seven for VTM3.0 and CE11.1.3, respectively. In the first aspect, to keep the number of luma sample lines needed to be stored in the line buffer as low as that in VTM3.0, during vertical filtering across horizontal coding tree unit (CTU) boundaries, the maximum number of to-be-read luma samples above the horizontal CTU boundary is reduced from eight to four, and in addition, the maximum number of to-be-modified luma samples above the horizontal CTU boundary is set to three. In the second aspect, to better support parallel deblocking at multiple parallel subblock boundaries on 8x8 grids, during horizontal filtering across vertical subblock boundaries and vertical filtering across horizontal subblock boundaries, if a subblock coding mode is chosen on one side of the subblock boundary, the number of to-be-read luma samples on that side of the subblock boundary is reduced from eight to four, and in addition, the maximum number of to-be-modified luma samples on that side of the subblock boundary is set to three. It is reported that the proposed deblocking results in negligible changes in BD-rate and encoding time, and the decoding time increase is 3% for VTM3.0 common test condition (CTC) and for VTM3.0 CTC with adaptive loop filter (ALF) off.

# Introduction

In both VTM3.0 and CE11.1.3 [1], deblocking can be performed at 8x8 grids and is skipped at non-8x8 grids. The maximum numbers of to-be-read luma samples on one side of an edge during deblocking are four and eight for VTM3.0 and CE11.1.3, respectively. The maximum numbers of to-be-modified luma samples on one side of an edge during deblocking are three and seven for VTM3.0 and CE11.1.3, respectively. Although the long deblocking filters in CE11.1.3 can improve visual quality, it increases the line buffer requirement from storing four to eight in units of luma sample lines of picture width. Moreover, parallel deblocking at two parallel subblock boundaries of distance equal to eight luma samples cannot be achieved because of the data dependency caused by the long deblocking filters.

HEVC deblocking filters (DF) consist of two main parts: filtering decisions and filtering processes. The filtering decisions are to determine filter type (e.g., no filtering, weak filtering, or strong filtering) for block edge. The filtering processes are to filter samples at both sides of block edge based on filtering decisions. However, as popular picture resolutions become higher and higher, HEVC DF cannot effectively reduce blocky artifacts at large block boundaries in high resolution videos (e.g., UHD). In this contribution, longer filters are added to DF based on CE11.1.3 with solutions to the two issues mentioned in the previous paragraph.

Some definitions in this contribution are described as follows. The left side and right side of a vertical block edge are called p-side and q-side, respectively. The upper side and lower side of a horizontal block edges are called p-side and q-side, respectively. In addition, each block edge is of 4-sample length by definition, and pij and qij (i=0, 1, 2, or 3; and j=0, 1, 2, …) represent the p-side and q-side samples on the ith line perpendicular to the block edge, j equal to 0 means the sample is adjacent to the block edge, and a larger j means the sample is at a larger distance from the block edge. The coding block (CB) length refers to CB width and CB height for horizontal filtering across vertical block edge and vertical filtering across horizontal block edge, respectively. The adjacent block edges refer to the left and right connected block edges for horizontal edge, and the upper and lower connected block edges for vertical edge.

# Proposed deblocking filters

## Flowchart of filtering decisions

The filtering decisions to determine no/weak/strong filtering and the weak filtering are exactly the same as those in HEVC. To improve DF, the strong filtering is extended. Three long strong filter sets, named as R8W7-ultra, R8W7-strong, and R4W3-strong, are added for strong filtering. The corresponding details of long strong filter sets are described in Section 2.5. As illustrated in Figure 1 to derive filtering decisions for each block edge, if the answer to “Whether to apply strong filtering?” is yes, “Whether to apply long strong filtering?” is further checked. The corresponding details of long strong filtering decision are described in Section 2.3. If the answer to “Whether to apply long strong filtering” is yes, one of the three long strong filter sets is selected for p-side, and one of the three long strong filter sets is selected for q-side. The selected long strong filter set for p-side and the selected long strong filter set for q-side can be different. The corresponding details of long strong filter set selection are described in Section 2.4.

Figure 1.  The proposed flowchart of filtering decisions

## Condition checks

Two conditions are first defined as follows.

[Condition P]:
p-side and q-side belong to different CTUs during vertical filtering or
p-side belongs to a coding unit (CU) with a subblock coding mode, i.e., subblock-based temporal motion vector prediction (SbTMVP) or affine.

[Condition Q]:
q-side belongs to a CU with a subblock coding mode.

## Long strong filtering decision

This subsection describes the answer to “Whether to apply long strong filtering?” in Figure 1. When all the following conditions are true, the answer is yes. Otherwise, the answer is no (i.e., apply HEVC strong filtering for p-side and q-side).

- The slice type is inter.

- The CB length at p-side and the CB length at q-side are either larger than or equal to 16.

- Both of the two inter-side difference values representing difference between p-side and q-side are larger than a threshold, inter_side_diff_th.

- Both of the two intra-side difference values representing variation within p-side plus variation within q-side are smaller than a threshold, intra_side_diff_th.

- Any one of adjacent block edges also satisfies all the above four conditions.

The two inter-side differences are described in Section 2.3.1, the two intra-side differences are described in Section 2.3.2.

### Inter-side difference

The two inter-side difference values, InterSideDiff0 and InterSideDiff3, are defined in Equation 1.

p0i-q1i+p1i-q0ia??1, where i = 0 or 3                       (1)

That is, for each block edge, only the first and fourth lines are used to obtain the inter-side difference. The threshold value, inter_side_diff_th, depends on quantization parameter (QP) and is shown in Figure 2.

Figure 2.  Derivation of inter_side_diff_th from QP

### Intra-side difference

The two intra-side difference values, IntraSideDiff0 and IntraSideDiff3, are defined in the following description. The threshold value, intra_side_diff_th, is set to theparameter divided by 2.

If Condition P is satisfied and Condition Q is not satisfied, the intra-side difference values of the current block edge are:

, where i = 0 or 3                                  (2)

Else if Condition Q is satisfied and Condition P is not satisfied, the intra-side difference values of the current block edge are:

, where i = 0 or 3                                  (3)

Else if both Condition P and Condition Q are satisfied, the intra-side difference values of the current block edge are:

, where i = 0 or 3                                          (4)

Else, the intra-side difference values of the current block edge are:

, where i = 0 or 3                    (5)

## Long strong filter set selection

When the long strong filtering decision output is yes (i.e., the answer to “Whether to apply long strong filtering?” in Figure 1 is yes), the long strong filter set selection (i.e., “Select long strong filter set for p-side and q-side separately” in Figure 1) is activated to select one of the three long strong filter sets for p-side and one of the three long strong filter sets for q-side. The selection based on the p-side and q-side length of the current block edge is described as follows, where lenp and lenq denote the CB length at p-side and the CB length at q-side, respectively.

- If “lenp ≥ 32 and lenq ≥ 32”, R8W7-ultra or R8W7-strong filtering is selected as follows:

  - If any one of adjacent block edges of the current block edge also satisfies “lenp ≥ 32 and lenq ≥ 32”,
  If Condition P is not satisfied and Condition Q is not satisfied,
    R8W7-ultra filtering is applied to both p-side and q-side of the current block edge.
  Else if Condition P is not satisfied and Condition Q is satisfied
    R8W7-strong filtering is applied to p-side of the current block edge.
  Else if Condition P is satisfied and Condition Q is not satisfied
    R8W7-strong filtering is applied to q-side of the current block edge.

  - Otherwise (i.e., “lenp ≥ 32 and lenq ≥ 32” is only true for the current block edge), R8W7-strong filtering is applied to p-side of the current block edge when Condition P is not satisfied, and R8W7-strong is applied to q-side of the current block edge when Condition Q is not satisfied.

- If “lenp > 16 and lenq = 16”, R8W7-strong filtering is applied to p-side of the current block edge when Condition P is not satisfied, and R4W3-strong filtering is applied to q-side of the current block edge regardless of Condition P and Condition Q.

- If “lenp = 16 and lenq > 16”, R4W3-strong filtering is applied to p-side of the current block edge regardless of Condition P and Condition Q, and R8W7-strong filtering is applied to q-side of the current block edge when Condition Q is not satisfied.

- If “lenp = 16 and lenq = 16”, R4W3-strong filtering is applied to both p-side and q-side of the current block edge.

- If Condition P is satisfied, R4W3-strong filtering is applied to p-side of the current block edge.

- If Condition Q is satisfied, R4W3-strong filtering is applied to q-side of the current block edge.

Figure 3 and Figure 4 give two examples for this section.

Figure 3.  A vertical filtering example at horizontal CTU boundary.

Figure 4.  A horizontal filtering example at subblock boundary.

## Long strong filter sets

The filter set naming rule is explained here. “Applying RxWy-fs filtering (x & y are numbers, R stands for read, W stands for write, fs stands for filtering strength and can be ultra or strong) to z-side (z can be p or q)” means at most x samples at z-side are read and y samples at z-side are written (i.e., modified).

After applying the selected long strong filter set to one side of block edge, a clipping function is applied to filtered sample values, which is the same as that in HEVC and bounds the changes of sample values within -2tc to 2tc.

Three long strong filter sets are described in the rest of this subsection.

### R8W7-ultra filtering

If R8W7-ultra filtering is applied to p-side, filtered sample values from  to  are as follows:

If R8W7-ultra filtering is applied to q-side, filtered sample values from  to  are as follows:

### R8W7-strong filtering

If R8W7-strong filtering is applied to p-side, filtered sample values from  to  are as follows:

If R8W7-strong filtering is applied to q-side, filtered sample values from  to  are as follows:

### R4W3-strong filtering

If R4W3-strong filtering is applied to p-side, filtered sample values from  to  are as follows:

If R4W3-strong filtering is selected at q-side, filtered sample values from  to  are as follows:

## Complexity evaluation

Complexity of the proposed method is evaluated based on the following criteria described in JVET-L1031 [2].

- Number of samples from the block boundary that can be modified by deblocking (P+Q):

  - 7+7, 7+3, 3+7, or 3+3

- Number of samples from the block boundary that are needed for deblocking decisions:

  - 16 samples are used (both 8 samples from p-side and q-side)

- Number of filtering operations:

  - Maximum 266 filtering operations for one line, including 168 additions, 34 multiplications, 28 comparisons, and 36 shift operations

  - Maximum 29 filtering decision operations for 8-sample boundary, including 12 additions, 0 multiplications, 15 comparisons, and 2 shift operations

- Number of line buffers:

  - 4 lines

- Parallelization capabilities:

  - All edges overlapping with 8 x 8 grid can be deblocked in parallel.

- Only applied to luma filtering. Chroma filtering is not changed.

# Experimental results

## Objective results

The proposed method has been implemented on VTM3.0 [3]. The simulations were performed following the JVET common test conditions (CTC) [4]. The objective results for CTC and for CTC with ALF switched off are summarized in the Table 1 and Table 2, respectively.

Table 1.  Results of the proposed DF; anchor: VTM3.0; test: anchor + proposed DF

|  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over VTM-3.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | 0.00% |  | 0.00% |  | 0.00% | 100% |  | 101% |  |
| Class A2 | 0.00% |  | 0.00% |  | 0.00% | 101% |  | 105% |  |
| Class B | 0.00% |  | 0.00% |  | 0.00% | 100% |  | 102% |  |
| Class C | 0.00% |  | 0.00% |  | 0.00% | 100% |  | 102% |  |
| Class E | 0.00% |  | 0.00% |  | 0.00% | 100% |  | 102% |  |
| Overall | 0.00% |  | 0.00% |  | 0.00% | 100% |  | 102% |  |
| Class D | 0.00% |  | 0.00% |  | 0.00% | 100% |  | 102% |  |
| Class F | 0.00% |  | 0.00% |  | 0.00% | 100% |  | 102% |  |
|  | Random access Main10 |  |  |  |  |  |  |  |  |
|  |  |  |  | Over VTM-3.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | -0.01% |  | -0.03% |  | -0.03% | 100% |  | 100% |  |
| Class A2 | -0.02% |  | 0.00% |  | 0.00% | 100% |  | 100% |  |
| Class B | -0.01% |  | -0.02% |  | 0.00% | 100% |  | 102% |  |
| Class C | 0.00% |  | 0.01% |  | 0.01% | 100% |  | 107% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% |  | -0.01% |  | 0.00% | 100% |  | 102% |  |
| Class D | 0.00% |  | 0.00% |  | 0.00% | 101% |  | 105% |  |
| Class F | -0.01% |  | 0.00% |  | 0.00% | 101% |  | 103% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  |  |  |  | Over VTM-3.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% |  | -0.17% |  | 0.01% | 100% |  | 104% |  |
| Class C | 0.03% |  | 0.04% |  | 0.08% | 100% |  | 103% |  |
| Class E | 0.02% |  | -0.07% |  | -0.34% | 101% |  | 108% |  |
| Overall | 0.03% |  | -0.08% |  | -0.05% | 100% |  | 104% |  |
| Class D | 0.00% |  | -0.09% |  | -0.14% | 101% |  | 108% |  |
| Class F | -0.10% |  | 0.09% |  | -0.20% | 100% |  | 103% |  |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |
|  |  |  |  | Over VTM-3.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% |  | 0.18% |  | 0.26% | 100% |  | 104% |  |
| Class C | 0.02% |  | -0.05% |  | -0.06% | 100% |  | 102% |  |
| Class E | -0.10% |  | -0.12% |  | -0.18% | 100% |  | 103% |  |
| Overall | -0.02% |  | 0.03% |  | 0.04% | 100% |  | 103% |  |
| Class D | -0.01% |  | 0.11% |  | 0.13% | 100% |  | 103% |  |
| Class F | 0.09% |  | -0.56% |  | -0.65% | 100% |  | 103% |  |

Table 2.  Results of the proposed DF; anchor: VTM3.0 with ALF off; test: anchor + proposed DF

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-3.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.02% | -0.01% | 100% | 100% |
| Class A2 | -0.01% | 0.01% | 0.00% | 100% | 97% |
| Class B | 0.00% | 0.01% | 0.00% | 100% | 103% |
| Class C | 0.00% | -0.01% | 0.01% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class F | 0.00% | 0.01% | 0.00% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.04% | -0.18% | 100% | 103% |
| Class C | -0.02% | 0.14% | -0.14% | 100% | 103% |
| Class E | -0.02% | 0.30% | -0.20% | 101% | 103% |
| Overall | -0.02% | 0.11% | -0.17% | 100% | 103% |
| Class D | -0.02% | -0.13% | 0.02% | 100% | 105% |
| Class F | 0.01% | 0.09% | -0.36% | 100% | 103% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.16% | -0.03% | 100% | 104% |
| Class C | 0.01% | 0.07% | -0.25% | 100% | 103% |
| Class E | -0.02% | 0.02% | -0.13% | 101% | 104% |
| Overall | -0.01% | -0.04% | -0.13% | 100% | 103% |
| Class D | -0.01% | 0.13% | -0.18% | 101% | 104% |
| Class F | -0.05% | -0.21% | -0.12% | 102% | 101% |

# Conclusions

This contribution modifies CE11.1.3 long deblocking filters with reduced line buffer requirement and enhanced parallel processing accessibility. The proposed method results in very minor changes in BD-rate and encoding time and slightly increases decoding time while the two issues are solved.

# References

- C.-M. Tsai, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE11.1.3: Long deblocking filters,” Document of Joint Video Experts Team, JVET-M0186.

- A. Norkin, A.M. Kotra, “Description of Core Experiment 11 (CE11): Deblocking,” Document of Joint Video Experts Team, JVET-L1031.

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).