| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0192_WD |
| --- | --- |

| Title: | Proposed WD for CE2-related: MV Derivation for Affine Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anish Tamse Min Woo Park Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | tamse.anish@samsung.com m.w.park@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Proposed Working Draft Changes

This document provides the proposed text of VVC Working Draft. The changes are highlighted in green and strikethrough.

Method 1:

#### Derivation process for motion vector arrays from affine control point motion vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the luma coding block,

- the number of control point motion vectors numCpMv,

- the control point motion vectors cpMvLX[ cpIdx ], with cpIdx = 0..numCpMv − 1 and X being 0 or 1,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY.

Outputs of this process are:

- the luma subblock motion vector array mvLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1,

- the chroma subblock motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1.

The following assignments are made for x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1:

CpMvLX[ x ][ y ][ 0 ] = cpMvLX[ 0 ]		(8-622)

CpMvLX[ x ][ y ][ 1 ] = cpMvLX[ 1 ]		(8-623)

CpMvLX[ x ][ y ][ 2 ] = cpMvLX[ 2 ]		(8-624)

The variables log2CbW and log2CbH are derived as follows:

log2CbW = Log2( cbWidth )		(8-625)

log2CbH = Log2( cbHeight )		(8-626)

The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = cpMvLX[ 0 ][ 0 ] << 7		(8-627)

mvScaleVer = cpMvLX[ 0 ][ 1 ] << 7		(8-628)

dHorX = ( cpMvLX[ 1 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbW ) 	(8-629)

dVerX = ( cpMvLX[ 1 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbW ) 	(8-630)

The variables dHorY and dVerY are derived as follows:

- If numCpMv is equal to 3, the following applies:

dHorY = ( cpMvLX[ 2 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbH ) 	(8-631)

dVerY = ( cpMvLX[ 2 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbH ) 	(8-632)

- Otherwise ( numCpMv is equal to 2), the following applies:

dHorY = − dVerX		(8-633)

dVerY = dHorX		(8-634)

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = 2 + ( xSbIdx << 2 )		(8-635)

yPosCb = 2 + ( ySbIdx << 2 )		(8-636)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-637)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-638)

- The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The average luma motion vector mvAvgLX is derived as follows:

mvAvgLX = ( mvLX[ ( xSbIdx >> 1 << 1) ][ (ySbIdx>>1<<1) ] + 	
	mvLX[ ( xSbIdx >> 1 << 1) + 1 ][ ( ySbIdx >> 1 << 1) ] + 	 (8-639)
	mvLX[ ( xSbIdx >> 1 << 1 ) ][ ( ySbIdx >> 1 << 1) + 1 ] + 	
	mvLX[ ( xSbIdx >> 1 << 1 ) + 1 ][ (ySbIdx>>1<<1) + 1 ] + 2 ) >> 2

- The derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvAvgLX as input, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.

Method 2:

#### Derivation process for motion vector arrays from affine control point motion vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the luma coding block,

- the number of control point motion vectors numCpMv,

- the control point motion vectors cpMvLX[ cpIdx ], with cpIdx = 0..numCpMv − 1 and X being 0 or 1,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY.

Outputs of this process are:

- the luma subblock motion vector array mvLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1,

- the chroma subblock motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1.

The following assignments are made for x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1:

CpMvLX[ x ][ y ][ 0 ] = cpMvLX[ 0 ]		(8-622)

CpMvLX[ x ][ y ][ 1 ] = cpMvLX[ 1 ]		(8-623)

CpMvLX[ x ][ y ][ 2 ] = cpMvLX[ 2 ]		(8-624)

The variables log2CbW and log2CbH are derived as follows:

log2CbW = Log2( cbWidth )		(8-625)

log2CbH = Log2( cbHeight )		(8-626)

The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = cpMvLX[ 0 ][ 0 ] << 7		(8-627)

mvScaleVer = cpMvLX[ 0 ][ 1 ] << 7		(8-628)

dHorX = ( cpMvLX[ 1 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbW ) 	(8-629)

dVerX = ( cpMvLX[ 1 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbW ) 	(8-630)

The variables dHorY and dVerY are derived as follows:

- If numCpMv is equal to 3, the following applies:

dHorY = ( cpMvLX[ 2 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbH ) 	(8-631)

dVerY = ( cpMvLX[ 2 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbH ) 	(8-632)

- Otherwise ( numCpMv is equal to 2), the following applies:

dHorY = − dVerX		(8-633)

dVerY = dHorX		(8-634)

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = 2 + ( xSbIdx << 2 )		(8-635)

yPosCb = 2 + ( ySbIdx << 2 )		(8-636)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-637)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-638)

- The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The average luma motion vector mvAvgLX is derived as follows:

mvAvgLX = ( mvLX[ ( xSbIdx >> 1 << 1) ][ (ySbIdx>>1<<1) ] + 	
	mvLX[ ( xSbIdx >> 1 << 1) + 1 ][ ( ySbIdx >> 1 << 1) ] + 	 (8-639)
	mvLX[ ( xSbIdx >> 1 << 1 ) ][ ( ySbIdx >> 1 << 1) + 1 ] + 	
	mvLX[ ( xSbIdx >> 1 << 1 ) + 1 ][ (ySbIdx>>1<<1) + 1 ] + 21 ) >> 21

- The derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvAvgLX as input, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.

Method 3:

#### Derivation process for motion vector arrays from affine control point motion vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the luma coding block,

- the number of control point motion vectors numCpMv,

- the control point motion vectors cpMvLX[ cpIdx ], with cpIdx = 0..numCpMv − 1 and X being 0 or 1,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY.

Outputs of this process are:

- the luma subblock motion vector array mvLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1,

- the chroma subblock motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1.

The following assignments are made for x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1:

CpMvLX[ x ][ y ][ 0 ] = cpMvLX[ 0 ]		(8-622)

CpMvLX[ x ][ y ][ 1 ] = cpMvLX[ 1 ]		(8-623)

CpMvLX[ x ][ y ][ 2 ] = cpMvLX[ 2 ]		(8-624)

The variables log2CbW and log2CbH are derived as follows:

log2CbW = Log2( cbWidth )		(8-625)

log2CbH = Log2( cbHeight )		(8-626)

The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = cpMvLX[ 0 ][ 0 ] << 7		(8-627)

mvScaleVer = cpMvLX[ 0 ][ 1 ] << 7		(8-628)

dHorX = ( cpMvLX[ 1 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbW ) 	(8-629)

dVerX = ( cpMvLX[ 1 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbW ) 	(8-630)

The variables dHorY and dVerY are derived as follows:

- If numCpMv is equal to 3, the following applies:

dHorY = ( cpMvLX[ 2 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbH ) 	(8-631)

dVerY = ( cpMvLX[ 2 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbH ) 	(8-632)

- Otherwise ( numCpMv is equal to 2), the following applies:

dHorY = − dVerX		(8-633)

dVerY = dHorX		(8-634)

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = 2 + ( xSbIdx << 2 )		(8-635)

yPosCb = 2 + ( ySbIdx << 2 )		(8-636)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-637)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-638)

- The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The average luma motion vector mvAvgLX is derived as follows:

mvAvgLX = ( mvLX[ ( xSbIdx >> 1 << 1) ][ (ySbIdx>>1<<1) ] + 	
	mvLX[ ( xSbIdx >> 1 << 1) + 1 ][ ( ySbIdx >> 1 << 1) ] + 	 (8-639)
	mvLX[ ( xSbIdx >> 1 << 1 ) ][ ( ySbIdx >> 1 << 1) + 1 ] + 	
	mvLX[ ( xSbIdx >> 1 << 1 ) + 1 ][ (ySbIdx>>1<<1) + 1 ] + 2 ) >> 2

mvAvgLX[ 0 ] = ( mvAvgLX[ 0 ] >= 0 ? ( mvAvgLX[ 0 ] + 2) >> 2 : 	
				− ( (− mvX[ 0 ] + 2 ) >>2 ) )	(8-640)

mvAvgLX[ 1 ] = ( mvAvgLX[ 1 ] >= 0 ? ( mvAvgLX[ 1 ] + 2) >> 2 : 	
				− ( (− mvX[ 1 ] + 2 ) >> 2 ) )	(8-641)

- The derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvAvgLX as input, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.

Method 4:

#### Derivation process for motion vector arrays from affine control point motion vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the luma coding block,

- the number of control point motion vectors numCpMv,

- the control point motion vectors cpMvLX[ cpIdx ], with cpIdx = 0..numCpMv − 1 and X being 0 or 1,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY.

Outputs of this process are:

- the luma subblock motion vector array mvLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1,

- the chroma subblock motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1.

The following assignments are made for x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1:

CpMvLX[ x ][ y ][ 0 ] = cpMvLX[ 0 ]		(8-622)

CpMvLX[ x ][ y ][ 1 ] = cpMvLX[ 1 ]		(8-623)

CpMvLX[ x ][ y ][ 2 ] = cpMvLX[ 2 ]		(8-624)

The variables log2CbW and log2CbH are derived as follows:

log2CbW = Log2( cbWidth )		(8-625)

log2CbH = Log2( cbHeight )		(8-626)

The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = cpMvLX[ 0 ][ 0 ] << 7		(8-627)

mvScaleVer = cpMvLX[ 0 ][ 1 ] << 7		(8-628)

dHorX = ( cpMvLX[ 1 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbW ) 	(8-629)

dVerX = ( cpMvLX[ 1 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbW ) 	(8-630)

The variables dHorY and dVerY are derived as follows:

- If numCpMv is equal to 3, the following applies:

dHorY = ( cpMvLX[ 2 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbH ) 	(8-631)

dVerY = ( cpMvLX[ 2 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbH ) 	(8-632)

- Otherwise ( numCpMv is equal to 2), the following applies:

dHorY = − dVerX		(8-633)

dVerY = dHorX		(8-634)

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = 2 + ( xSbIdx << 2 )		(8-635)

yPosCb = 2 + ( ySbIdx << 2 )		(8-636)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-637)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-638)

- The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The average luma motion vector mvAvgLX is derived as follows:

mvAvgLX = ( mvLX[ ( xSbIdx >> 1 << 1) ][ (ySbIdx>>1<<1) ] + 	
	mvLX[ ( xSbIdx >> 1 << 1) + 1 ][ ( ySbIdx >> 1 << 1) ] + 	 (8-639)
	mvLX[ ( xSbIdx >> 1 << 1 ) ][ ( ySbIdx >> 1 << 1) + 1 ] + 	
	mvLX[ ( xSbIdx >> 1 << 1 ) + 1 ][ (ySbIdx>>1<<1) + 1 ] + 2 ) >> 2

mvAvgLX[ 0 ] = ( mvAvgLX[ 0 ] >= 0 ? ( mvAvgLX[ 0 ] + 1) >> 1 : 	
				− ( (− mvX[ 0 ] + 1 ) >>1 ) )	(8-640)

mvAvgLX[ 1 ] = ( mvAvgLX[ 1 ] >= 0 ? ( mvAvgLX[ 1 ] + 1) >> 1 : 	
				− ( (− mvX[ 1 ] + 1 ) >> 1 ) )	(8-641)

- The derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvAvgLX as input, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).