| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET- M0193-v1 |
| --- | --- |

| Title: | CE4-related: Pairwise Average Candidate Reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anish Tamse Min Woo Park Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | tamse.anish@samsung.com m.w.park@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to reduce the number of pairwise average candidates added in the merge list. There are up to six pairwise average candidates defined during the generation of merge candidate list. However as shown in this proposal, it is never possible for some of the candidates to be used in the list generation. Also, other candidates that are used in the list generation, don’t improve the performance but add additional complexity for generation of the list. This contribution proposes several options to reduce the number of pairwise average candidates. The performance change by just using only one pairwise average candidate when tested on top of VTM3.0 under RA conditions is 0.01% for Y, 0.03% for U and 0.02% for V with no change in runtime.

# Introduction

In VVC [1], pairwise average candidates are introduced as a better alternative to the combination candidates of HEVC. The number of total allowed pairwise candidates is six. The pairwise average candidates are defined based on the list shown below.

{(0, 1), (0, 2), (1, 2), (0, 3), (1, 3), (2, 3)}

Here numbers denote the merge indices for the merge candidate list. If any one of the candidates denoted in the pair is not available, the averaging cannot be performed and the candidate is not used.

Below we analyze how many pairwise average candidate can be added to the merge list.

| PAC = Pairwise Average Candidates |  |  |  |
| --- | --- | --- | --- |
| Candidates present before PAC | Total PAC possible | Total PAC that can be added to the list | Comments |
| 1 | 0 | 0 | No PAC can be generated |
| 2 | 1 | 1 | 1 PAC can be generated and added |
| 3 | 3 | 3 | 3 PAC can be generated and added |
| 4 | 6 | 2 | 6 PAC can be generated but only 2 can be added as merge list would get full |
| 5 | 6 | 1 | 6 PAC can be generated but only 1 can be added as merge list would get full |
| 6 | 6 | 0 | 6 PAC can be generated but none can be added as merge list is already full |

As can be seen from the table above, only the first three candidates can be possibly utilized for a merge list of size 6. The same holds for a merge list of size 7 as well. Hence to simplify the pairwise average candidate construction process, it is proposed to reduce the number of candidate allowed. Three options are proposed in this contribution to this effect.

# Proposed Methods

The proposed options are to simply reduce the number of pairwise average candidates. More specifically, instead of the existing six possible combinations for pairwise average candidates, the number of possible combinations defined are reduced to a smaller number. This reduces the computations required for constructing the merge list, and removes the unused candidates existing in the draft text.

- Method 1) Allow only the first pairwise average candidates

- Method 2) Allow only the first two pairwise average candidates

- Method 3) Allow only the first three pairwise average candidate

# Experimental Results

The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Table 1 shows summary of all results.

Table 1. Summary of all results

|  | Test | BD-Rate (Y) | BD-Rate (U) | BD-Rate (V) | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| RA | Method 1 | 0.01% | 0.03% | 0.02% | 101% | 100% |
|  | Method 2 | -0.01% | 0.01% | 0.02% | 101% | 101% |
|  | Method 3 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| LDB | Method 1 | 0.00% | -0.09% | 0.09% | 101% | 101% |
|  | Method 2 | -0.01% | -0.11% | -0.05% | 100% | 101% |
|  | Method 3 | 0.00% | 0.00% | 0.00% | 101% | 100% |

Table 4. Results for Method 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.06% | 0.03% | 102% | 102% |
| Class A2 | -0.01% | -0.05% | 0.11% | 100% | 101% |
| Class B | 0.00% | 0.08% | -0.10% | 100% | 99% |
| Class C | 0.04% | 0.07% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.02% | 101% | 100% |
| Class D | 0.01% | 0.01% | 0.03% | 100% | 100% |
| Class F (optional) | 0.01% | 0.00% | 0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.27% | 0.04% | 100% | 98% |
| Class C | 0.03% | 0.10% | 0.13% | 102% | 106% |
| Class E | -0.02% | -0.04% | 0.10% | 101% | 101% |
| Overall | 0.00% | -0.09% | 0.09% | 101% | 101% |
| Class D | 0.00% | 0.05% | 0.31% | 101% | 100% |
| Class F (optional) | -0.03% | -0.30% | 0.07% | 102% | 100% |

Table 3. Results for Method 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | -0.04% | 101% | 101% |
| Class A2 | -0.01% | -0.01% | 0.13% | 100% | 101% |
| Class B | -0.01% | 0.09% | 0.02% | 100% | 100% |
| Class C | -0.02% | -0.11% | -0.02% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.02% | 101% | 101% |
| Class D | 0.02% | -0.02% | -0.03% | 101% | 101% |
| Class F (optional) | 0.01% | 0.00% | -0.01% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.22% | -0.26% | 100% | 100% |
| Class C | 0.02% | 0.12% | 0.19% | 101% | 102% |
| Class E | -0.09% | -0.23% | -0.01% | 101% | 102% |
| Overall | -0.01% | -0.11% | -0.05% | 100% | 101% |
| Class D | 0.00% | -0.39% | -0.50% | 101% | 98% |
| Class F (optional) | 0.08% | 0.27% | 0.05% | 100% | 100% |

Table 2. Results for Method 3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 105% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 101% | 105% |

# Conclusion

In this contribution, it is proposed to reduce the number of combinations listed for pairwise average candidate list. The list contains up to six possible combinations, however three of those combinations can never be used, and two of the remaining three candidates don’t add to the performance significantly. Hence is proposed to simplify the pairwise average candidate list construction process by allowing only one candidate. The performance change with the proposed modification on top of VTM3.0 under RA conditions is 0.01% for Y, 0.03% for U and 0.02% for V with no change in runtime. It is suggested to adopt the proposed method 3 into the next VVC working draft text.

# References

[1] “Versatile Video Coding (Draft 3),” JVET-L1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, October 2018.

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao,  October 2018.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).