| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET- M0194-v1 |
| --- | --- |

| Title: | CE10-related: Triangular Prediction Mode Harmonization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anish Tamse Min Woo Park Minsoo Park Seungsoo Jeong Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | tamse.anish@samsung.com m.w.park@samsung.com mss.park@samsung.com ss00.jeong@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Triangle prediction mode consists of candidate list generation as the first step of the process. The candidate list generation itself consists of generation of an MV list by checking the neighboring CUs and temporal MV. Based on this list, another list of uni-directional MVs is constructed, which is used in turn to create a combination of candidates. The first step of generating MV list by accessing neighbors and temporal MVs is very similar to the regular merge list construction process. Hence to avoid having additional logic module just for triangular prediction mode, it is proposed to harmonize the list construction process with the regular merge list construction process by using the regular merge list construction process for triangular mode list as well. Furthermore it is seen that only the first few candidates of the regular merge list are sufficient for the triangle prediction mode process. Since the regular merge list construction also uses HMVP and average candidates, an additional restriction is also tested which allows only the spatial and temporal candidates. This contribution also proposes a simplified pruning operation during construction of uni-directional MV list based on the spatial and temporal MV list from the previous step. The performance change after harmonization is -0.03% in Y, 0.03% in U and 0.00% in V without any change in runtime on top of VTM3.0 under RA test conditions.

# Introduction

In VVC [1], triangle prediction mode was adopted. This mode constructs a list of uni-directional MV pairs, and one MV is assigned to each triangular half of the block for motion compensation. The derivation process for this uni-directional MV pair can be described in following steps.

- Step 1. Construct an MV candidate list by checking spatially neighbour MV and adding up to two temporal MVs

- Step 2. Use the above constructed list to construct a new list of five uni-directional MVs

- Step 3. Use a predefined combination list to create up to 40 MV pair based on the uni-directional MV list.

Step 1 of the MV pair list construction process is similar to the regular merge list construction process, but not exactly the same. This introduces the need for an additional logic module which will be utilized only for triangular mode prediction.

Step 2 of the MV pair list construction process uses the MV list from step 1 and makes a new list of uni-directional MVs of five candidates. When doing this, each newly added uni-directional candidate is pruned against the existing uni-directional MV candidates in the list by comparing them. This results in many comparisons especially for the candidates added later, which need to be compared with all the previously added candidates..

This contribution proposes method to unify the triangle mode candidate derivation process with the existing merge candidate derivation process and simplify the pruning process of step 2.

# Proposed Methods

## Unification of candidate list derivation

To unify the step 1 of the triangle mode candidate derivation process with the regular merge list candidate derivation, step 1 is simply replaced with the regular merge list derivation process.

Furthermore since the step 2 requires only a few candidates from step 1 in order to completely fill up the uni-directional MV list of five candidates, only the first N of candidates from the regular merge candidate list are used. This restriction further simplifies the list derivation process

## Restriction to only spatial and temporal candidates

When using the regular merge list candidate construction process, HMVP buffer is accessed for adding HMVP candidates and averaging operations are performed for adding pairwise average candidates. These operations although rarely performed (since the method requires only the first N candidates, N=3,4), add new computations in the worst case compared to the original step 1 of the triangle merge list construction process. To address this, the regular merge candidate list construction process only till temporal candidates is followed, i.e. only spatial and temporal candidates are added. HMVP, average and zero candidates are not added. The list construction process stops after the

## Pruning simplification

Step 2 of the triangle mode candidate list constructs a list of five uni-directional MVs. When performing the pruning operation, each newly candidate added is added after comparing it with each of the existing MVs in the list. This results in many comparisons especially for the candidates added near the end of the list, which needs to be repeated if the candidate fails the pruning check. A simpler pruning method is proposed to reduce the total number of comparisons. Each candidate is only compared against the previously added candidate, and not all the previously added candidates. Hence each candidate undergoes only one MV comparison (except for the first candidate, which doesn’t require any comparison).

Combining the above described methods, the following options are proposed to unify the list construction process and simplify the pruning operations.

- Method 1) Use first 3 candidates of regular merge candidate list for triangle list construction

- Method 2) Use first 4 candidates of regular merge candidate list for triangle list construction

- Method 3) Method 2 + spatial temporal candidate only restriction

- Method 4) Method 3 + simplified pruning

# Experimental Results

The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Table 1 shows summary of all results.

Table 1. Summary of all results

|  | Test | BD-Rate (Y) | BD-Rate (U) | BD-Rate (V) | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| RA | Method 1 | -0.02% | 0.02% | -0.01% | 100% | 100% |
|  | Method 2 | -0.01% | 0.04% | -0.01% | 100% | 100% |
|  | Method 3 | -0.03% | 0.03% | 0.00% | 100% | 100% |
|  | Method 4 | 0.00% | 0.02% | 0.03% | 101% | 100% |
| LDB | Method 1 | 0.07% | 0.03% | 0.09% | 100% | 101% |
|  | Method 2 | -0.01% | -0.07% | 0.11% | 100% | 100% |
|  | Method 3 | 0.09% | 0.07% | 0.11% | 100% | 102% |
|  | Method 4 | 0.12% | 0.18% | 0.12% | 103% | 103% |

Table 2. Results for Method 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | -0.03% | 100% | 100% |
| Class A2 | -0.05% | 0.03% | -0.02% | 100% | 99% |
| Class B | -0.03% | 0.09% | -0.02% | 101% | 101% |
| Class C | 0.01% | -0.02% | 0.01% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.02% | -0.01% | 100% | 100% |
| Class D | -0.02% | -0.11% | -0.15% | 101% | 102% |
| Class F (optional) | 0.00% | -0.02% | -0.06% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.14% | -0.02% | 100% | 102% |
| Class C | 0.11% | 0.33% | 0.43% | 100% | 99% |
| Class E | 0.01% | -0.09% | -0.16% | 100% | 102% |
| Overall | 0.07% | 0.03% | 0.09% | 100% | 101% |
| Class D | 0.18% | 0.49% | 0.25% | 99% | 94% |
| Class F (optional) | -0.03% | -0.01% | 0.17% | 99% | 96% |

Table 3. Results for Method 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.01% | 100% | 99% |
| Class A2 | -0.05% | 0.03% | 0.02% | 100% | 100% |
| Class B | -0.02% | 0.04% | -0.05% | 100% | 100% |
| Class C | 0.02% | 0.07% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.04% | -0.01% | 100% | 100% |
| Class D | 0.05% | -0.02% | -0.10% | 100% | 101% |
| Class F (optional) | 0.01% | -0.01% | -0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.28% | 0.02% | 100% | 100% |
| Class C | 0.02% | 0.12% | 0.27% | 101% | 101% |
| Class E | -0.07% | -0.01% | 0.05% | 100% | 100% |
| Overall | -0.01% | -0.07% | 0.11% | 100% | 100% |
| Class D | 0.08% | 0.06% | 0.77% | 98% | 93% |
| Class F (optional) | 0.05% | 0.16% | -0.22% | 100% | 97% |

Table 4. Results for Method 3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.04% | 100% | 100% |
| Class A2 | -0.04% | -0.02% | 0.01% | 100% | 100% |
| Class B | -0.02% | 0.07% | 0.05% | 100% | 100% |
| Class C | -0.04% | 0.03% | -0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.03% | 0.00% | 100% | 100% |
| Class D | -0.02% | -0.14% | -0.13% | 100% | 101% |
| Class F (optional) | 0.00% | -0.01% | 0.04% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.19% | 0.00% | 101% | 103% |
| Class C | 0.07% | 0.19% | 0.14% | 100% | 102% |
| Class E | 0.19% | 0.35% | 0.24% | 100% | 100% |
| Overall | 0.09% | 0.07% | 0.11% | 100% | 102% |
| Class D | 0.07% | 0.07% | -0.06% | 100% | 101% |
| Class F (optional) | -0.02% | -0.03% | 0.22% | 100% | 100% |

Table 5. Results for Method 4

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.07% | -0.05% | 101% | 100% |
| Class A2 | -0.02% | 0.00% | 0.07% | 101% | 101% |
| Class B | -0.02% | 0.08% | 0.07% | 100% | 98% |
| Class C | 0.01% | 0.01% | 0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.03% | 101% | 100% |
| Class D | 0.01% | -0.04% | -0.01% | 98% | 97% |
| Class F (optional) | 0.00% | 0.00% | -0.01% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.17% | 0.12% | 104% | 105% |
| Class C | 0.14% | 0.57% | 0.32% | 102% | 103% |
| Class E | 0.17% | 0.23% | -0.16% | 101% | 98% |
| Overall | 0.12% | 0.18% | 0.12% | 103% | 103% |
| Class D | 0.12% | 0.08% | 0.16% | 98% | 97% |
| Class F (optional) | 0.10% | 0.21% | 0.12% | 105% | 100% |

# Conclusion

In this contribution, it is proposed to unify the candidate list construction process of triangle prediction mode with the regular merge candidate list construction process. It is also proposed to simplify the pruning process followed for generating the uni-directional MV candidate list. Method 4 of this proposal combines all the aspects of this contribution and shows marginal coding performance change. It is hence suggested to adopt the proposed method into the next VVC working draft text.

# References

[1] “Versatile Video Coding (Draft 3),” JVET-L1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, October 2018.

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao,  October 2018.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).