| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0207 |
| --- | --- |

| Title: | CE10-related: Joint optimizations of Triangular prediction unit mode and Multi-Hypothesis prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungsoo Jeong Min Woo Park Anish Tamse Minsoo Park Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | ss00.jeong@samsung.com m.w.park@samsung.com tamse.anish@samsung.com mss.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes optimization tests related to Triangular prediction unit mode (Triangle prediction mode) and Multi-Hypothesis (MH) prediction mode. This includes two optimization tests. The first optimization test is the removal of unnecessary flag signalling when MH mode flag is ON. Currently, when MH mode flag is ON, Triangular prediction unit mode flag is signalled. Since MH mode doesn’t use Triangular prediction unit, Triangular mode flag does not have to be signalled. Experimental results shows -0.02% luma BD-rate change in RA and -0.03% luma BD-rate change in LDB. The second optimization test is the unification of enable condition between MH mode and Triangular prediction unit mode. Currently, after skip and merge flag, there are MH mode flag and Triangular prediction mode flag. These two tools’ enable conditions are different with each other. After the unification of two tools’ enable conditions, two tools can be conducted with one enable condition without performance loss. Experimental results shows -0.01% luma BD-rate change in RA and 0.01% luma BD-rate change in LDB. The combination test of two optimization tests is also conducted. Experimental results shows -0.03% luma BD-rate change in RA and -0.04% luma BD-rate change in LDB.

Removal of redundant mode flag at MH prediction mode

Problem Statement

Currently, when Multi-Hypothesis (MH) prediction mode flag is ON, Triangle partition mode flag is signalled. Since MH mode doesn’t use Triangle partition, Triangle partition mode flag does not have to be signalled. Optimization 1 is the removal of this redundant Triangle partition mode flag, when MH mode is ON.

Proposed Syntax Structure

Table 1. The results of removal of redundant mode flag

Table 2. Proposed merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && slice_type = = B && mh_intra_flag[ x0 ][ y0 ] == 0 && cbWidth * cbHeight >= 64 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

Unification of enabling condition of MH and Triangle partition mode

Problem Statement

Currently, after skip and merge flag, there are MH mode flag and Triangle prediction mode flag. These two tools’ enable conditions are different with each other. Each tool enables under different condition.

Proposed Syntax Structure

The second optimization is the unification of enable condition between MH mode and Triangle prediction mode. These two tools’ enable conditions are different with each other. After the unification of two tools’ enable conditions, two tools can be conducted with one enable condition without performance loss.

Table 3. The unification of enable conditions

Table 4. Proposed merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && slice_type = = B && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

The Combination of Test 1 and Test 2

Table 3. The combination results of Test1 + Test2

Conclusion

This contribution includes two optimization tests. The first optimization test is the removal of unnecessary flag signalling when MH mode flag is ON. The second optimization test is the unification of enable condition between MH mode and Triangle prediction mode. The proposed optimization tests show -0.03% coding performance in RA and -0.04% coding performance in LDB. Therefore, it is suggested to adopt proposed methods into the next VVC working draft text.

The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Table 1 shows summary of all results.

References

[1] “Versatile Video Coding (Draft 3),” JVET-L1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).