| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0211-v1 |
| --- | --- |

| Title: | CE3-related: Fixed Reference Samples Design for CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo Xinwei Li Xiaoyan Chai Jianglin Wang Yanzhuo Ma Shuai Wan Fuzheng Yang YuanFang Yu Yang Liu | Email: | jyhuo@mail.xidian.edu.cn xwli_xidian@stu.xidian.edu.cn xychai@stu.xidian.edu.cn jlwang_2@stu.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

# Abstract

The current VVC includes a simplified CCLM parameter calculation which is derived by the maximum and the minimum luma/chroma of reference samples. This contribution further reduces the number of the reference samples for the MaxMin method. Given the block size, reference samples at fixed positions are used. The proposed method is used for CCLM mode. The results can get -0.05%, -0.35%, and -0.40% BD-rates on Y, Cb, and Cr components in All Intra configuration and -xxx%, -xxx%, and -xxx% BD-rates on Y, Cb, and Cr components in RandomAccess configuration compared to VTM3.0.

# Introduction

In VTM3.0, a simplified line fitting method of cross-component linear model (CCLM) was adopted, named as MaxMin method. Two Luma samples and their corresponding chroma samples are used to obtain the linear model parameters α and β. The two luma/chorma samples used in the MaxMin method are those with the minimum and the maximum luma value among the reference samples, respectively, as Point A and B shown in Figure 1.

A

B

Figure 1 Illustration of the MaxMin method

The linear model parameters α and β are obtained according to the following equations:

β= ∑Cn-α·∑LnN

α=yB-yAxB-xA                                        （1）

β= yA-αxA                                      （2）

Table 1 shows the number of operations which are used to derive the linear model parameter α and β.

Table 1 Number of operations used in the MaxMin method

| Operation type | Number of operations |
| --- | --- |
| Multiplications | 1 |
| Addition | 3 |
| “Division” | 1 |
| Comparisons | 2N |

As for the number of reference samples N, it can be calculated as follows:

For a coding block with a square shape, N is equal to twice of the width/height;

for a non-square coding block, N is equal to twice of the smaller size in the width and height, where the reference samples of the longer boundary are firstly subsampled to the same number of samples as for the shorter boundary;

if only the left neighbouring samples or the above neighbouring samples can be used, N is equal to the number of samples in the available boundary.

Compared with linear regression method, the MaxMin method can reduce the complexity to calculate the model parameters. Meanwhile, a small coding loss is introduced because only two points are used to calculate the linear model parameter in the MaxMin method.

# Proposed method

- Reference samples number reduction

As for the complexity, we can reduce the reference sample number N to further optimize the MaxMin method.

In the proposed CCLM method，the number of reference samples is restricted to 4 or 8 depending on the block size. Table 2 shows the number of reference samples of the MaxMin method and the proposed method.

Table 2 A comparison of the number of reference samples in two methods.

| Available short length of chroma block L | Reference samples number in the MaxMin method N | Reference samples number in the proposed method N’ |
| --- | --- | --- |
| 2 | 4 | 4 |
| 4 | 8 | 4 |
| 8 | 16 | 8 |
| 16 | 32 | 8 |
| 32 | 64 | 8 |

The available short length of chroma block, L, in Table 2 is defined as :

If both left and top reference samples are available, L equals to the short length of the chroma block;

If only left reference samples are available, the available short length of chroma block equals to the height of the chroma block;

If only top reference samples are available, the available short length of chroma block equals to the width of the chroma block.

As shown in Table 1, the sum of the number of operations except the “Division” used to derive the linear model parameters is 2N+4. Since the proposed method reduces the number of reference samples from N to N’, the number of operations is also reduced from 2N+4 to 2N’+4. Table 3 shows the number of operations of the MaxMin method and the proposed method.

Table 3 A comparison of the number of operations in two methods.

| Available short length of chroma block L | Operations number in MaxMin method 2N+4 | Operations number in proposed method 2N’+4 |
| --- | --- | --- |
| 2 | 12 | 12 |
| 4 | 20 | 12 |
| 8 | 36 | 20 |
| 16 | 68 | 20 |
| 32 | 132 | 20 |

- Reference samples position design

The selection of the reference samples is shown in Figure 3. Given the top reference line, we choose the middle part of reference samples because the middle part is more similar with the coding block than the left and right part. Similarly, given the left reference column, we choose the middle part of reference samples than the top and bottom part. The interval between reference samples is determined by the chroma block size and the N’ value.

Figure 3 The reference samples used for searching the minimum and maximum values in proposed method

In summary, the proposed method chooses reference samples which are assumed highly related to the current coding block. The proposed method can improve the robustness of the MaxMin method. It is noted that computational complexity is also reduced since the number of reference samples is reduced.

# Experimental results

The proposed scheme is implemented based on VTM3.0. The coding performance is provided in Table 4.

Anchor: VTM3.0

Tested: The proposed method

Table 4. Summary of experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.50% | -0.41% |  | 99% |
| Class A2 | -0.04% | -0.25% | -0.21% |  | 100% |
| Class B | -0.02% | -0.42% | -0.55% |  | 99% |
| Class C | -0.07% | -0.35% | -0.57% |  | 101% |
| Class E | -0.03% | -0.17% | -0.11% |  | 99% |
| Overall | -0.05% | -0.35% | -0.40% |  | 100% |
| Class D | -0.02% | -0.16% | -0.32% |  | 100% |
| Class F | -0.01% | -0.07% | -0.27% |  | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.31% | -0.52% |  |  |
| Class C | 0.01% | -0.31% | -0.25% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.03% | -0.32% | -0.39% |  |  |
| Class F | 0.00% | -0.26% | -0.53% |  |  |

# Conclusions

This contribution presents a modified CCLM method to reduce the number of reference samples and design the reference samples position. The proposed method gives 0.05%, 0.35%, and 0.40% BD rate savings on Y, Cb and Cr components for All Intra configuration and –x.xx%, -x.xx%, and –x.xx% BD rate savings on Y, Cb and Cr components for RandomAccess configuration.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).