| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0213-v1 |
| --- | --- |

| Title: | CE3-related: Chroma intra candidates modification based on directional DM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yanzhuo Ma Jianglin Wang Xinwei Li Junyan Huo Shuai Wan Fuzheng Yang YuanFang Yu Yang Liu | Email: | yzma@mail.xidian.edu.cn jlwang_2@stu.xidian.edu.cn xwli_xidian@stu.xidian.edu.cn jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

Abstract

In current VVC, chroma intra prediction modes are assigned to DM, CCLM, DC, Planar, Vertical and Horizontal. This contribution modifies chroma intra mode candidates according to the DM information. Specifically, if DM is a directional mode, Vertical and Horizontal are replaced by two derived modes from DM. The derived modes are obtained by adding -5 or +5 to DM mode.

The simulation results show -0.02%, -0.21%, and -0.30% BD-rates on Y, Cb, and Cr components, respectively, compared to VTM3.0 in All Intra configuration. The proposed method has negligible impact on the VTM encoder and decoder complexity.

Introduction

In VVC draft, chroma intra prediction modes are DM, CCLM and 4 additional default modes (Planar, Vertical, Horizontal and DC). When the chroma intra prediction mode is DM, the intra prediction mode of the co-located luma block is used. In case of DM being equal to any of the default modes, that default mode is replaced by VDIA_IDX (mode 66).

Proposed method

The coding trees for luma and chroma can be partitioned independently in VVC. Chroma DM is an effective mode which directly uses the intra prediction mode of the co-located luma block. There might exist multiple luma blocks corresponding to the current chroma block. In this way, the intra prediction mode from DM maybe a local optimal mode.

This contribution proposes to use DM±N when DM is an directional mode. We use the two derived mode to replace Vertical and Horizontal mode. Firstly, we compare the coding performance of different N  (N=3,5,7,10) for C and E test sequences. The modes (DM±5) are selected as the final derived mode.

The derived mode is calculated as follows. If the DM is mode M (2<=M<=66). Let M1 and M2 denote M+N and M-N. M1 and M2 are derived as follows:

offset = 61;

mod = 64;

M1 = (M -2 + N)% mod + 2;

M2 = ((M + offset + 1 - N) % mod) + 2;

The chroma intra prediction modes list of the proposed method is shown in Table 1.

Table 1: chroma intra prediction modes comparison between VTM3.0 and the proposed method.

| Chroma prediction mode | DM = Directional Mode |  |
| --- | --- | --- |
|  | VTM3.0 | The Proposed Method |
| 0 | VDIA_IDX | PLANAR_IDX |
| 1 | VER_IDX | DM+5 |
| 2 | HOR_IDX | DM-5 |
| 3 | DC_IDX | DC_IDX |
| 4 | LM | LM |
| 5 | LM_L | LM_L |
| 6 | LM_T | LM_T |
| 7 | PLANAR_IDX | X |

The proposed method has negligible impact on the VTM encoder and decoder complexity.

Experimental results

Test #1:

The preliminary experiment is conducted. Different N values are used to construct DM derived modes. The preliminary experiment is performed on AI configurations with 24 frames interval.

Anchor: VTM3.0

Tested: DM derived modes with different N values

Table 2: The tests results of different derived modes of DM

| BD-rates Performance | Class C |  |  | Class E |  |  | Average |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| DM±1 | 0.04% | -0.40% | -0.53% | 0.08% | -0.94% | -0.47% | 0.06% | -0.67% | -0.50% |
| DM±2 | 0.02% | -0.54% | -0.67% | 0.05% | -1.12% | -0.73% | 0.03% | -0.83% | -0.70% |
| DM±3 | -0.02% | -0.55% | -0.62% | -0.03% | -0.75% | -0.64% | -0.02% | -0.65% | -0.63% |
| DM±5 | -0.05% | -0.31% | -0.57% | -0.02% | -0.33% | -0.33% | -0.03% | -0.32% | -0.45% |
| DM±7 | -0.05% | -0.13% | -0.38% | -0.01% | -0.31% | -0.26% | -0.03% | -0.22% | -0.32% |
| DM±10 | -0.05% | -0.05% | 0.01% | -0.02% | 0.15% | 0.00% | -0.03% | 0.05% | 0.00% |

When N equals to 2,3,5,7, the BD-rates performance on luma increases and the performance on chroma decreases with the increase of N. And// to be added

Test #2:

Anchor: VTM3.0

Tested: The proposed method under the JVET common test condition.

Table 3. Summary of experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.19% | -0.13% |  | 100% |
| Class A2 | -0.02% | -0.14% | -0.20% |  | 100% |
| Class B | -0.01% | -0.17% | -0.28% |  | 100% |
| Class C | -0.03% | -0.29% | -0.46% |  | 101% |
| Class E | -0.04% | -0.24% | -0.37% |  | 101% |
| Overall | -0.02% | -0.21% | -0.30% |  | 100% |
| Class D | -0.01% | -0.25% | -0.37% |  | 101% |
| Class F | -0.01% | -0.24% | -0.24% |  | 101% |

Conclusions

This contribution presents chroma intra candidates modification based on directional DM. The proposed method gives 0.02%, 0.21%, and 0.30% BD rate savings on Y, Cb and Cr components for All Intra configuration when N is set to 5.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).