| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0213-v2 |
| --- | --- |

| Title: | CE3-related: Chroma intra candidates modification based on directional DM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yanzhuo Ma Jianglin Wang Xinwei Li Junyan Huo Shuai Wan Fuzheng Yang YuanFang Yu Yang Liu | Email: | yzma@mail.xidian.edu.cn jlwang_2@stu.xidian.edu.cn xwli_xidian@stu.xidian.edu.cn jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

Abstract

In current VVC, chroma intra prediction modes are assigned as DM, CCLM, DC, Planar, Vertical and Horizontal. This contribution modifies the chroma intra mode candidates according to the DM information. Specifically, if DM is a directional mode, Vertical and Horizontal candidates are replaced by two modes which are obtained by adding -4 or +4 to the DM value.

The simulation results show -0.03%, -0.31%, and -0.39% BD-rates on Y, Cb, and Cr components, respectively, compared to VTM3.0 in All Intra configuration. The proposed method has negligible impact on the VTM encoder and decoder complexity.

Introduction

In VVC draft, chroma intra prediction modes include DM, CCLM and 4 additional default modes (Planar, Vertical, Horizontal and DC). When the chroma intra prediction mode is DM, the intra prediction mode of the co-located luma block is used. In case of DM being equal to any of the default modes, that default mode is replaced by VDIA_IDX (mode 66).

Proposed method

The coding trees for luma and chroma can be partitioned independently in VVC. Chroma DM is an effective mode which directly uses the intra prediction mode of the co-located luma block. There might exist multiple luma blocks corresponding to the current chroma block. Only the mode of the center block in the region is used as DM.

This contribution proposes to use modes of DM±N to replace the Vertical and Horizontal modes, when DM is a directional mode. Firstly, we compare the coding performance of different Ns (N=2,3,4,5,7,10). The modes (DM±4) are selected as the final modes.

The DM±N modes are calculated as follows. Note the DM as mode M (2<=M<=66). Let M1 and M2 denote M-N and M+N, respectively. M1 and M2 are obtained as follows:

M1 = ((M + 62 - N) % 64) + 2;

M2 = (M -2 + N) % 64 + 2

The latest chroma intra prediction modes list of the method in VVC draft is shown in Table 1.

Table 1 Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] when sps_cclm_enabled_flag is equal to 1

.

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |

The chroma intra prediction modes list of the proposed method is shown in Table 2.

Table 2 The specification of IntraPredModeC[ xCb ][ yCb ] proposed.

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | X ( 2 <= X <= 66 ) |
| 0 | 66 | 0 | 0 |
| 1 | 50 | 50 | X-N |
| 2 | 18 | 18 | X+N |
| 3 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 |
| 7 | 0 | 1 | X |

The proposed method has negligible impact on the VTM encoder and decoder complexity.

Experimental results

The preliminary experiments of the proposed method with different N values are performed on AI configurations with 24 frames interval. The results are shown in Table 3. And the complete results of the experiments with N=4 under the JVET CTC are shown in Table 4.

Table 3:Performance of DM±N modes with different N values compared with VTM3.0

| BD-rates Performance | Class C |  |  | Class E |  |  | Average |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| N=2 | -0.02% | -0.61% | -0.86% | -0.04% | -1.12% | -1.00% | -0.03% | -0.86% | -0.93% |
| N=3 | -0.02% | -0.55% | -0.62% | -0.03% | -0.75% | -0.64% | -0.02% | -0.65% | -0.63% |
| N=4 | -0.07% | -0.26% | -0.53% | -0.04% | -0.51% | -0.70% | -0.06% | -0.38% | -0.61% |
| N=5 | -0.05% | -0.31% | -0.57% | -0.02% | -0.33% | -0.33% | -0.03% | -0.32% | -0.45% |
| N=7 | -0.05% | -0.13% | -0.38% | -0.01% | -0.31% | -0.26% | -0.03% | -0.22% | -0.32% |
| N=10 | -0.05% | -0.05% | 0.01% | -0.02% | 0.15% | 0.00% | -0.03% | 0.05% | 0.00% |

Table 4:

Anchor: VTM3.0

Tested: The proposed method with N = 4.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.34% | -0.14% |  | 101% |
| Class A2 | -0.02% | -0.26% | -0.22% |  | 101% |
| Class B | -0.01% | -0.23% | -0.38% |  | 100% |
| Class C | -0.03% | -0.31% | -0.58% |  | 99% |
| Class E | -0.05% | -0.48% | -0.55% |  | 100% |
| Overall | -0.03% | -0.31% | -0.39% |  | 100% |
| Class D | -0.04% | -0.33% | -0.47% |  | 99% |
| Class F | -0.01% | -0.29% | -0.25% |  | 100% |

Conclusions

This contribution presents chroma intra candidates modification based on directional DM. The proposed method gives 0.03%, 0.31%, and 0.39% BD rate savings on Y, Cb and Cr components for All Intra configuration when N is set to 4.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).