| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0222 |
| --- | --- |

| Title: | Context Reduction for CABAC in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Amir Said, Vadim Seregin, Jie Dong, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

# Abstract

In this proposal, contexts in CABAC are investigated. With 27 less contexts, experimental result shows negligible losses in BD-rate

AI: 0.03% (Y), -0.02% (Cb), -0.02% (Cr)

RA: 0.01% (Y), 0.00% (Cb), -0.01% (Cr)

LDB: 0.01% (Y), 0.04% (Cb), -0.12% (Cr)

# Problem description

In this proposal, the number of CABAC contexts for syntax elements, InterDir, AffMergeIdx, GBiIdx, ParFlag, GtxFlag, LastX, LastY, are investigated. The modifications reduce 27 contexts with negligible coding efficiency changes in the simulation results.

# Proposed Solution

## GBi Index

In generalized bi-prediction (Gbi) in VTM3.0, a set of five weights in combining inter prediction from L0 and L1 are supported for low delay coded pictures and a set of three weights are supported for bi-prediction coded pictures. The binarization on weight index for low delay pictures and non-low delay pictures are shown in Table 1.

| Low delay pictures |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| GBi index | 0 | 1 | 2 | 3 | 4 |
| Bin string | 1 | 01 | 001 | 0001 | 0000 |
| Non-low delay pictures |  |  |  |  |  |
| GBi index | 0 | 1 | 2 |  |  |
| Bin string | 1 | 01 | 00 |  |  |

Table 1: Binarization for Gbi index for low delay pictures in VTM3.0

| Bin | 1st bin | 2nd bin | 3rd bin | 4th bin |
| --- | --- | --- | --- | --- |
| Context index | 0 | 4 | 5 | 6 |

For the design of context set, one context is used for each bin in the bin string, as show in Table 2.

Table 2: Context set design for Gbi index in VTM3.0

In the proposed method, the same binarization as in VTM3.0 is applied. For context assignment, instead of assigning different context for different bin, one context is used for the first bin (indicating whether the GBbi index equals to 0) and one context is used for the rest, as shown in Table 3.

| Bin | 1st bin | 2nd – 4th bin |
| --- | --- | --- |
| Context index | 0 | 4 |

Table 3: Proposed context set design for Gbi index

## Affine merge index

In VTM3.0, up to five affine merge candidates are supported. The binarization on syntax element is shown in Table 4.

| Affine index | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Bin string | 0 | 10 | 110 | 1110 | 1111 |

Table 4: Binarization for Affine merge index in VTM3.0

For the design of context set, one context is used for each bin in the bin string, as show in Table 5.

| Bin | 1st bin | 2nd bin | 3rd bin | 4th bin |
| --- | --- | --- | --- | --- |
| Context index | 0 | 1 | 2 | 3 |

Table 5: Context set design for affine merge index in VTM3.0

In the proposed method, the same binarization as in VTM3.0 is applied. For context assignment, instead of assigning different context for different bin, one context is used for the first bin (indicating whether the affine merge index equals to 0) and one context is used for the rest, as shown in Table 6.

| Bin | 1st bin | 2nd – 4th bin |
| --- | --- | --- |
| Context index | 0 | 1 |

Table 6: Proposed context set design for affine merge index

## Inter Direction index

In VTM3.0, the syntax InterDir equals 1 for uni-directional prediction from L0 list, equals 2 for uni-directional prediction from L1 list, and equals 3 for bi-directional prediction. The binarization on syntax element is shown in Table 7.

| InterDir | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Bin string | 00 | 01 | 1 |

Table 7: Binarization for inter direction index in VTM3.0

For the design of context set, one context (context index = 4) is used for the second bin if uni-directional prediction is applied to indicate which reference list is used, and four contexts are used for the first bin to indicate if bi-directional prediction is applied. The context selection for the first bin depends on the size of coding unit. Define

S=7- (log2W+log2H+1)/2

for block width = W and block height = H.  The context set design is shown in Table 8.

| S | 0 | 1 | 2 | ≥3 |
| --- | --- | --- | --- | --- |
| Context index | 0 | 1 | 2 | 3 |

Table 8: Context set design for inter direction index in VTM3.0

In the proposed method, the same binarization as in VTM3.0 is applied and three contexts instead of four are used for signaling the first bin, as shown in Table 9.

| S | ≤1 | 2 | ≥3 |
| --- | --- | --- | --- |
| Context index | 1 | 2 | 3 |

Table 9: Proposed context set design for inter direction index

## Coefficient level coding

In VTM3.0, the significance (sig _flag), parity (par_flag), and greater than 1 flags (gt1_flag) are coded in the 1st pass over the scan positions in a subblock. The greater than 2 flags (gt2_flag) are coded in the 2nd pass. There are 32 contexts for parity (par_ flag), greater than 1 flag (gt1_flag) and greater than 2 flags (gt2_flag). Among the contexts, 21 of them are for luma and 11 of them are for chroma. The indices of the context are determined based on the position (x and y coordinates) and the absolute levels in a local neighbourhood of the current scan position. The template for local neighbourhood is illustrated in Figure 1.

Figure 1: Illustration of the template used for local neighborhood. The black square specifies the current scan position and the blue squares represent the local neighborhood used.

Define diag = x + y, and sumAbs and numSig as the sum of absolute levels and the number of non-zero levels in the local neighbourhood, the context selections for luma and chroma are determined as shown in Table 10 and Table 11.

| diag | ≥10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| min(sumAbs- numSig, 4) + 1 | 1 | 2 | 3 | 4 | 5 |
| Context index | 1 | 2 | 3 | 4 | 5 |
| diag | ≥ 3 and < 10 |  |  |  |  |
| min(sumAbs- numSig, 4) + 1 | 1 | 2 | 3 | 4 | 5 |
| Context index | 6 | 7 | 8 | 9 | 10 |
| diag | > 0 and < 3 |  |  |  |  |
| min(sumAbs- numSig, 4) + 1 | 1 | 2 | 3 | 4 | 5 |
| Context index | 11 | 12 | 13 | 14 | 15 |
| diag | 0 |  |  |  |  |
| min(sumAbs- numSig, 4) + 1 | 1 | 2 | 3 | 4 | 5 |
| Context index | 16 | 17 | 18 | 19 | 20 |

Table 10: Context selection for par_flag, gt1_flag and gt2_flag of luma in VTM3.0

| diag | > 0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| min(sumAbs- numSig, 4) + 1 | 1 | 2 | 3 | 4 | 5 |
| Context index | 1 | 2 | 3 | 4 | 5 |
| diag | 0 |  |  |  |  |
| min(sumAbs- numSig, 4) + 1 | 1 | 2 | 3 | 4 | 5 |
| Context index | 6 | 7 | 8 | 9 | 10 |

Table 11: Context selection for par_flag, gt1_flag and gt2_flag of chroma in VTM3.0

The context index is zero if the current scan position is equal to the position of the last non-zero level.

In the proposed method, the context selections for par_flag, gt1_flag and gt2_flag of luma and chroma are modified as in Table 12 and 13.

| diag | ≥10 |  |  |  |
| --- | --- | --- | --- | --- |
| min(sumAbs- numSig, 3) + 1 | 1 | 2 | 3 | 4 |
| Context index | 1 | 2 | 3 | 4 |
| diag | ≥ 3 and < 10 |  |  |  |
| min(sumAbs- numSig, 3) + 1 | 1 | 2 | 3 | 4 |
| Context index | 6 | 7 | 8 | 9 |
| diag | > 0 and < 3 |  |  |  |
| min(sumAbs- numSig, 3) + 1 | 1 | 2 | 3 | 4 |
| Context index | 11 | 12 | 13 | 14 |
| diag | 0 |  |  |  |
| min(sumAbs- numSig, 3) + 1 | 1 | 2 | 3 | 4 |
| Context index | 16 | 17 | 18 | 19 |

Table 12: Proposed context selection for par_flag, gt1_flag and gt2_flag of luma

| diag | > 0 |  |  |  |
| --- | --- | --- | --- | --- |
| min(sumAbs- numSig, 3) + 1 | 1 | 2 | 3 | 4 |
| Context index | 1 | 2 | 3 | 4 |
| diag | 0 |  |  |  |
| min(sumAbs- numSig, 3) + 1 | 1 | 2 | 3 | 4 |
| Context index | 6 | 7 | 8 | 9 |

Table 13: Proposed context selection for par_flag, gt1_flag and gt2_flag of chroma

## Last significant coefficient

In VTM3.0, there are 21 contexts for both last x and last y coordinates (syntax element LastX and LastY). In this proposal, we propose to reduce the contexts for block width (height) equal to 64 in high frequency area. For binarization, x (y) coordinates of the last significant coefficient are first mapped to 12 groups as shown in Table 14, and the corresponding group id is coded with unary coding.

| Coordinate (x or y) | 0 | 1 | 2 | 3 | 4-5 | 6-7 | 8-11 | 12-15 | 16-23 | 24-31 | 32-47 | 48-63 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group id | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

Table 14: Mapping of last x (y) coordinate to group IDs for binarization

In VTM3.0, context index 15-20 are assigned for LastX and LastY if block width and height equal 64. For context design of block width (height) equal to 64, one context is assigned to two bins in the bin string, as shown in Table 15.

| Bin | 1st – 2nd | 3rd – 4th | 5th – 6th | 7th-8th | 9th – 10th | 11th – 12th |
| --- | --- | --- | --- | --- | --- | --- |
| Context index | 15 | 16 | 17 | 18 | 19 | 20 |

Table 16: Context selection for position of last significant coefficient in VTM3.0

In the proposed method, we reduce the context assignment of LastX and LastY as shown in Table 16.

| Bin | 1st – 2nd | 3rd – 4th | 5th – 6th | 7th-12th |
| --- | --- | --- | --- | --- |
| Context index | 15 | 16 | 17 | 18 |

Table 17: Proposed context selection for position of last significant coefficient

## Last significant coefficient

Tables 18 lists the number of the contexts for all the syntax elements modified in VTM3.0 and proposed method. Totally 27 contexts are reduced.

| # of contexts | VTM3.0 | Proposed |
| --- | --- | --- |
| Gbi index | 4 | 2 |
| Affine merge index | 4 | 2 |
| Inter direction index | 4 | 3 |
| Parity, greater than 1, greater than 2 flags | 96 | 78 |
| Position of the last significant coefficient | 42 | 38 |
| Total | 150 | 123 |

Table 18: Comparison of number of contexts in the proposed method and VTM3.0

# Simulation results

The proposed method is evaluated according to the common test condition for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations on VTM3.0 software.

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |
|  | Y | U | V |
| Class A1 | 0.03% | -0.05% | 0.02% |
| Class A2 | 0.05% | 0.05% | 0.01% |
| Class B | 0.03% | 0.01% | 0.00% |
| Class C | 0.01% | -0.05% | -0.10% |
| Class E | 0.04% | -0.05% | -0.01% |
| Overall | 0.03% | -0.02% | -0.02% |
| Class D | 0.01% | 0.11% | 0.16% |
| Class F (optional) | 0.05% | 0.08% | 0.12% |
|  | Random Access Main 10 |  |  |
|  | Over VTM-3.0 |  |  |
|  | Y | U | V |
| Class A1 | 0.03% | -0.05% | 0.12% |
| Class A2 | 0.00% | 0.10% | 0.11% |
| Class B | 0.00% | 0.08% | -0.16% |
| Class C | 0.00% | -0.15% | -0.02% |
| Class E |  |  |  |
| Overall | 0.01% | 0.00% | -0.01% |
| Class D | -0.04% | -0.21% | -0.22% |
| Class F (optional) | 0.03% | 0.25% | 0.17% |
|  | Low delay B Main10 |  |  |
|  | Over VTM-3.0 |  |  |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -0.01% | -0.39% | -0.11% |
| Class C | 0.02% | 0.29% | 0.30% |
| Class E | 0.03% | 0.40% | -0.68% |
| Overall | 0.01% | 0.04% | -0.12% |
| Class D | 0.03% | 0.12% | 0.23% |
| Class F (optional) | 0.06% | 0.39% | 0.08% |

# Reference

[1] VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

[2] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, JVET 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).