| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0230 |
| --- | --- |

| Title: | Non-CE4: The Reduction of the Temporal MV Buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xianglin Wang | Email: | yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In VTM-3.0, for higher coding efficiency, 1/16-pel motion compensation is supported and all the MVs are stored in 1/16-pel precision in the temporal MV buffer. To reduce the size of temporal MV buffer, we propose to store these MVs in 1/4-pel precision in the temporal MV buffer. It is asserted that the proposed method can reduce the required temporal MV buffer size by about 10%. According to the results reported, the proposed simplifications incur 0.03% and 0.09% BD-rate increase under RA and LDB conditions respectively.

# Introduction

In VTM-3.0[1], some inter prediction modes such as the affine mode could derive MVs at 1/16-pel precision and motion compensated prediction is performed at 1/16-pel precision as well. Currently, all motion vectors are stored at 1/16-pel precision for more accurate MV predictions.

# Proposed Implementation

The size of the motion data of a block include 4 bits for reference index and 36 bits for motion vectors (18 bits respectively for horizontal and vertical components). In the current VVC, the 4:1 compression of temporal motion data is utilized, which means the motion data is stored for each 8x8 block. Based on these assumptions, it is estimated that the temporal MV buffer size in the current VVC is approximately 10Mbits/frame for a 4kx2k video.

To reduce the size of temporal MV buffer, it is proposed to store the MVs in 1/4-pel precision instead of 1/16-pel precision. Therefore, the temporal MV buffer size is approximately 9Mbits/frame for a 4kx2k video (i.e. 10% reduction).

# Results

The proposed modification is integrated into VTM-3.0 [1] and the results of the proposed implementation under CTC [2] are shown below against VTM3.0.

Table 1. Performance of the proposed modification under Random Access

Table 2. Performance of the proposed modification under Low delay B

# Conclusion

In this document, it is proposed to store the MVs into temporal MV buffer in 1/4-pel precision instead of 1/16-pel precision. With the proposed modifications, the size of temporal MV buffer could be reduced by 10% with minor coding performance loss. It is recommended to consider adoption of the proposed method into VVC.

# Reference

- VTM-3.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-L1010, October 2018.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).