added

deleted

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| mh_intra_luma_mode | ae(v) |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

Table 9-4- Syntax elements and associated binarizations

| merge_data( ) | mmvd_flag[ ][ ] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | mmvd_merge_flag[ ][ ] | FL | cMax = 1 |
|  | mmvd_distance_idx[ ][ ] | TR | cMax = 7, cRiceParam = 0 |
|  | mmvd_direction_idx[ ][ ] | FL | cMax = 3 |
|  | mh_intra_flag[ ][ ] | FL | cMax = 1 |
|  | mh_intra_luma_mode | TR | cMax = ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth )?3:4, cRiceParam = 0 |
|  | mh_intra_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | mh_intra_luma_mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | merge_subblock_flag[ ][ ] | FL | cMax = 1 |
|  | merge_subblock_idx[ ][ ] | TR | cMax = MaxNumSubblockMergeCand − 1, cRiceParam = 0 |
|  | merge_triangle_flag[ ][ ] | FL | cMax = 1 |
|  | merge_triangle_idx[ ][ ] | EG1 | - |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |

    - General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Output of this process is a modified reconstructed picture before in-loop filtering.

The derivation process for quantization parameters as specified in clause 8.4.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType set equal to SINGLE_TREE as inputs.

The decoding process for coding units coded in inter prediction mode consists of the following ordered steps:

- The motion vector components and reference indices of the current coding unit are derived as follows:

- If merge_triangle_flag[ xCb ][ yCb ], inter_affine_flag[ xCb ][ yCb ] and merge_subblock_flag[ xCb ][ yCb ] are all equal to 0, the following applies:

- The derivation process for motion vector components and reference indices as specified in clause 8.3.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the chroma motion vectors mvCL0[ 0 ][ 0 ] and mvCL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], and the bi-prediction weight index gbiIdx as outputs.

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are both set equal to 1.

- Otherwise, if merge_triangle_flag[ xCb ][ yCb ] is equal to 1, inter_affine_flag[ xCb ][ yCb ] and merge_subblock_flag[ xCb ][ yCb ] are both equal to 0, the derivation process for triangle motion vector components and reference indices as specified in clause 8.3.3.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vectors mvLA and mvLB, the chroma motion vectors mvCLA and mvCLB, the reference indices refIdxLA and refIdxLB and the prediction list flags predFlagLA and predFlagLB as outputs.

- Otherwise (inter_affine_flag[ xCb ][ yCb ] or merge_subblock_flag[ xCb ][ yCb ] is equal to 1), the following applies:

- The derivation process for subblock motion vector components and reference indices as specified in clause 8.3.4.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight as inputs, and the reference indices refIdxL0 and refIdxL1, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the prediction list utilization flags predFlagLX[ xSbIdx ][ ySbIdx ], the luma motion vector array mvLX[ xSbIdx ][ ySbIdx ], and the chroma motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..(cbWidth >> 2) − 1, and ySbIdx = 0..( cbHeight >> 2 ) − 1, and with X being 0 or 1, and the bi-prediction weight index gbiIdx as outputs.

- When mh_intra_flag[ xCb ][ yCb ] is equal to 1, the derivation process for intra prediction mode in combined merge and intra prediction as specified in 8.3.5 is invoked.the variable IntraPredModeY[ x ][ y ] with x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is derived usin mh_intra_modes as specified in Table-XX

Table XX

| mh_intra_modes | cbWidth is greater than 2 * cbHeight | cbHeight is greater than 2 * cbWidth | Others |
| --- | --- | --- | --- |
| 0 | INTRA_PLANAR | INTRA_PLANAR | INTRA_PLANAR |
| 1 | INTRA_DC | INTRA_DC | INTRA_DC |
| 2 | INTRA_ANGULAR50 | INTRA_ANGULAR18 | INTRA_ANGULAR18 |
| 3 | - | - | INTRA_ANGULAR50 |

- The prediction samples of the current coding unit are derived as follows:

- If merge_triangle_flag[ xCb ][ yCb ] is equal to 0, the prediction samples of the current coding unit are derived by invoking the decoding process for inter blocks as specified in clause 8.3.6.1 with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the luma motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ], the chroma motion vectors mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the reference indices refIdxL0 and refIdxL1, the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ], and the generalization bi-prediction weight index gbiIdx as inputs, and the inter prediction samples (predSamples) that are an (cbWidth)x(cbHeight) array predSamplesL of prediction luma samples and two (cbWidth / 2)x(cbHeight / 2) arrays predSamplesCr and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr, as outputs.

- Otherwise (merge_triangle_flag[ xCb ][ yCb ] is equal to 0), the decoding process for triangular inter blocks as specified in clause 8.3.7.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the luma motion vectors mvLA and mvLB, the chroma motion vectors mvCLA and mvCLB, the reference indices refIdxLA and refIdxLB, and the prediction list flags predFlagLA and predFlagLB as inputs, and the inter prediction samples (predSamples) that are an (cbWidth)x(cbHeight) array predSamplesL of prediction luma samples and two (cbWidth / 2)x(cbHeight / 2) arrays predSamplesCb and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr, as outputs.

- The variables NumSbX[ xCb ][ yCb ] and NumSbY[ xCb ][ yCb ] are set equal to numSbX and numSbY, respectively.

- The residual samples of the current coding unit are derived as follows:

- The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.8 is invoked with the location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the width nTbW set equal to the luma coding block width cbWidth, the height nTbH set equal to the luma coding block height cbHeight and the variable cIdxset equal to 0 as inputs, and the array resSamplesL as output.

- The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdxset equal to 1 as inputs, and the array resSamplesCb as output.

- The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdxset equal to 2 as inputs, and the array resSamplesCr as output.

- The reconstructed samples of the current coding unit are derived as follows:

- The picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb, yCb ), the block width bWidth set equal to cbWidth, the block height bHeight set equal to cbHeight, the variable cIdx set equal to 0, the (cbWidth)x(cbHeight) array predSamples set equal to predSamplesL and the (cbWidth)x(cbHeight) array resSamples set equal to resSamplesL as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- The picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx set equal to 1, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCb and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCb as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- The picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx set equal to 2, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCb and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCb as inputs, and the output is a modified reconstructed picture before in-loop filtering.

    - Derivation process for intra prediction mode in combined merge and intra prediction

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb ) and ( xCb, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_DC.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and mh_intra_flag[ xNbX ][ yNbX ] is not equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is derived as follows:

- If IntraPredModeY[ xNbX ][ yNbX ] is greater than INTRA_ANGULAR34, candIntraPredModeX is set equal to INTRA_ANGULAR50.

- Otherwise, if IntraPredModeY[ xNbX ][ yNbX ] is less than or equal to INTRA_ANGULAR34 and IntraPredModeY[ xNbX ][ yNbX ] is greater than INTRA_DC, candIntraPredModeX is set equal to INTRA_ANGULAR18.

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candidate mode list candModeList[ x ] with x = 0..2 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

- If candIntraPredModeA is less than 2 (i.e., equal to INTRA_PLANAR or INTRA_DC), candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = INTRA_PLANAR	(8-640)

candModeList[ 1 ] = INTRA_DC		(8-641)

candModeList[ 2 ] = INTRA_ANGULAR50	(8-642)

- Otherwise, candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-643)

candModeList[ 1 ] = INTRA_PLANAR	(8-644)

candModeList[ 2 ] = INTRA_DC		(8-645)

- Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

- candModeList[ 0 ] and candModeList[ 1 ] are derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-646)

candModeList[ 1 ] = candIntraPredModeB	(8-647)

- If neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_PLANAR, candModeList[ 2 ] is set equal to INTRA_PLANAR,

- Otherwise, if neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_DC, candModeList[ 2 ] is set equal to INTRA_DC,

- Otherwise, candModeList[ 2 ] is set equal to INTRA_ANGULAR50.

- The following process is applied to update candModeList:

- If cbHeight is greater than 2 * cbWidth and one of candModeList[ x ], with x = 0..2 is equal to INTRA_ANGULAR50, the coressponding candModeList[ x ] is replaced with candIntraPredModeC, which is derived as follows:

- If none of candModeList[ x ], with x = 0..2 is equal to INTRA_PLANAR, candIntraPredModeC is set equal to INTRA_PLANAR.

- Otherwise, if none of candModeList[ x ], x = 0..2 is equal to INTRA_DC, candIntraPredModeC is set equal to INTRA_DC.

- Otherwise, if none of candModeList[ x ], with x = 0..2 is equal to INTRA_ANGULAR50, candIntraPredModeC is set equal to INTRA_ANGULAR50.

- Otherwise (none of candModeList[ x ], x = 0..2 is equal to INTRA_ANGULAR18). candIntraPredModeC is set equal to INTRA_ANGULAR18.

- Otherwise, if cbWidth is greater than 2 * cbHeight and if one of candModeList[ x ], with x = 0..2 is equal to INTRA_ANGULAR18, the coressponding candModeList[ x ] is replaced with candIntraPredModeC, which is derived as follows:

- If none of candModeList[ x ], with x = 0..2 is equal to INTRA_PLANAR, candIntraPredModeC is set equal to INTRA_PLANAR.

- Otherwise, if none of candModeList[ x ], x = 0..2 is equal to INTRA_DC, candIntraPredModeC is set equal to INTRA_DC.

- Otherwise, if none of candModeList[ x ], with x = 0..2 is equal to INTRA_ANGULAR50, candIntraPredModeC is set equal to INTRA_ANGULAR50.

- Otherwise (none of candModeList[ x ], x = 0..2 is equal to INTRA_ANGULAR18). candIntraPredModeC is set equal to INTRA_ANGULAR18.

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If mh_intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is set to equal to candIntraPredModeC, which is derived as follows:

- If none of candModeList[ x ], with x = 0..2 is equal to INTRA_PLANAR, candIntraPredModeC is set equal to INTRA_PLANAR.

- Otherwise, if none of candModeList[ x ], x = 0..2 is equal to INTRA_DC, candIntraPredModeC is set equal to INTRA_DC.

- Otherwise, if none of candModeList[ x ], with x = 0..2 is equal to INTRA_ANGULAR50, candIntraPredModeC is set equal to INTRA_ANGULAR50.

- Otherwise (none of candModeList[ x ], x = 0..2 is equal to INTRA_ANGULAR18). candIntraPredModeC is set equal to INTRA_ANGULAR18.

The variable IntraPredModeY[ x ][ y ] with x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].