| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0232 |
| --- | --- |

| Title: | Non-CE10: Simplification of Intra Mode coding for CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xianglin Wang | Email: | yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, a modification of intra mode coding for combined inter and intra prediction (CIIP) is proposed. In the current CIIP scheme, most probable mode (MPM) mechanism is used to predict the intra modes of CIIP. In this contribution, we propose to remove the MPM mechanism for the intra mode coding of CIIP. Instead, the truncated unary codewords are used for the 4 intra modes of CIIP. The results reportedly show that the proposed simplification incurs -0.01% and 0.01% BD-rate increase for RA and LDB, respectively.

# Introduction

In VTM-3.0, when a CU is coded in combined inter and intra prediction (CIIP) mode, merge index and intra prediction modes are signaled to derive the corresponding inter and intra prediction. An intra prediction mode is first derived from two additional syntax elements: mpm flag and mpm index. At the decoder side, the CIIP MPM candidate list is derived as follows:

- The left and top neighbouring blocks are set as A and B, respectively

- The intra prediction modes of block A and block B, denoted as intraModeA and intraModeB, respectively, are derived as follows:

  - Let X be either A or B

  - intraModeX is set to DC if 1) block X is not available; or 2) block X is not predicted using the CIIP mode or the intra mode; 3) block B is outside of the current CTU

  - otherwise, intraModeX is set to 1) DC or PLANAR if the intra prediction mode of block X is DC or PLANAR; or 2) VERTICAL if the intra prediction mode of block X is a “vertical-like” angular mode (larger than 34), or 3) HORIZONTAL if the intra prediction mode of block X is a “horizontal-like” angular mode (smaller than or equal to 34)

- If intraModeA and intraModeB are the same:

  - If intraModeA is PLANAR or DC, then the three MPMs are set to {PLANAR, DC, VERTICAL} in that order

  - Otherwise, the three MPMs are set to {intraModeA, PLANAR, DC} in that order

- Otherwise (intraModeA and intraModeB are different):

  - The first two MPMs are set to {intraModeA, intraModeB} in that order

  - Uniqueness of PLANAR, DC and VERTICAL is checked in that order against the first two MPM candidate modes; as soon as a unique mode is found, it is added to as the third MPM

# Proposed Implementation

We propose to remove the MPM process in CIIP and directly signal the intra modes of CIIP using the truncated unary (TU) codewords as shown in the table below. The first bin is context coded and the rest bins are bypass coded.

Table 1. Proposed codewords for the CIIP intra modes

| Modes | General Shape |  |  | Wide Shape |  | Narrow Shape |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Planar | 1 |  |  | 1 |  | 1 |  |
| DC | 0 | 1 |  | 0 | 1 | 0 | 1 |
| HOR | 0 | 0 | 1 | - | - | 0 | 0 |
| VER | 0 | 0 | 0 | 0 | 0 | - | - |

# Results

The proposed modification is integrated into VTM-3.0 [1] and the results of the proposed implementation are shown below against the VTM3.0 under CTC [2].

Table 2. Performance of the proposed modification under Random Access

Table 3. Performance of the proposed modification under Low delay B

# Conclusion

In this proposal, the MPM derivation process is removed from intra mode coding for CIIP. Truncated unary codewords are used to signal the intra modes for CIIP. The results reported show that -0.01% and 0.01% BD-rate increases are incurred with the proposed simplification. It is recommended to adopt the proposed modifications into VVC for CIIP mode.

# Reference

- VTM-3.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-L1010, October 2018.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).