| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0233-v1 |
| --- | --- |

| Title: | Non-CE10: Triangle prediction merge list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianglin Wang Yi-Wen Chen | Email: | xianglinwang@kwai.com yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, some modifications are proposed on the merge candidate list construction process for triangle prediction mode. Firstly, it is proposed to use the regular merge candidate list motion vectors for deriving the uni-prediction merge list for triangle prediction. Secondly, L0 and L1 motion vectors from the regular candidate list are selected in a simple interleaving manner into the triangle merge list. The reported results show -0.05% and 0.02% BD-rate impact with the proposed simplifications for RA and LDB conditions respectively.

# Introduction

In VTM-3.0[1], triangle prediction can be used to enable triangular shape prediction units for inter prediction. It is currently supported under merge mode, with its own merge candidate list construction process that is different from the regular merge candidate list construction.

Figure 1. Motion vectors used for triangle merge list construction

Figure 1 shows the block locations where motion vectors are derived for triangle merge list construction， where block 1 to 5 are spatial neighboring block and block 6 and 7 are collocated temporal block.

Currently these up-to seven motion vectors are first put into a list. To form a uni-prediction merge list for triangle prediction, the neighboring motion vector list is scanned, and uni-prediction motion vectors are first selected into the merge list. If the merge list is not full, L0 motion vectors of the bi-prediction motion vectors in the neighboring motion vector list are added into the merge list. Then L1 motion vectors of those bi-prediction motion vectors are added. After that, the average motion vector of L0 and L1 motion vectors are added. At last, uni-prediction zero vectors may be added to make the merge list full.

Overall, the merge list construction process is quite different from the regular merge list construction.

# Proposed method

According to this proposal, to maximize the common operations shared by different merge modes, merge list for triangle prediction is constructed from the regular merge list motion vectors.

Once the regular merge list is available, for each candidate in the regular merge list, its L0 motion vector and L1 motion vector are added in an interleaving manner into the uni-prediction merge list. More specifically, if a candidate in the regular merge list is uni-prediction motion vector, it is added directly into the uni-prediction merge list. Otherwise, for a bi-directional prediction motion vector in the regular merge list, its L0 motion vector is first added into the uni-prediction merge list, followed by its L1 motion vector.

# Results

The proposed modification is integrated into VTM-3.0 and the results of the proposed implementation under CTC [2] are shown below against VTM3.0.

Table 1. Performance of the proposed modification under Random Access

Table 2. Performance of the proposed modification under Low delay B

# Conclusion

In this proposal, a different merge list construction process was proposed for triangle prediction. The proposed scheme reuses the regular merge list, followed by a simple process to form the uni-prediction merge list for triangle prediction. The reported results show nearly no performance impact with the proposed simplifications. It is recommended to consider adoption of the proposed scheme into VVC for triangle prediction.

# Reference

- VTM-3.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-L1010, October 2018.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).