| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0235-v2 |
| --- | --- |

| Title: | CE13: HEC with Pre-rotation based on test 1.1a and 1.1b (Test 4.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akhil Konda Chirag Pujara Anubhav Singh | Email: | akhil.konda@samsung.com chirag.p@samsung.com anubhav.23@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports simulation result of CE13 test 4.2 Hybrid Equiangular Cubemap (HEC) implementation over 360Lib8.0 merged with VTM3.0. This test is implemented with both test 1.1a (HEC with in-loop filters disabled) and 1.1b (PHEC with in loop filter disabled). Test 4.2a shows the gain of {-0.96%, -1.54%, -1.33%} over CTC anchor and {-0.95%, -1.25%, -1.20%} over CE anchor (test 1.1b), while Test 4.2b shows the gain of {-0.40%, -1.18%, -1.32%} over CTC anchor and {-0.39%, -0.89%, -1.19%} over CE anchor (test 1.1b) under 360 Common Test Condition (CTC) for E2EWSPSNR. Gains are observed majorly in 8k sequences while 6k sequences did not show any loss. We have used a coarse motion estimation algorithm that decides if rotation need to be applied or not.  Maximum gain is observed for Gaslamp sequence, which is around -2.71% for test 4.2b.

# Introduction

Simulations were performed under 360 video CTC [JVET-L1012] using the code uploaded at https://vcgit.hhi.fraunhofer.de/JVET-L-CE13/VVCSoftware_VTM/tree/CE13.4.2. The simulations were run with HEC implementation over 360Lib8.0 merged with VTM3.0 software.

The tests performed are as follows:

- 4.2 PHEC with pre-rotation: Rotation is implemented over base HEC(CE-13.1.1a)/PHEC (CE-13.1.1b). An algorithm to compute best rotation angle is implemented as described in CE13 document [JVET-L1033] and also in the CfP document [JVET-J0024/0025] and the results are compiled for VTM3.0 software.

# Analysis

(a)                                                                             (b)

(c)                                                                                           (d)

Figure 1. Frame from Gaslamp sequence and its edge image. (a) image with no rotation, (b) its edge image, (c) image with optimal rotation (140 degrees), (d) its edge image

- (b)

(c)                                                                              (d)

Figure 2. Frame from Landing sequence and its edge image. (a) Image with no rotation, (b) its edge image, (c) image with optimal rotation (170 degrees), (d) its edge image

From the image 1, 2 it can be seen that the edges in the image are aligned to horizontal and vertical directions in case of optimal rotation angle which improves the coding gain.

- (b)

(c)                                                                                        (d)

Figure 3. BD-rate and Horizontal + vertical edge percentage w.r.t. rotation from 0 to 360 degrees, (a) averaged over full  Landing2 sequence, (b) averaged over first RAP of Landing2 sequence, (c) averaged over full sequence of Broadway and (d) averaged over first RAP of Broadway

As per our analysis for the loss in case of BD-rate gain for the Landing2 sequence is due to very high camera motion with in the sequence. It can be seen from image 2 that the BD-rate curve is not symmetric over rotation (which is the case for other sequences like Balboa, Broadway in 6k category). This makes the algorithm less appropriate for this kind of sequence.

As this algorithm analyses only first frame, we added an additional check if the rotation should be performed or not based on the motion estimation from the I frame of first and second RAP. If the SAD between these two frames is above some threshold then we do not perform rotation. With this condition, we can avoid the compression losses for 6k sequences.

# Experimental Results

Snapshot of experimental results is as follows,

- CE 13.4.2b vs CTC anchor (PHEC)

- CE 13.4.2a vs CTC anchor (PHEC)

- CE 13.4.2b vs CE anchor (PHEC + In-loop filters disabled)- Test 13.1.1b

- CE 13.4.2a vs  CE anchor (PHEC + In-loop filters disabled)- Test 13.1.1b

# Observations

Rotation provides gain of around 0.7% for 8k sequences and we don’t see any gain for 6k sequences. Highest gain is observed for Gaslamp sequence around 2.71% for the Luma component.

We propose to add rotation signaling in SEI information in VVC.

# Cross check

The cross-check has been performed by P. Hanhart from InterDigital.

# References

- P. Hanhart, J.-L. Lin, C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, JVET-L1033, Oct. 2018, Macau, CN.

- P. Hanhart, J. Boyce, K. Choi, J.-L. Lin “JVET common test conditions and evaluation procedures for 360° video”, JVET-L1012, Oct. 2018, Macau, CN.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).