| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0239-v1 |
| --- | --- |

| Title: | Non-CE3: Modification of MPM derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Hahyun Lee Sung-Chang Lim Jungwon Kang Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr jungwon@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution presents a modification method of most probable mode (MPM) derivation when luma reference line index is not equal to zero. It reduces number of condition checks to derive 6 MPM candidates. It is reported that the average luma BD-rate for AI and RA case is 0.05% and 0.02% compared to VTM 3.0, respectively.

Introduction

The MPM derivation is different according to luma reference line index as described in Fig. 1. After deriving neighbouring intra prediction mode of A and B, 6 MPMs are derived by checking whether the A and B are same or whether the neighbouring mode is an angular mode. When luma reference line index is not equal to zero MPM candidates are derived only using angular prediction modes. And distance of two angular modes from neighbouring blocks is checked to derive non-duplicated angular modes.

Figure 1. MPM derivation process in VTM 3.0

- Algorithm description

In this contribution, a modification of MPM derivation is proposed to reduce complexity as described in Fig. 2. When luma reference line index is not equal to zero candidate mode C is derived by comparing A and B. If the mode C is an angular mode, adjacent angular modes are included in MPM list, and if the mode C is a non-angular mode, default modes [50, 18, 34, 66, 2, 26] are included.

Figure 2. MPM derivation process in proposed method

Experimental results

Experiments were performed based on VTM 3.0 software [1] under JVET common test condition [2].

Table 1 shows experimental results of the proposed method compared to VTM 3.0.

Table 1. Results of the proposed method over VTM 3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.01% | 101% | 100% |
| Class A2 | 0.03% | -0.01% | 0.00% | 101% | 100% |
| Class B | 0.07% | 0.04% | 0.01% | 101% | 100% |
| Class C | 0.09% | 0.04% | -0.04% | 100% | 100% |
| Class E | 0.04% | 0.04% | 0.12% | 100% | 100% |
| Overall | 0.05% | 0.02% | 0.02% | 101% | 100% |
| Class D | 0.03% | 0.17% | 0.11% | 100% | 100% |
| Class F (optional) | 0.19% | 0.15% | 0.23% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.11% | 0.08% | 100% | 100% |
| Class A2 | 0.04% | 0.09% | 0.16% | 100% | 100% |
| Class B | 0.02% | 0.15% | -0.08% | 100% | 100% |
| Class C | 0.03% | -0.14% | 0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.05% | 0.05% | 100% | 100% |
| Class D | 0.00% | -0.26% | -0.27% | 100% | 100% |
| Class F (optional) | 0.15% | 0.05% | 0.19% | 100% | 100% |

Conclusions

This contribution reports a modification method of MPM derivation when luma reference line index is not equal to zero. The average luma BD-rates is marginal when the proposed method is applied, and encoding/decoding time is almost same. We suggest the proposed method to be adopted into VTM 4.0.

Reference

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, November 2018.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).