- Decoding process (Based on JVET-L1001-v7)

- 8.2.2 Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and mh_intra_flag[ xNbX ][ yNbX ] is not equal to 1.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..5 is derived as follows:

- If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-4)

candModeList[ 1 ] = INTRA_PLANAR	(8-5)

candModeList[ 2 ] = INTRA_DC		(8-6)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-7)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-8)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-9)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-10)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-11)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-12)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-13)

candModeList[ 4 ] = 2 + ( candIntraPredModeA % 64 )	(8-14)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 59 ) % 64 )	(8-15)

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-1610)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-1711)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-1812)

candModeList[ 1 ] = candIntraPredModeB	(8-1913)

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 2 ] = INTRA_PLANAR	(8-2014)

candModeList[ 3 ] = INTRA_DC		(8-2115)

  - If maxAB − minAB is in the range of 2 to 62, inclusive, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-2216)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-2317)

  - Otherwise, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-2418)

candModeList[ 5 ] = 2 + ( ( maxAB ) % 64 )	(8-2519)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

  - If maxAB − minAB is equal to 1, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-26)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-27)

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-28)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-29)

  - Otherwise if maxAB − minAB is equal to 2, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-30)

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-31)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-32)

candModeList[ 5 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-33)

  - Otherwise if maxAB − minAB is greater than 61, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-34)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-35)

candModeList[ 4 ] = 2 + ( minAB % 64 )	(8-36)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-37)

  - Otherwise, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-38)

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-39)

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-40)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-41)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-4220)

candModeList[ 1 ] = candIntraPredModeB	(8-4321)

candModeList[ 2 ] = 1 − minAB		(8-4422)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-4523)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-4624)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-4725)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = maxAB		(8-48)

candModeList[ 1 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-49)

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-50)

candModeList[ 3 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-51)

candModeList[ 4 ] = 2 + ( maxAB % 64 )	(8-52)

candModeList[ 5 ] = 2 + ( ( maxAB + 59 ) % 64 )	(8-53)

- Otherwise, the following applies:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-5426)

candModeList[ 1 ] = ( candModeList[0]  = =  INTRA_PLANAR )  ?  INTRA_DC  :  	(8-5527) 
		INTRA_PLANAR

candModeList[ 2 ] = INTRA_ANGULAR50	(8-5628)

candModeList[ 3 ] = INTRA_ANGULAR18	(8-5729)

candModeList[ 4 ] = INTRA_ANGULAR46	(8-5830)

candModeList[ 5 ] = INTRA_ANGULAR54	(8-5931)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = INTRA_ANGULAR50	(8-60)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-61)

candModeList[ 2 ] = INTRA_ANGULAR2	(8-62)

candModeList[ 3 ] = INTRA_ANGULAR34	(8-63)

candModeList[ 4 ] = INTRA_ANGULAR66	(8-64)

candModeList[ 5 ] = INTRA_ANGULAR26	(8-65)

- Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

- Candidate intra prediction mode candIntraPredModeC is derived as follows:

candIntraPredModeC = candIntraPredModeA > candIntraPredModeB ? candIntraPredModeA : 
	                                                    candIntraPredModeB  (8-32)

- If candIntraPredModeC is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeC		(8-33)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeC + 61 ) % 64 )		(8-34)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeC − 1 ) % 64 )		(8-35)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeC + 60 ) % 64 )		(8-36)

candModeList[ 4 ] = 2 + ( candIntraPredModeC % 64 )		(8-37)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeC + 59 ) % 64 )		(8-38)

- Otherwise, the following applies:

candModeList[ 0 ] = INTRA_ANGULAR50		(8-39)

candModeList[ 1 ] = INTRA_ANGULAR18		(8-40)

candModeList[ 2 ] = INTRA_ANGULAR34		(8-41)

candModeList[ 3 ] = INTRA_ANGULAR66		(8-42)

candModeList[ 4 ] = INTRA_ANGULAR2		(8-43)

candModeList[ 5 ] = INTRA_ANGULAR26		(8-44)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candModeList[ i ] is greater than candModeList[ j ] for i = 0..4 and for each i, j = ( i + 1 )..5, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-66)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].