| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9-18 Jan. 2019 | Document: JVET-M0244-v1 |
| --- | --- |

| Title: | CE6 : MTS using DST-4 and transposed DCT-2 (test 6-1.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin Jianhua Zheng Quanhe Yu Na Zhang Ce Zhu | Email: | linyongbing@hisilicon.com zhengjianhua@hisilicon.com yuquanhe@hisilicon.com zhangna25@hisilicon.com eczhu@uestc.edu.cn |
| Source: | HiSilicon Technologies Univ. of Electronic Science and Technology of China |  |  |

_____________________________

- Abstract

This contribution reports the test results of CE6-1.3a/b on low-complexity MTS approach, by using DST-4 and transposed DCT-2 as primary transform to replace DST-7 and DCT-8 in VTM-3.0. With the proposed approach, it is reported that all the transform cores used in VVC can be extracted from one single 64-point DCT-2 transform matrix. The extracted DCT-2 transform cores from 4-point to 32-point are fully compatible with those in HEVC standard. Meanwhile, partial butterfly implementation can be applied to the transposed DCT-2 transform. Test results reportedly show coding performance impact of -0.03% ~ 0.22% on average, and decoding time decreases by ~7% for CTC and CTC+interMTS conditions.

In addition, test results of combination between CE6-1.3 and CE6-1.1c are also reported, where 64-point transform is replaced by COT and 32-point DST-7 transform embedded in the COT can be applied.

Introduction

In MTS as described in VVC darft3 [1], besides DCT-2, two additional primary transform types of DST-7 and DCT-8 are used. These additional transforms require additional memory for storing the transform cores. Meanwhile the DST-7 and DCT-8 transforms are lacking of fast algorithm, e.g. partial butterfly implementation. In the last JVET meeting, JVET-L0353 was proposed to use DST-4 and transposed DCT-2 in replacement of DST-7 and DCT-8 [2]. In this contribution the approach proposed in JVET-L0353 is further tested on top of VTM-3.0 as described in CE6-1.3 [3].

Table 1: Descriptions on CE6 Test CE6-1.3a and CE6-1.3b

| Test # | Description |
| --- | --- |
| CE6-1.3a | MTS: DST-4 and DCT-2 transpose Max MTS size: 32 |
| CE6-1.3b | 32-point MTS: flipped DCT-2 transpose and DCT-2 transpose 4-, 8- and 16-point MTS: DST-4 and DCT-2 transpose Max MTS size: 32 |

There are two proposed tests for CE6-1.3 as detailed in Table 1.

- CE6-1.3a: DST-4 and transposed DCT-2 are used to replace DST-7 and DCT-8 for all transform sizes from 4-point to 32-point.

- CE6-1.3b: Based on CE6-1.3a, the 32-point DST transform is replaced by horizontally flipped transform of the transposed DCT-2, in order to further enable partial butterfly implementation for 32-point transform cores.

With the proposed approach, all the transform cores can be extracted from on single 64-point DCT-2 transform matrix, so no additional memory is required to store the proposed transform coefficients. At the same time, the extracted DCT-2 transform cores form 4-point to 32-point are fully compatible with those in HEVC standard. Furthermore, partial butterfly implementation can be enabled for the (flipped) transposed DCT-2 transform.

Test results

In this section, results for above two CE tests, including CE6-1.3a and CE6-1.3b, are reported.

According to CE6 description, for each test, two experiments are conducted. The first experiment reports the coding performance under CTC condition [4], as shown in Table 2 and 4. The second experiment reports the coding performance under CTC condition with inter MTS enabled, as shown in Table 3 and 5.

Table 2. Coding performance of Test CE6-1.3a (CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.47% | 0.51% | 0.63% | 97% | 96% |
| Class A2 | 0.28% | 0.19% | 0.16% | 97% | 95% |
| Class B | 0.07% | 0.07% | 0.00% | 98% | 97% |
| Class C | -0.23% | -0.09% | -0.13% | 98% | 99% |
| Class E | -0.05% | -0.02% | -0.02% | 98% | 97% |
| Overall | 0.08% | 0.11% | 0.10% | 98% | 97% |
| Class D | -0.29% | -0.26% | -0.18% | 99% | 98% |
| Class F (optional) | -0.16% | -0.15% | -0.06% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | 0.40% | 0.44% | 100% | 100% |
| Class A2 | 0.12% | 0.32% | 0.25% | 100% | 100% |
| Class B | 0.06% | 0.30% | -0.02% | 99% | 99% |
| Class C | -0.06% | 0.03% | 0.22% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.25% | 0.19% | 100% | 99% |
| Class D | -0.16% | -0.03% | -0.07% | 100% | 99% |
| Class F (optional) | -0.11% | 0.10% | 0.11% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.24% | 0.15% | 100% | 102% |
| Class C | -0.02% | 0.40% | 0.45% | 100% | 101% |
| Class E | -0.12% | 1.13% | 0.47% | 100% | 101% |
| Overall | -0.02% | 0.32% | 0.33% | 100% | 101% |
| Class D | -0.02% | -0.19% | 0.36% | 100% | 99% |
| Class F (optional) | 0.11% | -0.21% | 0.32% | 100% | 99% |

Table 3. Coding performance of Test CE6-1.3a (CTC + inter MTS enabled)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.47% | 0.51% | 0.63% | 97% | 96% |
| Class A2 | 0.28% | 0.19% | 0.16% | 97% | 96% |
| Class B | 0.07% | 0.07% | 0.00% | 98% | 97% |
| Class C | -0.23% | -0.09% | -0.13% | 99% | 97% |
| Class E | -0.05% | -0.02% | -0.02% | 98% | 98% |
| Overall | 0.08% | 0.11% | 0.10% | 98% | 97% |
| Class D | -0.29% | -0.26% | -0.18% | 99% | 100% |
| Class F (optional) | -0.16% | -0.15% | -0.06% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.28% | 0.45% | 0.47% | 99% | 99% |
| Class A2 | 0.27% | 0.21% | 0.21% | 99% | 99% |
| Class B | 0.11% | 0.18% | 0.00% | 99% | 100% |
| Class C | -0.08% | 0.01% | -0.02% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.20% | 0.13% | 99% | 99% |
| Class D | -0.12% | -0.01% | -0.13% | 99% | 98% |
| Class F (optional) | -0.09% | -0.03% | 0.12% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.15% | -0.09% | 99% | 98% |
| Class C | -0.09% | -0.09% | 0.08% | 99% | 97% |
| Class E | 0.00% | 1.01% | 0.14% | 99% | 98% |
| Overall | -0.03% | 0.28% | 0.03% | 99% | 98% |
| Class D | -0.03% | -0.41% | -0.56% | 99% | 98% |
| Class F (optional) | -0.10% | -0.21% | -0.49% | 99% | 98% |

Table 4. Coding performance of Test CE6-1.3b (CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.79% | 1.15% | 1.19% | 96% | 89% |
| Class A2 | 0.36% | 0.40% | 0.34% | 96% | 92% |
| Class B | 0.19% | 0.21% | 0.25% | 97% | 93% |
| Class C | -0.21% | -0.03% | -0.24% | 98% | 97% |
| Class E | 0.15% | 0.15% | 0.29% | 97% | 93% |
| Overall | 0.22% | 0.34% | 0.32% | 97% | 93% |
| Class D | -0.28% | -0.09% | -0.20% | 98% | 97% |
| Class F (optional) | -0.13% | 0.00% | 0.00% | 98% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.46% | 0.69% | 0.77% | 99% | 99% |
| Class A2 | 0.18% | 0.40% | 0.32% | 99% | 100% |
| Class B | 0.16% | 0.42% | 0.31% | 99% | 99% |
| Class C | -0.06% | 0.13% | 0.15% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | 0.39% | 0.36% | 99% | 99% |
| Class D | -0.13% | -0.22% | -0.24% | 100% | 100% |
| Class F (optional) | -0.05% | 0.12% | 0.17% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.01% | 0.16% | 100% | 101% |
| Class C | -0.01% | 0.32% | 0.29% | 100% | 101% |
| Class E | 0.01% | 0.91% | -0.27% | 100% | 103% |
| Overall | 0.03% | 0.34% | 0.10% | 100% | 101% |
| Class D | -0.02% | -0.40% | 0.57% | 100% | 99% |
| Class F (optional) | -0.06% | -0.15% | -0.20% | 100% | 98% |

Table 5. Coding performance of Test CE6-1.3b (CTC + inter MTS enabled)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.79% | 1.15% | 1.19% | 96% | 88% |
| Class A2 | 0.36% | 0.40% | 0.34% | 96% | 92% |
| Class B | 0.19% | 0.21% | 0.25% | 97% | 95% |
| Class C | -0.21% | -0.03% | -0.24% | 98% | 96% |
| Class E | 0.15% | 0.15% | 0.29% | 97% | 93% |
| Overall | 0.22% | 0.34% | 0.32% | 97% | 93% |
| Class D | -0.28% | -0.09% | -0.20% | 98% | 97% |
| Class F (optional) | -0.13% | 0.00% | 0.00% | 98% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.51% | 0.64% | 0.71% | 98% | 97% |
| Class A2 | 0.33% | 0.22% | 0.28% | 98% | 98% |
| Class B | 0.21% | 0.31% | 0.29% | 98% | 98% |
| Class C | -0.09% | 0.19% | -0.09% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.21% | 0.32% | 0.27% | 98% | 98% |
| Class D | -0.12% | -0.52% | -0.30% | 98% | 98% |
| Class F (optional) | -0.01% | -0.03% | 0.14% | 98% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | 0.26% | -0.29% | 98% | 97% |
| Class C | -0.04% | -0.27% | 0.15% | 98% | 94% |
| Class E | 0.17% | 0.56% | -0.06% | 98% | 100% |
| Overall | 0.06% | 0.16% | -0.09% | 98% | 97% |
| Class D | -0.02% | 0.12% | -0.02% | 98% | 96% |
| Class F (optional) | -0.23% | -0.26% | -0.52% | 98% | 96% |

Combination of CE6-1.3 and CE6-1.1c

In this section, additional results of combination of CE6-1.3 and CE6-1.1c are also reported. In CE6-1.1c, Compound Orthonormal Transform (COT) with embedded DST-7 is tested. In the combination test, 64-point DCT-2 is replaced by the COT and 32-point DST-7 transform is still used.

Table 6: combination of CE6-1.3 and CE6-1.1c

| Test # | Description |
| --- | --- |
| Combination test | 64-point DCT-2 is replaced by COT 32-point MTS: DST-7 and DCT-2 transpose 4-, 8- and 16-point MTS: DST-4 and DCT-2 transpose Max MTS size: 32 |

From the test results shown in Table 7 and Table 8, it is observed that overall coding performance is slightly improved in comparison with the test CE6-1.3a.

Table 7. Coding performance of combination of CE6-1.3 and CE6-1.1c (CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.30% | 0.27% | 0.32% | 96% | 100% |
| Class A2 | 0.15% | 0.04% | 0.10% | 96% | 98% |
| Class B | -0.03% | -0.03% | -0.08% | 97% | 97% |
| Class C | -0.24% | -0.18% | -0.14% | 98% | 100% |
| Class E | -0.11% | -0.13% | -0.09% | 97% | 97% |
| Overall | -0.01% | -0.02% | 0.00% | 97% | 98% |
| Class D | -0.32% | -0.15% | -0.13% | 98% | 99% |
| Class F (optional) | -0.18% | -0.10% | 0.02% | 98% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.15% | 0.23% | 99% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.02% | 0.32% | 0.11% | 99% | 96% |
| Class C | -0.06% | 0.02% | 0.29% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.15% | -0.69% | -0.15% | 100% | 99% |
| Class F (optional) | -0.12% | -0.02% | 0.05% | 99% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.02% | 0.17% | 0.26% | 100% | 98% |
| Class E | -0.04% | 1.14% | -0.78% | 100% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.01% | 0.60% | 0.27% | 100% | 101% |
| Class F (optional) | 0.02% | 0.14% | 0.27% | 99% | 100% |

Table 8. Coding performance of combination of CE6-1.3 and CE6-1.1c (CTC+ inter MTS enabled)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.30% | 0.27% | 0.32% | 96% | 97% |
| Class A2 | 0.15% | 0.04% | 0.10% | 96% | 97% |
| Class B | -0.03% | -0.03% | -0.08% | 96% | 98% |
| Class C | -0.24% | -0.18% | -0.14% | 98% | 97% |
| Class E | -0.11% | -0.13% | -0.09% | 97% | 98% |
| Overall | -0.01% | -0.02% | 0.00% | 97% | 97% |
| Class D | -0.32% | -0.15% | -0.13% | 98% | 98% |
| Class F (optional) | -0.18% | -0.10% | 0.02% | 98% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.25% | 0.20% | 0.31% | 98% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.07% | 0.16% | 0.12% | 99% | 99% |
| Class C | -0.09% | 0.08% | 0.10% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.12% | -0.61% | -0.18% | 99% | 99% |
| Class F (optional) | -0.07% | -0.11% | 0.12% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.06% | -0.01% | 0.17% | 98% | 97% |
| Class E | 0.14% | 0.64% | -0.66% | 98% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.07% | -0.25% | 0.06% | 99% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusions

It is proposed to use DST-4 and the transposed DCT-2 as transform cores for MTS. The proposed approach enables all transform cores extracted from one single 64-point DCT-2 transform matrix. Meanwhile, the proposed transform design is fully compatible with those in HEVC standard. Test results show decoding time is reduced by up to 7% due to partial butterfly implementation.

References

- B. Bross, J. Chen, S. Liu, "Versatile Video Coding (Draft 3)", JVET-L1001, 12th Meeting: Macao, CN, 3-12 Oct. 2018

- Y. Lin, J. Zheng, Q. Yu, N. Zhang, C. Zhu, "CE6-related: MTS using DST-4 and transposed DCT-2", JVET-L0353, 12th Meeting: Macao, CN, 3-12 Oct. 2018

- A. Said, X. Zhao, "Description of Core Experiment 6 (CE6): Transforms and transform signaling”, JVET-L1026, 12th Meeting: Macao, CN, 3-12 Oct. 2018

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th Meeting: Macao, CN, 3-12 Oct. 2018

Patent rights declaration(s)

HiSilicon Technologies, Univ. of Electronic Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).