| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0249 |
| --- | --- |

| Title: | Non-CE9: Modifications on Bi-Directional Optical Flow |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Jizheng Xu, Yue Wang, Pengwei Zhao, Dingkun Hong Yingdu Mansion F11, 48 Zhichun Road, Building B, Haidian District, 100086 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC, Bi-directional Optical Flow (BDOF) could be performed for large CUs like 128x128, 128x64 and 64x128 CUs, which requires large memory. Meanwhile, in early termination stage of BDOF, Sum of Absolute Difference (SAD) between two reference blocks shall be calculated, which increases the worst-case computational complexity. This contribution proposes to modify BDOF in two aspects: first, when a CU contains more than N (e.g., N = 4096 or 1024) pixels, it is split into multiple sub-blocks and BDOF is performed for each sub-block; second, SAD is calculated only for some pixels instead of all pixels. Simulation results reportedly show negligible impact on compression performance.

# Introduction

The versatile video coding test model (VTM) [1] employs BDOF to improve prediction of bi-predicted CUs. BDOF could be applied to large CUs like 128x128, 128x64 and 64x128 CUs, which requires large memory. Meanwhile, an early termination method is used in BDOF to reduce the average decoding complexity. SAD between two reference blocks of the CU is calculated as follows: first, SAD is calculated for each 4x4 sub-block, then SAD of all 4x4 sub-blocks is summed up. If SAD between the two reference blocks is smaller than a first threshold, BDOF is skipped for the entire CU; otherwise, for each 4x4 sub-block, if SAD between the two reference 4x4 blocks is smaller than a second threshold, BDOF is skipped for the 4x4 sub-block. Though the early termination method reduces the average decoding complexity, it increases the worst-case computational complexity.

# Proposed method

To address the problems mentioned above, two methods are proposed:

- Method A: If a CU contains more than N pixels, it is split into multiple sub-blocks, each of which contains N pixels. BDOF is performed for each sub-block.

- Method B: SAD is calculated for the 4 corner pixels, instead of all 16 pixels, for each 4x4 sub-block.

In this way, the memory required in BDOF is dramatically reduced by 93.75%. Meanwhile, operations required in SAD calculation is also dramatically reduced, by approximately 75%.

# Experimental results

The proposed methods were implemented on top of reference software VTM-3.0 [1], and simulations were performed following the common test conditions [2]. Results are provided for the following four tests:

- Test #1: Method A with N = 4096.

- Test #2: Method A with N = 1024.

- Test #3: Method B.

- Test #4: Test #2 + Test #3.

The results are shown in Table 1~Table 4.

Table 1: Coding performance of Test #1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.05% | 101% | 99% |
| Class A2 | -0.01% | 0.01% | 0.01% | 100% | 100% |
| Class B | 0.00% | -0.02% | -0.05% | 101% | 100% |
| Class C | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.01% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class F | -0.01% | 0.01% | 0.01% | 100% | 101% |

Table 2: Coding performance of Test #2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | -0.01% | 100% | 101% |
| Class A2 | -0.01% | 0.03% | 0.04% | 100% | 100% |
| Class B | -0.01% | 0.03% | 0.00% | 101% | 100% |
| Class C | -0.01% | 0.03% | -0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | 0.01% | 100% | 100% |
| Class D | 0.01% | -0.02% | 0.05% | 101% | 101% |
| Class F | -0.01% | 0.00% | 0.00% | 101% | 103% |

Table 3: Coding performance of Test #3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.02% | 0.03% | 100% | 101% |
| Class B | 0.00% | 0.03% | -0.06% | 100% | 100% |
| Class C | 0.02% | 0.03% | -0.04% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | -0.02% | 100% | 99% |
| Class D | 0.04% | -0.05% | 0.06% | 101% | 99% |
| Class F | -0.01% | -0.05% | -0.04% | 101% | 100% |

Table 4: Coding performance of Test #4

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.06% | 100% | 101% |
| Class A2 | 0.00% | -0.03% | 0.05% | 100% | 101% |
| Class B | 0.01% | 0.01% | -0.02% | 99% | 100% |
| Class C | 0.01% | 0.00% | -0.03% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.01% | 99% | 100% |
| Class D | 0.05% | -0.09% | 0.03% | 101% | 102% |
| Class F | 0.00% | -0.04% | -0.03% | 99% | 100% |

# Conclusions

This contribution proposes modifications of BDOF, which can dramatically reduce the memory required in BDOF and the operations required in SAD calculation. Meanwhile, it has no impact on compression performance. It is recommended to adopt this proposal into VVC.

# References

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, Doc. JVET-J1010, April 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).