| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0263 |
| --- | --- |

| Title: | CE3: CCLM prediction with single-line neighbouring luma samples (Test 2.6.1 and Test 2.6.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of CE3 Test 3.6.1 and Test 3.6.2. The two tests provide uniform solutions for CCLM predictions regardless whether the current block is at a CTU-row boundary or not. In test 2.6.1, neighbouring luma samples located at one row above and one column left to the current block are required. In test 2.6.2, neighbouring luma samples located at one row above and three columns left to the current block are required. Simulation results reportedly show 0.09% and 0.03% BD-rate loss under All Intra (AI) configurations in the two tests, respectively.

# Introduction

With the CCLM prediction modes in VTM-3.0, the luma samples are down-sampled in different ways when the current CU is inside a CTU-row or at a CTU-row boundary. In the former case, a six-tap down-sampling filter is used so two rows of neighbouring luma samples above the current block and three columns of neighbouring luma samples left to the current block are required. In the latter case, one row of neighbouring luma samples above the current block and three columns left to the current block are required.

# Proposed methods

It is proposed to provide a uniform solution without considering whether the current block is at a CTU-row boundary or not. The down-sampling method is the same for LM/LM-A/LM-L modes.

## Test 2.6.1: Single line of above and left neighbouring samples

To constrain the neighbouring luma samples required by the training process in a single line, down-sampling filters with less taps are applied as shown in Fig. 1:

- For neighbouring luma samples above:

| . | (1) |
| --- | --- |

- For neighbouring luma samples left:

| . | (2) |
| --- | --- |

The luma samples inside the block are still down-sampled with the six-tap filter.

Fig.1 Single-line neighbouring luma samples.

## Test 2.6.2: Single line of above neighbouring samples

In this test, only one row of above neighbouring samples is required as described in test 2.6.1. Three columns of left neighbouring samples are still required as in VTM-3.0.

# Experimental Results

The proposed methods have been implemented on top of VTM-3.0. Results of AI and RA configuration are summarized in Table 1-Table 2 for the two tests. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results for Test 2.6.1

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 1.62% | 1.59% | 99% | 96% | 0.17% | 1.25% | 1.06% | 100% | 99% |
| Class A2 | 0.09% | 0.47% | 0.28% | 99% | 97% | 0.01% | 0.54% | 0.38% | 100% | 100% |
| Class B | 0.04% | 0.48% | 0.78% | 100% | 97% | 0.02% | 0.86% | 1.02% | 100% | 99% |
| Class C | 0.15% | 1.00% | 1.02% | 100% | 98% | 0.06% | 0.88% | 1.21% | 100% | 99% |
| Class E | 0.02% | 0.01% | 0.20% | 99% | 98% |  |  |  |  |  |
| Overall | 0.09% | 0.71% | 0.79% | 99% | 97% | 0.06% | 0.88% | 0.95% | 100% | 99% |
| Class D | 0.08% | 0.69% | 0.77% | 100% | 95% | 0.02% | 0.59% | 0.73% | 100% | 97% |
| Class F | 0.28% | 1.20% | 1.62% | 100% | 99% | 0.15% | 1.00% | 0.98% | 100% | 100% |

Table 2: Results of Test 2.6.2

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.31% | 0.47% | 99% | 96% | 0.04% | 0.30% | 0.38% | 100% | 99% |
| Class A2 | 0.03% | 0.14% | 0.04% | 100% | 97% | -0.03% | 0.40% | 0.11% | 100% | 100% |
| Class B | 0.01% | 0.11% | 0.27% | 100% | 97% | -0.03% | 0.39% | 0.42% | 100% | 99% |
| Class C | 0.04% | 0.21% | 0.17% | 100% | 98% | 0.02% | 0.29% | 0.42% | 100% | 99% |
| Class E | 0.01% | 0.03% | 0.06% | 99% | 98% |  |  |  |  |  |
| Overall | 0.03% | 0.16% | 0.21% | 100% | 97% | 0.00% | 0.35% | 0.35% | 100% | 99% |
| Class D | 0.01% | 0.25% | 0.05% | 100% | 98% | 0.00% | -0.07% | 0.14% | 100% | 95% |
| Class F | 0.05% | 0.22% | 0.30% | 100% | 99% | 0.04% | 0.12% | 0.24% | 100% | 99% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-L1010, “JVET common test conditions and software reference configurations for SDR video”, Oct. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).