| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0269_v1 |
| --- | --- |

| Title: | Non-CE6 : Extension of transform skip block size to 8x8 |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jungah Choi Jin Heo Jangwon Choi Ling Li Jaehyun Lim 19 Yangjaedaero-11gil, Seocho-gu Seoul 06772, South Korea Seunghwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this contribution, the extension of transform skip mode up to 8x8 transform size has been investigated. It is reportedly shown that Y BD-rate changes are -0.03% (AI) and -0.05% (RA) under the common test conditions. The BD-rate changes for Class F and SCC sequences (which are studied in CE8) are -1.88% and -6.81% in AI, and -1.83% and -6.77% in RA, respectively.

Introduction

In the current VTM, the transform skip is enabled to blocks smaller than or equal to 4x4, whereas in HEVC RExt, the transform skip is available from 4x4 to 32x32 transform blocks. Transform skip method has been shown to provide  more efficient coding performance for screen contents, which are artificially generated and do not have noise which has been observed in camera-captured video.

Test results

In the simulation, the maximally allowed block sizes for transform skip is 8x8. Therefore, it is enabled that 4x4, 4x8, 8x4, 8x8 blocks for luma component, and 2x2, 2x4, 2x8, 4x2, 4x4, 4x8, 8x2, 8x4, 8x8 blocks for chroma component are transform skipped.

Table 1 Transform skip enabled up to 8x8 (All intra)

Table 2 Transform skip enabled up to 8x8 (Random access)

Conclusion

In conclusion, it is reported that the transform skip is allowed up to 8x8. With reasonable increase of encoding complexity, the coding efficiency is remarkable on the computer-generated materials. Therefore, it is recommended to include the proposed transform skip extension up to 8x8 for next version of VTM/VVC.

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- R. Joshi, S. Liu, G. Sullivan, G. Tech, Y.-K. Wang, J. Xu, Y. Ye, "HEVC Screen Content Coding Draft Text 6," JCTVC-W1005, 23rd Meeting: San Diego, USA, 19–26 February 2016.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).