| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0270-v2 |
| --- | --- |

| Title: | CE2-related: An alternative storing method for affine inheritance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, two methods are proposed to modify the affine inheritance process. Method 1 is proposed to address the line-buffer memory issue. It is proposed to inherit the affine model from MVs of a neighbouring 4×4 block and its adjacent 4×4 block. Therefore, the side information required to be stored in the line-buffer, including the CU width and the x-component of the CU top-left coordinate, are totally removed from line buffer. Method 2 is proposed to address the In-CTU-buffer memory issue. It is proposed to store affine parameters instead of CPMVs, CU top-left coordinate, CU width and CU height. It is reported that the additional line-buffer memory required by affine inheritance can be reduced from 8192 bits (or 2K bytes in a byte-alignment implementation) to 0. And the additional In-CTU-buffer memory required by affine inheritance can be reduced by about 70%. Simulation results reportedly show 0.01%, 0.01% and 0.02% BD-rate changing under Random Access (RA) configuration with Method 1, Method 2 and Method 1+2, respectively.

# Introduction

At the Macau meeting, JVET-L0046/JVET-L0047/JVET-L0694 were adopted to cleanup the affine motion vector storage and reduce the extra memory required by the affine inheritance. With the current design in VTM-3.0, affine inheritance is conducted in two different ways, depending on whether affine inheritance crosses the CTU rows or not.

## Affine motion derivation

With the affine model, a MV (mvh, mvv) at the position (x, y) can be derived as

(1)

with the 4-parameter affine model, or

(2)

with the 6-parameter affine model, wherein MVbase (mvhbase, mvvbase) is a base MV at a base position (xbase, ybase), and (a, b), (a, b, c, d) represent affine parameters for the 4-parameter affine model and 6-paramter affine model, respectively. The affine parameters can be calculated as:

, 									(3)

, , 								(4)

,									(5)

where MV0 (mv0h, mv0v), MV1 (mv1h, mv1v) and MV2 (mv2h, mv2v), represent three Control Point MVs (CPMVs) at position (x0, y0), (x1, y1) and (x2, y2), respectively. (x0, y0), (x1, y1) and (x2, y2) are usually set to be the top-left, top-right and bottom-left corner of an affine coded block with size equal to w×h, so Lx is set to be w and Ly is set to be h.

It should be noted that the base MV is not necessary to be one of the CPMVs, although it is set to be the CPMV MV0 at the top-left corner (x0, y0) in VTM-3.0.

## Affine inheritance crossing CTU rows

Fig. 1 Affine inheritance crossing CTU rows.

When the affine model of the current CU is inherited from a neighbouring 4×4 block in the above CTU row, MVs stored in the regular MV buffer are accessed. Fig. 1 shows an example when the current CU applies affine merge mode, inheriting the affine model from a neighbouring block B0. In this case, the codec needs to store the width of the CU covering B0 (Wn in Fig. 1), MVs of the bottom-left 4×4 block (N0) and bottom-right 4×4 block (N1) of the CU (MVN0 and MVN1 in Fig. 1), and the base position which is the bottom-right coordinator (xN0, yN0) of N0. In this example, MVN0 is used as the base MV.

## Affine inheritance inside a CTU row

When the affine model of the current CU is inherited from a neighbouring 4×4 block in the current CTU row, CPMVs stored in a special CPMV buffer are accessed. The current CU can inherit the 4-parameter or the 6-parameter affine model from the neighbouring CU.

## Problem statements

There are four problems in the current design of affine inheritance.

- Line-buffer size is increased.

For a 4×4 block at line-buffer, the width Wn and the x-component of the bottom-left coordinate xN0 of the CU containing the 4×4 block must be stored. For each 8×8 block, 3 bits for Wn and 5 bits for xN0 are required to be stored. For an 8K sequence, the line buffer size is increased by 8K bits (or 2K bytes in a byte-alignment implementation) in total.

- Long-distance memory accessing.

At the CTU-row boundary, the affine inheritance design requires to access 96 4×4 blocks from LL to RR instead of 34 4×4 blocks from B2 to R without affine inheritance, which means that additional information with size 62 * 72 = 4464 bits (or 62 * 10 = 620 bytes in a byte-alignment implementation) needs to be loaded on local memory or cache.

- In-CTU-buffer size is increased.

For a 4×4 block inside a CTU, three CPMVs for two reference lists, the width, the height and the top-left coordinate of the CU containing the 4×4 block must be stored. Therefore, 2×3 bits for width and height, 2×5 bits for the coordinate, and 2×3×2×16 bits for CPMVs, are required to be stored for each 8×8 block inside a CTU. Totally, the In-CTU-buffer is increased by 48×(6+10+192) = 9984 bits (or 48×(2+2+24) = 1344 bytes in a byte-alignment implementation). There are 48 8×8 units to be stored as suggested by [2].

- Redundant computation.

First, the affine parameters are unnecessarily derived several times. The affine parameters are calculated for an affine-code CU when it is coded. And those parameters are re-calculated whenever coding a proceeded block which inherits affine model from this block. Besides, the affine parameters are unnecessarily calculated twice: from the neighbouring CU and from the CPMVs of the current CU for the affine-inheritance merge mode.

Second, the derivation of the CPMVs of the current CU is unnecessary for the affine-inheritance merge mode. The MVs of sub-blocks should be calculated directly.

# Proposed methods

Two methods are proposed in this contribution. Method 1 addresses the problems in affine inheritance crossing CTU rows, while method 2 addresses the problems in affine inheritance inside a CTU.

## Mothed 1: Affine inheritance crossing CTU rows

It is proposed to remove the storage of the CU width and the x-component of the bottom-left coordinate of each 8×8 block from line-buffer. When the current CU applies affine inheritance from a neighbouring 4×4 block such as B0 as Fig. 1, the 4×4 block right-next to B0, or left-next to B0 which is also affine-coded and has the same reference index as B0, is chosen as B0’. The MVs stored in B0 and B0’ are accessed as MVB and MVB’, MV0 and MV1 are set to be MVB and MVB’ to derive a and b by eq. (3) with Lx=4. CPMVs of the current CU are derived by eq. (1) with the center position of B0 as the base position and MVB as the base MV.

With the proposed method, at most 36 4×4 blocks from B3 to R2 as shown in Fig. 1 may be accessed at CTU-row boundary. The additional information needs to be loaded on cache is reduced from 4464 bits to 2 * 72 = 144 bits (or 2*10 = 20 bytes in a byte-alignment implementation).

## Mothed 2: Affine inheritance inside a CTU row

It is proposed to store affine parameters instead of three CPMVs and the block dimensions. When the current CU applies affine-inheritance merge mode, affine parameters are directly copied from the neighbouring 4×4 block B to be inherited. And the MV of each sub-block in the current CU is derived by eq. (2) with the center position of B as the base position and MVB as the base MV. When the current CU applies affine AMVP mode, CPMVs of the current CU are derived by eq. (2) also with the center position of B as the base position and MVB as the base MV, and the derived CPMVs will serve as the MVPs.

Each parameter is stored as an 8-bit signed integer. So 2×4×8 = 64 bits are required to be store affine parameters in each 8×8 block inside a CTU.

With the proposed method, the In-CTU-buffer is increased by 48×64 = 3072 bits (or 48×8 = 384 bytes in a byte-alignment implementation).

With the proposed method, one set of affine parameters are calculated at most only once.

## Summary of Storage analysis

The additional storage required by affine inheritance in VTM-3.0 and with the proposed methods are summarized in Table 1.

Table 1: Summary of storage analysis

|  | Line buffer size | MV in cache size | In-CTU buffer Size |
| --- | --- | --- | --- |
| VTM-3.0 | 8k bits | 4464 bits | 9984 bits |
| Proposed | 0 | 144 bits | 3072 bits |
| Proposed/VTM-3.0 | 0% | 3.2% | 30.8% |

# Experimental Results

The proposed methods have been implemented on top of VTM-3.0. Test 1, Test 2 and Test 3 are conducted to verify Method 1, Method 2 and Method 1 + Method 2, respectively. Results of RA/LB configuration are summarized in Table 2-Table 4 for the three tests. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 2: Results for Test 1

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.03% | 0.09% | 99% | 99% |  |  |  |  |  |
| Class A2 | 0.02% | 0.04% | 0.08% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.06% | -0.04% | 99% | 95% | 0.07% | -0.19% | -0.01% | 100% | 100% |
| Class C | 0.01% | -0.03% | 0.02% | 100% | 95% | 0.07% | 0.26% | 0.21% | 100% | 103% |
| Class E |  |  |  |  |  | 0.03% | -0.17% | 0.12% | 100% | 98% |
| Overall | 0.01% | 0.02% | 0.02% | 100% | 97% | 0.06% | -0.03% | 0.10% | 100% | 101% |
| Class D | -0.01% | -0.02% | -0.10% | 100% | 96% | 0.05% | 0.02% | -0.07% | 99% | 97% |
| Class F | -0.02% | -0.01% | 0.04% | 100% | 97% | 0.00% | -0.03% | -0.06% | 100% | 98% |

Table 3: Results for Test 2

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.03% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.05% | 0.05% | 0.13% | 100% | 100% |  |  |  |  |  |
| Class B | -0.01% | 0.05% | -0.09% | 100% | 97% | 0.01% | -0.18% | -0.05% | 100% | 100% |
| Class C | -0.01% | 0.04% | 0.06% | 101% | 98% | 0.00% | 0.27% | 0.03% | 100% | 101% |
| Class E |  |  |  |  |  | 0.02% | 0.13% | -0.18% | 99% | 99% |
| Overall | 0.01% | 0.03% | 0.02% | 100% | 98% | 0.01% | 0.05% | -0.05% | 100% | 100% |
| Class D | -0.01% | 0.01% | -0.04% | 101% | 93% | 0.01% | -0.22% | 0.40% | 99% | 99% |
| Class F | 0.00% | 0.02% | 0.05% | 101% | 97% | 0.06% | 0.12% | -0.45% | 100% | 99% |

Table 4: Results for Test 3

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.03% | 0.04% | 100% | 99% |  |  |  |  |  |
| Class A2 | 0.07% | 0.11% | 0.18% | 100% | 100% |  |  |  |  |  |
| Class B | 0.01% | 0.08% | -0.03% | 99% | 95% | 0.09% | -0.24% | -0.03% | 99% | 101% |
| Class C | 0.02% | 0.04% | 0.03% | 100% | 95% | 0.03% | 0.29% | -0.10% | 100% | 102% |
| Class E |  |  |  |  |  | 0.14% | 0.36% | 0.24% | 100% | 97% |
| Overall | 0.02% | 0.05% | 0.04% | 100% | 97% | 0.08% | 0.09% | 0.01% | 100% | 100% |
| Class D | 0.03% | -0.02% | 0.05% | 100% | 102% | 0.06% | 0.36% | 0.39% | 99% | 104% |
| Class F | 0.01% | 0.00% | 0.03% | 99% | 99% | -0.01% | 0.24% | 0.19% | 101% | 98% |

# Conclusions

This contribution proposes two methods to change the MV derivation and storage method for affine inheritance. Both aspects are beneficial for hardware/software implementation with almost identical performance. It is recommended to adopt these two aspects into the next version of VVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-L1010, “JVET common test conditions and software reference configurations for SDR video”, Oct. 2018.

- M. Zhao, B. Heng, JVET-L0047, “CE4-related: A clean up for affine mode”, Oct. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).