| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0272 |
| --- | --- |

| Title: | CE4-related: Restrictions on History-based Motion Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Kai Zhang, Hongbin Liu, Jizheng Xu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Ln, San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com zhangkai.video@bytedance.com liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Two restrictions are proposed for the History-based Motion Vector Prediction (HMVP) design. Firstly, for 4×4 blocks, HMVP candidates are excluded from the regular merge list and the associated motion information is disallowed to update the HMVP table. Secondly, whether motion information of one block could be added to the HMVP table further depends on the category of decoded merge candidate. If the merge candidate is a virtual merge candidate (i.e., candidates inserted after the HMVP candidates), such motion information could be utilized to update the HMVP table. With proposed restrictions, it is asserted that the worst case complexity of regular merge list construction process could be reduced and the bottleneck is changed from 4 × 4 blocks to larger blocks in terms of MV throughput. Simulation results reportedly show that the restricted HMVP achieves 0.00% and 0.02% BD rate reduction for RA Main10 and LDB Main 10 configurations, respectively.

# Introduction

The History-based Motion Vector Prediction (HMVP) was adopted in the 3rd VVC meeting [1]. In HMVP, a HMVP table is maintained to store non-duplicated motion information of previously coded blocks. When decoding a block, the HMVP candidates could be treated as merge candidates for the current block in addition to conventional spatial and temporal merge candidates. After decoding the block, the decoded motion information may be further added to the HMVP table with redundancy removal.

More specifically, the regular merge list (i.e., non-affine, non-triangular merge list) construction process includes the following sequential of steps:

- Step 1: Initial candidates derivation

  - Step 1.1: Spatial merge candidates derivation

  - Step 1.2: Redundancy check/removal for spatial candidates (up to 4 times of pruning operations)

  - Step 1.3: Temporal merge candidates derivation

  - Step 1.4: HMVP candidates with redundancy check/removal (up to 10 times of pruning operations [1])

- Step 2: Virtual merge candidates insertion

  - Step 2.1: Creation of pairwise bi-predictive candidates

  - Step 2.2: Insertion of default motion candidates (zero motion candidates)

Once the number of available merge candidates reaches the maximumly allowed value (e.g., 6 in the common test conditions), there is no need to further add the remaining merge candidates.

It is noticed that the worst case for the regular merge mode happens under all the following conditions are both true:

–	the current coding unit (CU) has the smallest CU size (4×4 in current common test conditions),

–	the decoded merge candidate is virtual merge candidate

–	the decoded merge candidate is different from any existing HMVP candidates or identical to the last HMVP candidate checked in the HMVP table

For the worst case, there are 20 times pruning operations, including 5 (redundancy check of spatial merge candidates), 10 (redundancy check of HMVP candidates in the merge list construction process) and 5 (redundancy check in HMVP table updating process).

Even in current VVC design, the triangular merge list construction process requires even larger number of pruning operations compared to the regular merge list construction process. It is still desirable to further reduce the worst case complexity for the regular merge list construction process.

# Proposed methods

To reduce the worst case complexity for the regular merge list construction process, the following two restrictions are proposed:

- For 4×4 blocks, their motion information is disallowed to update the HMVP table, and HMVP candidates are excluded from the regular merge list;

- For the virtual merge candidates (such as pairwise bi-predictive candidates and zero motion candidates), they are disallowed to update the HMVP table.

With the first restriction, the bottleneck of 4×4 blocks merge list construction process and HMVP table updating process of 4×4 blocks is removed. That is, for 4×4 blocks, the worst case of pruning operations is kept to be the same as without HMVP, i.e., 5 times.

With the second restriction, it could be known that for blocks larger than 4×4, the number of pruning operations could also be reduced. The detailed analysis for each case is provided in the table below.

Table 1. Analysis of pruning operations for different merge candidate category (for the worst case)

| Category of decoded merge candidate | No. of pruning operations for spatial merge candidates | No. of pruning operations for HMVP candidates in the regular merge list construction process | No. of pruning operations in HMVP table updating process | Total No. of pruning operations |
| --- | --- | --- | --- | --- |
| spatial or temporal merge | 5 | 0 | 5 | 10 |
| HMVP candidate | 5 | 10 | 0 | 15 |
| Virtual merge candidate | 5 | 10 | 0 | 15 |

Based on the above analysis, the number of pruning operations could be reduced from 20 to 15 for the worst case of the regular merge list. More important, the bottleneck is changed from 4 × 4 blocks to larger blocks.

# Experimental Results

The proposed method is implemented on VTM-3.0 [2]. The simulations were performed following the JVET CTC [3]. Results of Random Access (RA) configuration and Low-delay B (LB) configuration are summarized in the Table 2. More detail results can be found in the attached spreadsheets.

Table 2. Results of proposed methods compared to VTM-3.0

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | 0.05% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.03% | 0.03% | 0.10% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | -0.01% | -0.02% | 100% | 100% | 0.02% | -0.28% | 0.01% | 100% | 98% |
| Class C | 0.03% | -0.01% | -0.08% | 101% | 100% | -0.02% | 0.29% | 0.11% | 100% | 97% |
| Class E |  |  |  |  |  | -0.10% | -0.21% | -0.31% | 100% | 96% |
| Overall | 0.00% | -0.01% | 0.00% | 101% | 101% | -0.02% | -0.08% | -0.04% | 100% | 97% |
| Class D | -0.04% | -0.15% | 0.01% | 100% | 100% | -0.06% | -0.02% | -0.20% | 100% | 101% |
| Class F | 0.04% | 0.00% | 0.06% | 101% | 100% | -0.02% | 0.09% | 0.62% | 100% | 99% |

# Conclusions

In this contribution, two restrictions on the HMVP method are proposed to reduce the worst case complexity of the regular merge candidate list construction process and change the bottleneck of MV throughput from 4×4 to larger blocks. It is recommended to consider adoption of the proposed methods to the next version of VTM.

# Reference

- “CE4: History-based Motion Vector Prediction (Test 4.4.7) ”, L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong, Doc. JVET-L0266, Oct. 2018.

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, Doc. JVET-J1010, April 2018.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).