| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0280-v3 |
| --- | --- |

| Title: | CE6-related: Context selection for entropy coding the MTS flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modified method for entropy coding the syntax element tu_mts_flag for indicating the use of the multiple primary transform selection. The context modeling scheme for coding tu_mts_flag in VVC working draft 3 is conditioned on the quad-tree depth of the current CU. Some of assigned contexts are not efficiently used under the current CU size constraints. In the proposed method, the context modeling scheme is conditioned on the CU size using 2 contexts. The proposed method reportedly generates -0.01%, -0.01%, and +0.02% luma BD-rates for the AI, RA, and LB settings, respectively, under the VTM3.0 CTCs while reducing the total number of context variables by 4. The proposed method reportedly generates 0.00% and 0.00% luma BD-rates for the RA and LB settings, respectively, under the VTM3.0 CTCs with the Inter MTS mode being enabled.

# Introduction

The adaptive multiple transform was first proposed for a quad-tree partitioning coding unit (CU) structure using square block transforms [1]. The context selection for signalling the use of adaptive multiple core transform was determined by the quad-tree depth of the current CU. It is also directly related to the CU size, which is dependent on the current quad-tree depth and the size of the associated coding tree unit (CTU) size.

In VVC working draft (WD) 3 as specified in [2], each CTU can be partitioned into one or multiple smaller size CUs by a quad-tree with further nested multi-type tree using binary and ternary split. The CUs of the same quad-tree depth may correspond to different CU sizes. In VVC WD 3, entropy coding the syntax element tu_mts_flag for indicating the use of the multiple primary transform selection is also determined by the quad-tree depth of the current CU. Six context variables are assigned for entropy coding tu_mts_flag in VTM3.0 [3]. However, the multiple primary transform selection is only enabled when both the width and the height of the current CU are less than or equal to 32. Some CU partitioning constraints have been imposed for partitioning the CUs of width or height greater than 64. As a result, under the VTM3.0 common test conditions (CTCs), the context variables associated with the quad-tree depths 0 and 1 are never used in an Intra slice. The context associated with the quad-tree depth 0 in an inter slice is only used for signalling tu_mts_flag in the partial CUs along the picture boundaries allowing to have the binary-tree depth greater than or equal to 4.

# Proposed method

In the proposed method, context modeling for entropy coding syntax element tu_mts_flag is conditioned on the size of the current CU using two context variables. When the size of the current CU is less than 256, the value of the context index increment ctxInc is equal to 1. Otherwise, ctxInc is equal to 0.

# Experimental results

The proposed method was integrated into the VTM3.0 reference software [3]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM3.0 reference software. Table 1 summarizes the experimental results conducted under the CTCs specified in [4]. Table 2 summarizes the experimental results conducted under the CTCs with the Inter MTS mode being enabled. It is shown that the proposed method with context variable reduction does not exhibit significant difference in BD-rate and run time compared with the anchor results. The detailed simulation results can be found in the attached spreadsheets.

Table 1: Summary of the average BD bitrate and run time results under the CTCs

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | 0.02% | 100% | 101% |
| Class A2 | -0.02% | 0.04% | 0.02% | 100% | 101% |
| Class B | -0.01% | -0.04% | 0.01% | 101% | 100% |
| Class C | -0.01% | -0.06% | -0.07% | 100% | 101% |
| Class E | -0.02% | -0.04% | 0.09% | 102% | 101% |
| Overall | -0.01% | -0.04% | 0.01% | 101% | 101% |
| Class D | 0.02% | 0.00% | -0.02% | 100% | 101% |
| Class F (optional) | 0.02% | -0.07% | 0.02% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.06% | 99% | 99% |
| Class A2 | -0.01% | 0.05% | 0.08% | 99% | 98% |
| Class B | -0.01% | 0.06% | -0.11% | 100% | 100% |
| Class C | 0.00% | -0.05% | 0.03% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.00% | 100% | 98% |
| Class D | 0.01% | -0.21% | -0.20% | 100% | 100% |
| Class F (optional) | -0.01% | 0.16% | 0.22% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.24% | 0.06% | 99% | 101% |
| Class C | 0.05% | 0.15% | 0.18% | 100% | 96% |
| Class E | 0.01% | 0.87% | -0.06% | 99% | 97% |
| Overall | 0.02% | 0.17% | 0.07% | 99% | 98% |
| Class D | 0.03% | 0.30% | 0.52% | 100% | 101% |
| Class F (optional) | 0.04% | 0.07% | 0.50% | 100% | 97% |

Table 2: Summary of the average BD bitrate and run time results under the CTCs with the Inter MTS mode being enabled

|  | Random Access Main 10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  | 0.02% | 0.15% | 0.14% | 100% | 100% |
| Class A2 |  | -0.02% | 0.10% | 0.08% | 100% | 102% |
| Class B |  | -0.01% | 0.00% | 0.19% | 100% | 100% |
| Class C |  | 0.01% | -0.05% | 0.15% | 100% | 102% |
| Class E |  |  |  |  |  |  |
| Overall |  | 0.00% | 0.04% | 0.15% | 100% | 101% |
| Class D |  | 0.03% | -0.25% | -0.07% | 100% | 100% |
| Class F (optional) |  | 0.02% | 0.23% | 0.06% | 100% | 100% |
|  |  | Low delay B Main10 |  |  |
|  |  | Over VTM-3.0 |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  | -0.03% | 0.04% | -0.48% | 100% | 102% |
| Class C |  | 0.03% | 0.18% | 0.09% | 100% | 100% |
| Class E |  | 0.03% | 0.79% | -0.68% | 100% | 101% |
| Overall |  | 0.00% | 0.27% | -0.34% | 100% | 101% |
| Class D |  | 0.03% | -0.08% | 0.19% | 100% | 101% |
| Class F (optional) |  | 0.00% | 0.53% | -0.06% | 100% | 101% |

# Conclusions

The proposed method reportedly generates -0.01%, -0.01%, and +0.02% luma BD-rates for the AI, RA, and LB settings, respectively, under the VTM3.0 CTCs and generates 0.00% and 0.00% luma BD-rates for the RA and LB settings, respectively, under the VTM3.0 CTCs with the Inter MTS mode being enabled while reducing the total number of context variables by 4.

# References

- J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang, X. Zhao, “Coding tools investigation for next generation video coding,” ITU-T SG16 Doc. COM16–C806, Feb. 2015.

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 3),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1001, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- The software VTM3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0 .

- F. Bossen, J. Boyce, X. Li, and V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).