    - Transformation process for scaled transform coefficients

      - General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-16 depending on mts_idx[ xTbY ][ yTbY ][ cIdx ].

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, 32 )		(8-810)

nonZeroH = Min( nTbH, 32 )		(8-811)

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- Each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the following one-dimensional transformation process for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list d[ x ][ y ] with y = 0..nonZeroH − 1 and the transform type variable trType set equal to trTypeVer as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

  - If nTbH is equal to 16 or 32 and trTypeVer is equal to 1 or 2, the one-dimensional transform process as specified in clause 8.4.4.3 is invoked.

  - Otherwise, the one-dimensional transform process as specified in clause 8.4.4.2 is invoked.

- The intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(8-812)

- Each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the following one-dimensional transformation process for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1 and the transform type variable trType set equal to trTypeHor as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

  - If nTbW is equal to 16 or 32 and trTypeHor is equal to 1 or 2, the one-dimensional transform process as specified in clause 8.4.4.3 is invoked.

  - Otherwise, the one-dimensional transform process as specified in clause 8.4.4.2 is invoked.

Table 8-16 – Specification of trTypeHor and trTypeVer depending on mts_idx[ x ][ y ][ cIdx ]

| mts_idx[ xTbY ][ yTbY ][ cIdx ] | trTypeHor | trTypeVer |
| --- | --- | --- |
| −1 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 2 | 1 |
| 2 | 1 | 2 |
| 3 | 2 | 2 |

      - Transformation process except for the case that the sample size of transformed samples is 16 or 32 and the transform kernel type is 1 or 2

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of transformed samples,

- a variable nonZeroS specifying the horizontal sample size of non-zero scaled transform coefficients,

- a list of scaled transform coefficients x[ j ] with j = 0..nonZeroS − 1,

- a transform kernel type variable trType.

Output of this process is the list of transformed samples y[ i ] with i = 0..nTbS − 1.

The transformation matrix derivation process as specified in clause 8.4.4.4 in invoked with the transform size nTbS and the transform kernel Type trType as inputs, and the transformation maxtrix transMatrix as output.

Depending on the value of trType, the following applies:, the list of transformed samples y[ i ] with i = 0..nTbS − 1 is derived as follows:

- If trType is equal to 0, the following transform matrix multiplication applies:

y[i]= j=0nonZeroS-1transMatrix[i][j*26-Log2(nTbS) ]*x[j]  with i = 0..nTbS − 1	(8-813)

- Otherwise (trType is equal to 1 or trType is equal to 2), the following transform matrix multiplication applies:

y[i]= j=0nonZeroS-1transMatrix[i][j]*x[j]  with i = 0..nTbS − 1	(8-814)

      - Transformation process for the case that the sample size of transformed samples is 16 or 32 and the transform kernel type is 1 or 2

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of transformed samples,

- a variable nonZeroS specifying the horizontal sample size of non-zero scaled transform coefficients,

- a list of scaled transform coefficients x[ j ] with j = 0..nonZeroS − 1,

- a transform kernel type variable trType.

Output of this process is the list of transformed samples y[ i ] with i = 0..nTbS − 1.

The variable sclTrCoeffPermIdx[ j ] with j = 0..nTbS – 1 and the variable sclTrCoeffPermSgn[ j ] with j = 0..nTbS – 1 are derived as follows:

- If nTbS is equal to 16, the variable sclTrCoeffPermIdx[ j ] with j = 0..nTbS – 1 and the variable sclTrCoeffPermSgn[ j ] with j = 0..nTbS – 1 are specified in Table 8-1.

- If nTbS is equal to 32, the variable sclTrCoeffPermIdx[ j ] with j = 0..nTbS – 1 and the variable sclTrCoeffPermSgn[ j ] with j = 0..nTbS – 1 are specified in Table 8-2.

Table 8-1 ― Specification of sclTrCoeffPermIdx and sclTrCoeffPermSgn in the case that nTbS is equal to 16

| j | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sclTrCoeffPermIdx[ j ] | 5 | 4 | 15 | 6 | 7 | 3 | 14 | 13 |
| sclTrCoeffPermSgn[ j ] | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 |
| j | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| sclTrCoeffPermIdx[ j ] | 8 | 2 | 1 | 12 | 9 | 10 | 0 | 11 |
| sclTrCoeffPermSgn[ j ] | -1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 |

Table 8-2 ― Specification of sclTrCoeffPermIdx and sclTrCoeffPermSgn in the case that nTbS is equal to 32

| j | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sclTrCoeffPermIdx[ j ] | 6 | 19 | 7 | 31 | 5 | 20 | 18 | 17 | 8 | 30 | 4 | 21 | 22 | 16 | 9 | 29 |
| sclTrCoeffPermSgn[ j ] | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 |
| j | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| sclTrCoeffPermIdx[ j ] | 3 | 2 | 23 | 15 | 10 | 28 | 27 | 1 | 24 | 14 | 11 | 12 | 26 | 0 | 25 | 13 |
| sclTrCoeffPermSgn[ j ] | 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 |

The variable u[ j ] with j = 0..nTbS – 1 is derived as follows:

- If trType is equal to 1, the following applies:

  - u[ j ] = ( sclTrCoeffPermSgn[ j ] > 0 ) ? x[ sclTrCoeffPermIdx[ j ] ] : -x[ sclTrCoeffPermIdx[ j ] ] with j = 0..nonZeroS – 1.

  - When nTbS > nonZeroS, u[ j ] with j=nonZeroS..nTbS – 1 is equal to 0.

- Otherwise (trType is equal to 2), the following applies:

  - u[ j ] = ( ( ( 1 – ( j % 2 ) << 1 ) * sclTrCoeffPermSgn[ j ] ) > 0 ) ? x[ sclTrCoeffPermIdx[ j ] ] : -x[ sclTrCoeffPermIdx[ j ] ] with j = 0..nonZeroS – 1.

  - When nTbS > nonZeroS, u[ j ] with j=nonZeroS..nTbS – 1 is equal to 0.

The variable z[ i ] with i = 0..nTbS – 1 is derived as follows:

- If nTbS is equal to 16, the following ordered steps apply:

- The variable v[ j ] with j = 0..15 is derived as follows:

  - The variable v[ 11 ] is derived as follows:

v[ 11 ] = ( -u[ 0 ] * 111 + ( 1 << 6 ) ) >> 7

  - The transformation matrix transMatrix3x3 is derived as follows:

transMatrix3x3[ m ][ n ] =

{

{ 128   128  128 }

{ 128   -64  -64 }

{   0  -111  111 }

}

  - For i = 0..4, the following ordered steps apply:

    - The variable inMatrix3x3[ k ] with k = 0..2 is equal to u[ ( 3 * i ) + k + 1 ].

    - The variable outMatrix3x3[ k ] with k = 0..2 is derived as follows:

outMatrix3x3[k]= j=02transMatrix3x3[k][j]*inMatrix3x3[j]  with k = 0..2

    - The variable v[ j ] with j = i, ( i + 5 ), ( i + 11 ) is derived as follows:

v[ i + ( 5 * k ) + ( ( k == 2 ) ? 1 : 0 ) ] = ( outMatrix3x3[ k ] + ( 1 << 6 ) ) >> 7  with k = 0..2

- The transformation matrices transMatrix5x5 and transMatrix6x6 are derived as follows:

transMatrix5x5[ m ][ n ] =

{

{ -48  -81  -88  -67  -25 }

{ -81  -67   25   88   48 }

{ -88   25   81  -48  -67 }

{ -67   88  -48  -25   81 }

{ -25   48  -67   81  -88 }

},

transMatrix6x6[ m ][ n ] =

{

{  89   89   89   89   89   89  }

{  89   75   37  -13  -15  -86  }

{  89   37  -58  -86  -13   75  }

{  89  -13  -86   37   75  -58  }

{  89  -58  -13   75  -86   37  }

{  89  -86   75  -58   37  -13  }

}

- The variable z[ i ] with i = 0..15 is derived as follows:

  - The variable z[ i ] with i = 0..4 is derived as follows:

z[ i ]= j=04transMatrix5x5[ i ][ j ]*v[ j ]  with i = 0..4

  - The variable outMatrix5x5[ j ] with j = 0..4 and the variable outMatrix6x6[ k ] with k = 0..5 are derived as follows:

outMatrix5x5[ j ]= l=04transMatrix5x5[ j ][ l ]*v[ 5+l ]  with j = 0..4

outMatrix6x6[ k ]= l=05transMatrix6x6[ j ][ l ]*v[ 10+l ]  with k = 0..5

  - The variable z[ i ] with i = 5..15 is derived as follows:

z[ 5 ] = outMatrix6x6[ 0 ],

z[ 6 + j ] = outMatrix6x6[ j  + 1 ] + outMatrix5x5[ j ]    with j = 0..4,

z[11 + j ] = outMatrix6x6[ 5 - j ] - outMatrix5x5[ 4 - j ]    with j = 0..4

- Otherwise (nTbS is equal to 32), the following ordered steps apply:

  - The variable v[ j ] with j = 0..31 is derived as follows:

    - The variable v[ 12 ] and v[ 25 ] are derived as follows:

v[ 12 ] = ( -u[ 0 ] * 122 – u[ 1 ] * 75 + ( 1 << 6 ) ) >> 7

v[ 25 ] = ( -u[ 0 ] * 75 + u[ 1 ] * 122 + (1 << 6 ) ) >> 7

    - The transformation matrix transMatrix5x5 is derived as follows:

transMatrix5x5[ m ][ n ] =

{

{ 128   128   128   128   128 }

{ 128    40  -104  -104    40 }

{   0  -122   -75    75   122 }

{ 128  -104    40    40  -104 }

{   0   -75   122  -122    75 }

}

    - For i = 0..4, the following ordered steps apply:

      - The variable inMatrix5x5[ k ] with k = 0..4 is equal to u[ ( 5 * i ) + k ].

      - The variable outMatrix3x3[ k ] with k = 0..4 is derived as follows:

outMatrix5x5[k]= j=04transMatrix5x5[k][j]*inMatrix5x5[j]  with k = 0..4

      - The variable v[ j ] with j = i, ( i + 6 ), ( i + 13 ), ( i + 19 ), ( i + 26 ) is derived as follows:

v[ i + ( 6 * k ) + ( ( k > 1 ) ? 1 : 0 ) + ( ( k == 4 ) ? 1 : 0 ) ] = ( outMatrix5x5[ k ] + ( 1 << 6 ) ) >> 7   with k = 0..4

  - The transformation matrices transMatrix6x6 and transMatrix7x7 are derived as follows:

transMatrix6x6[ m ][ n ] =

{

{  -42  -74  -89  -84  -60  -21  }

{  -74  -84  -21   60   89   42  }

{  -89  -21   84   42  -74  -60  }

{  -84   60   42  -89   21   74  }

{  -60   89  -74   21   42  -84  }

{  -21   42  -60   74  -84   89  }

},

transMatrix7x7[ m ][ n ] =

{

{  90   90   90   90   90   90   90  }

{  90   80   51   11  -32  -67  -87  }

{  90   51  -32  -87  -67   11   80  }

{  90   11  -87  -32   80   51  -67  }

{  90  -32  -67   80   11  -87   51  }

{  90  -67   11   51  -87   80  -32  }

{  90  -87   80  -67   51  -32   11  }

}

  - The variable z[ i ] with i = 0..31 is derived as follows:

    - The variable z[ i ] with i = 0..5 is derived as follows:

z[ i ]= j=04transMatrix6x6[ i ][ j ]*v[ j ]  with i = 0..5

    - For l = 0,1, the following ordered steps apply:

      - The variable outMatrix6x6[ j ] with j = 0..5 and the variable outMatrix7x7[ k ] with k = 0..6 are derived as follows:

outMatrix6x6[ j ]= i=05transMatrix6x6[ j ][ i ]*v[ 6+( 13*l )+i ]  with j = 0..5

outMatrix7x7[ k ]= i=06transMatrix7x7[ j ][ i ]*v[ 12+( 13*l )+i ]  with k = 0..6

      - The variable z[ i ] with i = ( 6 + ( 13 * l ) )..( 18 + ( 13 * l ) ) is derived as follows:

z[ 6 + ( 13 * l ) ] = outMatrix7x7[ 0 ],

z[ 7 + ( 13 * l ) + j ] = outMatrix6x6[ j ] + outMatrix7x7[ j + 1 ]   with j = 0..5,

z[ 13 + ( 13 * l  )+ j ] = -outMatrix6x6[ 5 - j ] + outMatrix7x7[ 6 – j ]   with j = 0..5

The variable trafoSamplePermIdx[ j ] with j = 0..nTbS – 1 and the variable trafoSamplePermSgn[ j ] with j = 0..nTbS – 1 are derived as follows:

- If nTbS is equal to 16, the variable trafoSamplePermIdx[ j ] with j = 0..nTbS – 1 and the variable trafoSamplePermSgn[ j ] with j = 0..nTbS – 1 are specified in Table 8-3.

- If nTbS is equal to 32, the variable trafoSamplePermIdx[ j ] with j = 0..nTbS – 1 and the variable trafoSamplePermSgn[ j ] with j = 0..nTbS – 1 are specified in Table 8-4.

Table 8-3 ― Specification of trafoSamplePermIdx and trafoSamplePermSgn in the case that nTbS is equal to 16

| j | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| trafoSamplePermIdx[ j ] | 12 | 13 | 0 | 11 | 14 | 1 | 10 | 15 |
| trafoSamplePermSgn[ j ] | 1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 |
| j | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| trafoSamplePermIdx[ j ] | 2 | 9 | 5 | 3 | 8 | 6 | 4 | 7 |
| trafoSamplePermSgn[ j ] | -1 | -1 | -1 | 1 | 1 | 1 | -1 | -1 |

Table 8-4 ― Specification of trafoSamplePermIdx and trafoSamplePermSgn in the case that nTbS is equal to 32

| j | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| trafoSamplePermIdx[ j ] | 27 | 16 | 17 | 26 | 0 | 28 | 15 | 18 | 25 | 1 | 29 | 14 | 6 | 24 | 2 | 30 |
| trafoSamplePermSgn[ j ] | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 |
| J | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| trafoSamplePermIdx[ j ] | 13 | 7 | 23 | 3 | 31 | 12 | 8 | 22 | 4 | 19 | 11 | 9 | 21 | 5 | 20 | 10 |
| trafoSamplePermSgn[ j ] | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 |

The list of transformed samples y[ i ] with i = 0..nTbS − 1 is derived as follows:

- If trType is equal to 1, the following applies:

u[ j ] = ( sclTrCoeffPermSgn[ j ] > 0 ) ? x[ sclTrCoeffPermIdx[ j ] ] : -x[ sclTrCoeffPermIdx[ j ] ] with j = 0..nTbS – 1.

- Otherwise (trType is equal to 2), the following applies:

u[ j ] = ( ( sclTrCoeffPermSgn[ nTbS - 1 - j ] ) > 0 ) ? x[ sclTrCoeffPermIdx[ nTbS - 1 - j ] ] : -x[ sclTrCoeffPermIdx[ nTbS - 1 - j ] ] with j = 0..nTbS – 1.

      - Transformation matrix derivation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- the transformation kernel type trType.

Output of this process is the transformation matrix transMatrix.

The transformation matrix transMatrix is derived based on trType and nTbs as follows:

- If trType is equal to 0, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..63	(8-815)

transMatrixCol0to15 =	(8-816)

{

{  64   64   64   64   64   64   64   64   64   64   64   64   64   64   64   64 }

{  91   90   90   90   88   87   86   84   83   81   79   77   73   71   69   65 }

{  90   90   88   85   82   78   73   67   61   54   46   38   31   22   13    4 }

{  90   88   84   79   71   62   52   41   28   15    2  −11  −24  −37  −48  −59 }

{  90   87   80   70   57   43   25    9   −9  −25  −43  −57  −70  −80  −87  −90 }

{  90   84   73   59   41   20   −2  −24  −44  −62  −77  −86  −90  −90  −83  −71 }

{  90   82   67   46   22   −4  −31  −54  −73  −85  −90  −88  −78  −61  −38  −13 }

{  90   79   59   33    2  −28  −56  −77  −88  −90  −81  −62  −37   −7   24   52 }

{  89   75   50   18  −18  −50  −75  −89  −89  −75  −50  −18   18   50   75   89 }

{  88   71   41    2  −37  −69  −87  −90  −73  −44   −7   33   65   86   90   77 }

{  88   67   31  −13  −54  −82  −90  −78  −46   −4   38   73   90   85   61   22 }

{  87   62   20  −28  −69  −90  −84  −56  −11   37   73   90   81   48    2  −44 }

{  87   57    9  −43  −80  −90  −70  −25   25   70   90   80   43   −9  −57  −87 }

{  86   52   −2  −56  −87  −84  −48    7   59   88   83   44  −11  −62  −90  −81 }

{  85   46  −13  −67  −90  −73  −22   38   82   88   54   −4  −61  −90  −78  −31 }

{  84   41  −24  −77  −90  −56    7   65   91   69   11  −52  −88  −79  −28   37 }

{  83   36  −36  −83  −83  −36   36   83   83   36  −36  −83  −83  −36   36   83 }

{  83   28  −44  −88  −73  −11   59   91   62   −7  −71  −90  −48   24   81   84 }

{  82   22  −54  −90  −61   13   78   85   31  −46  −90  −67    4   73   88   38 }

{  81   15  −62  −90  −44   37   88   69   −7  −77  −84  −24   56   91   52  −28 }

{  80    9  −70  −87  −25   57   90   43  −43  −90  −57   25   87   70   −9  −80 }

{  79    2  −77  −81   −7   73   83   11  −71  −84  −15   69   86   20  −65  −87 }

{  78   −4  −82  −73   13   85   67  −22  −88  −61   31   90   54  −38  −90  −46 }

{  77  −11  −86  −62   33   90   44  −52  −90  −24   69   83    2  −81  −71   20 }

{  75  −18  −89  −50   50   89   18  −75  −75   18   89   50  −50  −89  −18   75 }

{  73  −24  −90  −37   65   81  −11  −88  −48   56   86    2  −84  −59   44   90 }

{  73  −31  −90  −22   78   67  −38  −90  −13   82   61  −46  −88   −4   85   54 }

{  71  −37  −90   −7   86   48  −62  −79   24   91   20  −81  −59   52   84  −11 }

{  70  −43  −87    9   90   25  −80  −57   57   80  −25  −90   −9   87   43  −70 }

{  69  −48  −83   24   90    2  −90  −28   81   52  −65  −71   44   84  −20  −90 }

{  67  −54  −78   38   85  −22  −90    4   90   13  −88  −31   82   46  −73  −61 }

{  65  −59  −71   52   77  −44  −81   37   84  −28  −87   20   90  −11  −90    2 }

{  64  −64  −64   64   64  −64  −64   64   64  −64  −64   64   64  −64  −64   64 }

{  62  −69  −56   73   48  −79  −41   83   33  −86  −24   88   15  −90   −7   91 }

{  61  −73  −46   82   31  −88  −13   90   −4  −90   22   85  −38  −78   54   67 }

{  59  −77  −37   87   11  −91   15   86  −41  −73   62   56  −79  −33   88    7 }

{  57  −80  −25   90   −9  −87   43   70  −70  −43   87    9  −90   25   80  −57 }

{  56  −83  −15   90  −28  −77   65   44  −87   −2   88  −41  −69   73   33  −90 }

{  54  −85   −4   88  −46  −61   82   13  −90   38   67  −78  −22   90  −31  −73 }

{  52  −87    7   83  −62  −41   90  −20  −77   71   28  −91   33   69  −79  −15 }

{  50  −89   18   75  −75  −18   89  −50  −50   89  −18  −75   75   18  −89   50 }

{  48  −90   28   65  −84    7   79  −73  −15   87  −59  −37   91  −41  −56   88 }

{  46  −90   38   54  −90   31   61  −88   22   67  −85   13   73  −82    4   78 }

{  44  −91   48   41  −90   52   37  −90   56   33  −90   59   28  −88   62   24 }

{  43  −90   57   25  −87   70    9  −80   80   −9  −70   87  −25  −57   90  −43 }

{  41  −90   65   11  −79   83  −20  −59   90  −48  −33   87  −71   −2   73  −86 }

{  38  −88   73   −4  −67   90  −46  −31   85  −78   13   61  −90   54   22  −82 }

{  37  −86   79  −20  −52   90  −69    2   65  −90   56   15  −77   87  −41  −33 }

{  36  −83   83  −36  −36   83  −83   36   36  −83   83  −36  −36   83  −83   36 }

{  33  −81   87  −48  −15   71  −90   62   −2  −59   90  −73   20   44  −86   83 }

{  31  −78   90  −61    4   54  −88   82  −38  −22   73  −90   67  −13  −46   85 }

{  28  −73   91  −71   24   33  −77   90  −69   20   37  −79   90  −65   15   41 }

{  25  −70   90  −80   43    9  −57   87  −87   57   −9  −43   80  −90   70  −25 }

{  24  −65   88  −86   59  −15  −33   71  −90   83  −52    7   41  −77   91  −79 }

{  22  −61   85  −90   73  −38   −4   46  −78   90  −82   54  −13  −31   67  −88 }

{  20  −56   81  −91   83  −59   24   15  −52   79  −90   84  −62   28   11  −48 }

{  18  −50   75  −89   89  −75   50  −18  −18   50  −75   89  −89   75  −50   18 }

{  15  −44   69  −84   91  −86   71  −48   20   11  −41   65  −83   90  −87   73 }

{  13  −38   61  −78   88  −90   85  −73   54  −31    4   22  −46   67  −82   90 }

{  11  −33   52  −69   81  −88   91  −87   79  −65   48  −28    7   15  −37   56 }

{   9  −25   43  −57   70  −80   87  −90   90  −87   80  −70   57  −43   25   −9 }

{   7  −20   33  −44   56  −65   73  −81   86  −90   91  −90   87  −83   77  −69 }

{   4  −13   22  −31   38  −46   54  −61   67  −73   78  −82   85  −88   90  −90 }

{   2   −7   11  −15   20  −24   28  −33   37  −41   44  −48   52  −56   59  −62 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..63	(8-817)

transMatrixCol16to31 =	(8-818)

{

{  64   64   64   64   64   64   64   64   64   64   64   64   64   64   64   64 }

{  62   59   56   52   48   44   41   37   33   28   24   20   15   11    7    2 }

{  −4  −13  −22  −31  −38  −46  −54  −61  −67  −73  −78  −82  −85  −88  −90  −90 }

{ −69  −77  −83  −87  −90  −91  −90  −86  −81  −73  −65  −56  −44  −33  −20   −7 }

{ −90  −87  −80  −70  −57  −43  −25   −9    9   25   43   57   70   80   87   90 }

{ −56  −37  −15    7   28   48   65   79   87   91   88   81   69   52   33   11 }

{  13   38   61   78   88   90   85   73   54   31    4  −22  −46  −67  −82  −90 }

{  73   87   90   83   65   41   11  −20  −48  −71  −86  −91  −84  −69  −44  −15 }

{  89   75   50   18  −18  −50  −75  −89  −89  −75  −50  −18   18   50   75   89 }

{  48   11  −28  −62  −84  −90  −79  −52  −15   24   59   83   91   81   56   20 }

{ −22  −61  −85  −90  −73  −38    4   46   78   90   82   54   13  −31  −67  −88 }

{ −79  −91  −77  −41    7   52   83   90   71   33  −15  −59  −86  −88  −65  −24 }

{ −87  −57   −9   43   80   90   70   25  −25  −70  −90  −80  −43    9   57   87 }

{ −41   15   65   90   79   37  −20  −69  −90  −77  −33   24   71   91   73   28 }

{  31   78   90   61    4  −54  −88  −82  −38   22   73   90   67   13  −46  −85 }

{  83   86   44  −20  −73  −90  −59    2   62   90   71   15  −48  −87  −81  −33 }

{  83   36  −36  −83  −83  −36   36   83   83   36  −36  −83  −83  −36   36   83 }

{  33  −41  −87  −77  −15   56   90   65   −2  −69  −90  −52   20   79   86   37 }

{ −38  −88  −73   −4   67   90   46  −31  −85  −78  −13   61   90   54  −22  −82 }

{ −86  −73   −2   71   87   33  −48  −90  −59   20   83   79   11  −65  −90  −41 }

{ −80   −9   70   87   25  −57  −90  −43   43   90   57  −25  −87  −70    9   80 }

{ −24   62   88   28  −59  −90  −33   56   90   37  −52  −90  −41   48   91   44 }

{  46   90   38  −54  −90  −31   61   88   22  −67  −85  −13   73   82    4  −78 }

{  88   56  −41  −91  −37   59   87   15  −73  −79    7   84   65  −28  −90  −48 }

{  75  −18  −89  −50   50   89   18  −75  −75   18   89   50  −50  −89  −18   75 }

{  15  −79  −69   33   91   28  −71  −77   20   90   41  −62  −83    7   87   52 }

{ −54  −85    4   88   46  −61  −82   13   90   38  −67  −78   22   90   31  −73 }

{ −90  −33   73   69  −41  −88   −2   87   44  −65  −77   28   90   15  −83  −56 }

{ −70   43   87   −9  −90  −25   80   57  −57  −80   25   90    9  −87  −43   70 }

{  −7   88   33  −79  −56   62   73  −41  −86   15   91   11  −87  −37   77   59 }

{  61   73  −46  −82   31   88  −13  −90   −4   90   22  −85  −38   78   54  −67 }

{  91    7  −90  −15   88   24  −86  −33   83   41  −79  −48   73   56  −69  −62 }

{  64  −64  −64   64   64  −64  −64   64   64  −64  −64   64   64  −64  −64   64 }

{  −2  −90   11   90  −20  −87   28   84  −37  −81   44   77  −52  −71   59   65 }

{ −67  −54   78   38  −85  −22   90    4  −90   13   88  −31  −82   46   73  −61 }

{ −90   20   84  −44  −71   65   52  −81  −28   90    2  −90   24   83  −48  −69 }

{ −57   80   25  −90    9   87  −43  −70   70   43  −87   −9   90  −25  −80   57 }

{  11   84  −52  −59   81   20  −91   24   79  −62  −48   86    7  −90   37   71 }

{  73   31  −90   22   78  −67  −38   90  −13  −82   61   46  −88    4   85  −54 }

{  90  −44  −59   84    2  −86   56   48  −88   11   81  −65  −37   90  −24  −73 }

{  50  −89   18   75  −75  −18   89  −50  −50   89  −18  −75   75   18  −89   50 }

{ −20  −71   81    2  −83   69   24  −90   52   44  −90   33   62  −86   11   77 }

{ −78   −4   82  −73  −13   85  −67  −22   88  −61  −31   90  −54  −38   90  −46 }

{ −87   65   20  −86   69   15  −84   71   11  −83   73    7  −81   77    2  −79 }

{ −43   90  −57  −25   87  −70   −9   80  −80    9   70  −87   25   57  −90   43 }

{  28   52  −91   56   24  −84   77   −7  −69   88  −37  −44   90  −62  −15   81 }

{  82  −22  −54   90  −61  −13   78  −85   31   46  −90   67    4  −73   88  −38 }

{  84  −81   24   48  −90   71   −7  −62   91  −59  −11   73  −88   44   28  −83 }

{  36  −83   83  −36  −36   83  −83   36   36  −83   83  −36  −36   83  −83   36 }

{ −37  −28   79  −88   52   11  −69   91  −65    7   56  −90   77  −24  −41   84 }

{ −85   46   13  −67   90  −73   22   38  −82   88  −54   −4   61  −90   78  −31 }

{ −81   90  −62   11   44  −83   88  −59    7   48  −84   87  −56    2   52  −86 }

{ −25   70  −90   80  −43   −9   57  −87   87  −57    9   43  −80   90  −70   25 }

{  44    2  −48   81  −90   73  −37  −11   56  −84   90  −69   28   20  −62   87 }

{  88  −67   31   13  −54   82  −90   78  −46    4   38  −73   90  −85   61  −22 }

{  77  −90   86  −65   33    7  −44   73  −90   87  −69   37    2  −41   71  −88 }

{  18  −50   75  −89   89  −75   50  −18  −18   50  −75   89  −89   75  −50   18 }

{ −52   24    7  −37   62  −81   90  −88   77  −56   28    2  −33   59  −79   90 }

{ −90   82  −67   46  −22   −4   31  −54   73  −85   90  −88   78  −61   38  −13 }

{ −71   83  −90   90  −86   77  −62   44  −24    2   20  −41   59  −73   84  −90 }

{  −9   25  −43   57  −70   80  −87   90  −90   87  −80   70  −57   43  −25    9 }

{  59  −48   37  −24   11    2  −15   28  −41   52  −62   71  −79   84  −88   90 }

{  90  −90   88  −85   82  −78   73  −67   61  −54   46  −38   31  −22   13   −4 }

{  65  −69   71  −73   77  −79   81  −83   84  −86   87  −88   90  −90   90  −91 }

},

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 ) * transMatrixCol16to31[47 −m ][ n ]	(8-819)
		 with m =32..47, n = 0..63

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 ) * transMatrixCol0to15[63 −m ][ n ]	(8-820)
		with m =48..63, n = 0..63

- Otherwise, if trType is equal to 1 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-821)

{

{  29   55   74   84 }

{  74   74    0  −74 }

{  84  −29  −74   55 }

{  55  −84   74  −29 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-822)

{

{  17   32   46   60   71   78   85   86 }

{  46   78   86   71   32  −17  −60  −85 }

{  71   85   32  −46  −86  −60   17   78 }

{  85   46  −60  −78   17   86   32  −71 }

{  86  −17  −85   32   78  −46  −71   60 }

{  78  −71  −17   85  −60  −32   86  −46 }

{  60  −86   71  −17  −46   85  −78   32 }

{  32  −60   78  −86   85  −71   46  −17 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-823)

{

{   9   17   25   33   41   49   56   62   66   72   77   81   83   87   89   90 }

{  25   49   66   81   89   89   81   66   49   25    0  −25  −49  −66  −81  −89 }

{  41   72   89   83   62   25  −17  −56  −81  −90  −77  −49   −9   33   66   87 }

{  56   87   81   41  −17  −66  −90  −72  −25   33   77   89   62    9  −49  −83 }

{  66   89   49  −25  −81  −81  −25   49   89   66    0  −66  −89  −49   25   81 }

{  77   77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77 }

{  83   56  −49  −87   −9   81   62  −41  −89  −17   77   66  −33  −90  −25   72 }

{  89   25  −81  −49   66   66  −49  −81   25   89    0  −89  −25   81   49  −66 }

{  90   −9  −89   17   87  −25  −83   33   81  −41  −77   49   72  −56  −66   62 }

{  87  −41  −66   72   33  −89    9   83  −49  −62   77   25  −90   17   81  −56 }

{  81  −66  −25   89  −49  −49   89  −25  −66   81    0  −81   66   25  −89   49 }

{  72  −83   25   56  −90   49   33  −87   66    9  −77   81  −17  −62   89  −41 }

{  62  −90   66   −9  −56   89  −72   17   49  −87   77  −25  −41   83  −81   33 }

{  49  −81   89  −66   25   25  −66   89  −81   49    0  −49   81  −89   66  −25 }

{  33  −62   81  −90   83  −66   41   −9  −25   56  −77   89  −87   72  −49   17 }

{  17  −33   49  −62   72  −81   87  −90   89  −83   77  −66   56  −41   25   −9 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..31	(8-824)

transMatrixCol0to15 =	(8-825)

{

{   4    9   13   17   21   26   30   34   38   42   45   50   53   56   60   63 }

{  13   26   38   50   60   68   77   82   86   89   90   88   85   80   74   66 }

{  21   42   60   74   84   89   89   84   74   60   42   21    0  −21  −42  −60 }

{  30   56   77   88   89   80   63   38    9  −21  −50  −72  −85  −90  −84  −68 }

{  38   68   86   88   74   45    9  −30  −63  −84  −90  −78  −53  −17   21   56 }

{  45   78   90   77   42   −4  −50  −80  −90  −74  −38    9   53   82   89   72 }

{  53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53 }

{  60   89   74   21  −42  −84  −84  −42   21   74   89   60    0  −60  −89  −74 }

{  66   90   56  −13  −74  −88  −45   26   80   84   34  −38  −85  −78  −21   50 }

{  72   86   34  −45  −89  −63   13   78   82   21  −56  −90  −53   26   84   77 }

{  77   80    9  −72  −84  −17   66   86   26  −60  −88  −34   53   90   42  −45 }

{  80   72  −17  −86  −60   34   90   45  −50  −89  −30   63   85   13  −74  −78 }

{  84   60  −42  −89  −21   74   74  −21  −89  −42   60   84    0  −84  −60   42 }

{  86   45  −63  −78   21   90   26  −77  −66   42   88    4  −85  −50   60   80 }

{  88   30  −78  −56   60   77  −34  −88    4   89   26  −80  −53   63   74  −38 }

{  90   13  −88  −26   84   38  −78  −50   72   60  −63  −68   53   77  −42  −82 }

{  90   −4  −90    9   89  −13  −88   17   88  −21  −86   26   85  −30  −84   34 }

{  89  −21  −84   42   74  −60  −60   74   42  −84  −21   89    0  −89   21   84 }

{  88  −38  −72   68   42  −86   −4   88  −34  −74   66   45  −85   −9   89  −30 }

{  85  −53  −53   85    0  −85   53   53  −85    0   85  −53  −53   85    0  −85 }

{  82  −66  −30   90  −42  −56   86  −13  −77   74   17  −88   53   45  −89   26 }

{  78  −77   −4   80  −74   −9   82  −72  −13   84  −68  −17   85  −66  −21   86 }

{  74  −84   21   60  −89   42   42  −89   60   21  −84   74    0  −74   84  −21 }

{  68  −88   45   30  −84   78  −17  −56   90  −60  −13   77  −85   34   42  −88 }

{  63  −90   66   −4  −60   90  −68    9   56  −89   72  −13  −53   88  −74   17 }

{  56  −88   80  −38  −21   72  −90   68  −17  −42   82  −86   53    4  −60   88 }

{  50  −82   88  −66   21   30  −72   90  −78   42    9  −56   85  −86   60  −13 }

{  42  −74   89  −84   60  −21  −21   60  −84   89  −74   42    0  −42   74  −89 }

{  34  −63   82  −90   84  −66   38   −4  −30   60  −80   90  −85   68  −42    9 }

{  26  −50   68  −82   89  −88   80  −66   45  −21   −4   30  −53   72  −84   90 }

{  17  −34   50  −63   74  −82   88  −90   88  −84   77  −66   53  −38   21   −4 }

{   9  −17   26  −34   42  −50   56  −63   68  −74   78  −82   85  −88   89  −90 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..31	(8-826)

transMatrixCol16to31 =	(8-827)

{

{  66   68   72   74   77   78   80   82   84   85   86   88   88   89   90   90 }

{  56   45   34   21    9   −4  −17  −30  −42  −53  −63  −72  −78  −84  −88  −90 }

{ −74  −84  −89  −89  −84  −74  −60  −42  −21    0   21   42   60   74   84   89 }

{ −45  −17   13   42   66   82   90   86   74   53   26   −4  −34  −60  −78  −88 }

{  80   90   82   60   26  −13  −50  −77  −89  −85  −66  −34    4   42   72   88 }

{  34  −13  −56  −84  −88  −68  −30   17   60   85   88   66   26  −21  −63  −86 }

{ −85  −85  −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85 }

{ −21   42   84   84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84 }

{  88   72    9  −60  −90  −63    4   68   89   53  −17  −77  −86  −42   30   82 }

{   9  −66  −88  −42   38   88   68   −4  −74  −85  −30   50   90   60  −17  −80 }

{ −90  −50   38   89   56  −30  −88  −63   21   85   68  −13  −82  −74    4   78 }

{   4   82   68  −21  −88  −56   38   90   42  −53  −88  −26   66   84    9  −77 }

{  89   21  −74  −74   21   89   42  −60  −84    0   84   60  −42  −89  −21   74 }

{ −17  −90  −30   74   68  −38  −88   −9   84   53  −56  −82   13   89   34  −72 }

{ −86    9   90   21  −82  −50   66   72  −42  −85   13   90   17  −84  −45   68 }

{  30   86  −17  −89    4   90    9  −88  −21   85   34  −80  −45   74   56  −66 }

{  82  −38  −80   42   78  −45  −77   50   74  −53  −72   56   68  −60  −66   63 }

{ −42  −74   60   60  −74  −42   84   21  −89    0   89  −21  −84   42   74  −60 }

{ −77   63   50  −84  −13   90  −26  −78   60   53  −82  −17   90  −21  −80   56 }

{  53   53  −85    0   85  −53  −53   85    0  −85   53   53  −85    0   85  −53 }

{  68  −80   −4   84  −63  −34   90  −38  −60   85   −9  −78   72   21  −88   50 }

{ −63  −26   88  −60  −30   88  −56  −34   89  −53  −38   90  −50  −42   90  −45 }

{ −60   89  −42  −42   89  −60  −21   84  −74    0   74  −84   21   60  −89   42 }

{  72   −4  −66   89  −50  −26   82  −80   21   53  −90   63    9  −74   86  −38 }

{  50  −88   77  −21  −45   86  −78   26   42  −85   80  −30  −38   84  −82   34 }

{ −78   34   26  −74   90  −66   13   45  −84   85  −50   −9   63  −89   77  −30 }

{ −38   77  −90   74  −34  −17   63  −88   84  −53    4   45  −80   89  −68   26 }

{  84  −60   21   21  −60   84  −89   74  −42    0   42  −74   89  −84   60  −21 }

{  26  −56   78  −89   86  −72   45  −13  −21   53  −77   88  −88   74  −50   17 }

{ −88   78  −63   42  −17   −9   34  −56   74  −85   90  −86   77  −60   38  −13 }

{ −13   30  −45   60  −72   80  −86   90  −89   85  −78   68  −56   42  −26    9 }

{  90  −88   86  −84   80  −77   72  −66   60  −53   45  −38   30  −21   13   −4 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-828)

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-829)

{

{  86   85   78   71   60   46   32   17 }

{  85   60   17  −32  −71  −86  −78  −46 }

{  78   17  −60  −86  −46   32   85   71 }

{  71  −32  −86  −17   78   60  −46  −85 }

{  60  −71  −46   78   32  −85  −17   86 }

{  46  −86   32   60  −85   17   71  −78 }

{  32  −78   85  −46  −17   71  −86   60 }

{  17  −46   71  −85   86  −78   60  −32 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-830)

{

{  90   89   87   83   81   77   72   66   62   56   49   41   33   25   17    9 }

{  89   81   66   49   25    0  −25  −49  −66  −81  −89  −89  −81  −66  −49  −25 }

{  87   66   33   −9  −49  −77  −90  −81  −56  −17   25   62   83   89   72   41 }

{  83   49   −9  −62  −89  −77  −33   25   72   90   66   17  −41  −81  −87  −56 }

{  81   25  −49  −89  −66    0   66   89   49  −25  −81  −81  −25   49   89   66 }

{  77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77  −77 }

{  72  −25  −90  −33   66   77  −17  −89  −41   62   81   −9  −87  −49   56   83 }

{  66  −49  −81   25   89    0  −89  −25   81   49  −66  −66   49   81  −25  −89 }

{  62  −66  −56   72   49  −77  −41   81   33  −83  −25   87   17  −89   −9   90 }

{  56  −81  −17   90  −25  −77   62   49  −83   −9   89  −33  −72   66   41  −87 }

{  49  −89   25   66  −81    0   81  −66  −25   89  −49  −49   89  −25  −66   81 }

{  41  −89   62   17  −81   77   −9  −66   87  −33  −49   90  −56  −25   83  −72 }

{  33  −81   83  −41  −25   77  −87   49   17  −72   89  −56   −9   66  −90   62 }

{  25  −66   89  −81   49    0  −49   81  −89   66  −25  −25   66  −89   81  −49 }

{  17  −49   72  −87   89  −77   56  −25   −9   41  −66   83  −90   81  −62   33 }

{   9  −25   41  −56   66  −77   83  −89   90  −87   81  −72   62  −49   33  −17 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..31	(8-831)

transMatrixCol0to15 =	(8-832)

{

{  90   90   89   88   88   86   85   84   82   80   78   77   74   72   68   66 }

{  90   88   84   78   72   63   53   42   30   17    4   −9  −21  −34  −45  −56 }

{  89   84   74   60   42   21    0  −21  −42  −60  −74  −84  −89  −89  −84  −74 }

{  88   78   60   34    4  −26  −53  −74  −86  −90  −82  −66  −42  −13   17   45 }

{  88   72   42    4  −34  −66  −85  −89  −77  −50  −13   26   60   82   90   80 }

{  86   63   21  −26  −66  −88  −85  −60  −17   30   68   88   84   56   13  −34 }

{  85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53  −85  −85 }

{  84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84  −84  −42   21 }

{  82   30  −42  −86  −77  −17   53   89   68    4  −63  −90  −60    9   72   88 }

{  80   17  −60  −90  −50   30   85   74    4  −68  −88  −38   42   88   66   −9 }

{  78    4  −74  −82  −13   68   85   21  −63  −88  −30   56   89   38  −50  −90 }

{  77   −9  −84  −66   26   88   53  −42  −90  −38   56   88   21  −68  −82   −4 }

{  74  −21  −89  −42   60   84    0  −84  −60   42   89   21  −74  −74   21   89 }

{  72  −34  −89  −13   82   56  −53  −84    9   88   38  −68  −74   30   90   17 }

{  68  −45  −84   17   90   13  −85  −42   72   66  −50  −82   21   90    9  −86 }

{  66  −56  −74   45   80  −34  −85   21   88   −9  −90   −4   89   17  −86  −30 }

{  63  −66  −60   68   56  −72  −53   74   50  −77  −45   78   42  −80  −38   82 }

{  60  −74  −42   84   21  −89    0   89  −21  −84   42   74  −60  −60   74   42 }

{  56  −80  −21   90  −17  −82   53   60  −78  −26   90  −13  −84   50   63  −77 }

{  53  −85    0   85  −53  −53   85    0  −85   53   53  −85    0   85  −53  −53 }

{  50  −88   21   72  −78   −9   85  −60  −38   90  −34  −63   84   −4  −80   68 }

{  45  −90   42   50  −90   38   53  −89   34   56  −88   30   60  −88   26   63 }

{  42  −89   60   21  −84   74    0  −74   84  −21  −60   89  −42  −42   89  −60 }

{  38  −86   74   −9  −63   90  −53  −21   80  −82   26   50  −89   66    4  −72 }

{  34  −82   84  −38  −30   80  −85   42   26  −78   86  −45  −21   77  −88   50 }

{  30  −77   89  −63    9   50  −85   84  −45  −13   66  −90   74  −26  −34   78 }

{  26  −68   89  −80   45    4  −53   84  −88   63  −17  −34   74  −90   77  −38 }

{  21  −60   84  −89   74  −42    0   42  −74   89  −84   60  −21  −21   60  −84 }

{  17  −50   74  −88   88  −77   53  −21  −13   45  −72   86  −89   78  −56   26 }

{  13  −38   60  −77   86  −90   85  −74   56  −34    9   17  −42   63  −78   88 }

{   9  −26   42  −56   68  −78   85  −89   90  −86   80  −72   60  −45   30  −13 }

{   4  −13   21  −30   38  −45   53  −60   66  −72   77  −80   84  −86   88  −90 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..31	(8-833)

transMatrixCol16to31 =	(8-834)

{

{  63   60   56   53   50   45   42   38   34   30   26   21   17   13    9    4 }

{ −66  −74  −80  −85  −88  −90  −89  −86  −82  −77  −68  −60  −50  −38  −26  −13 }

{ −60  −42  −21    0   21   42   60   74   84   89   89   84   74   60   42   21 }

{  68   84   90   85   72   50   21   −9  −38  −63  −80  −89  −88  −77  −56  −30 }

{  56   21  −17  −53  −78  −90  −84  −63  −30    9   45   74   88   86   68   38 }

{ −72  −89  −82  −53   −9   38   74   90   80   50    4  −42  −77  −90  −78  −45 }

{ −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53 }

{  74   89   60    0  −60  −89  −74  −21   42   84   84   42  −21  −74  −89  −60 }

{  50  −21  −78  −85  −38   34   84   80   26  −45  −88  −74  −13   56   90   66 }

{ −77  −84  −26   53   90   56  −21  −82  −78  −13   63   89   45  −34  −86  −72 }

{ −45   42   90   53  −34  −88  −60   26   86   66  −17  −84  −72    9   80   77 }

{  78   74  −13  −85  −63   30   89   50  −45  −90  −34   60   86   17  −72  −80 }

{  42  −60  −84    0   84   60  −42  −89  −21   74   74  −21  −89  −42   60   84 }

{ −80  −60   50   85   −4  −88  −42   66   77  −26  −90  −21   78   63  −45  −86 }

{ −38   74   63  −53  −80   26   89    4  −88  −34   77   60  −56  −78   30   88 }

{  82   42  −77  −53   68   63  −60  −72   50   78  −38  −84   26   88  −13  −90 }

{  34  −84  −30   85   26  −86  −21   88   17  −88  −13   89    9  −90   −4   90 }

{ −84  −21   89    0  −89   21   84  −42  −74   60   60  −74  −42   84   21  −89 }

{ −30   89   −9  −85   45   66  −74  −34   88   −4  −86   42   68  −72  −38   88 }

{  85    0  −85   53   53  −85    0   85  −53  −53   85    0  −85   53   53  −85 }

{  26  −89   45   53  −88   17   74  −77  −13   86  −56  −42   90  −30  −66   82 }

{ −86   21   66  −85   17   68  −84   13   72  −82    9   74  −80    4   77  −78 }

{ −21   84  −74    0   74  −84   21   60  −89   42   42  −89   60   21  −84   74 }

{  88  −42  −34   85  −77   13   60  −90   56   17  −78   84  −30  −45   88  −68 }

{  17  −74   88  −53  −13   72  −89   56    9  −68   90  −60   −4   66  −90   63 }

{ −88   60   −4  −53   86  −82   42   17  −68   90  −72   21   38  −80   88  −56 }

{ −13   60  −86   85  −56    9   42  −78   90  −72   30   21  −66   88  −82   50 }

{  89  −74   42    0  −42   74  −89   84  −60   21   21  −60   84  −89   74  −42 }

{   9  −42   68  −85   90  −80   60  −30   −4   38  −66   84  −90   82  −63   34 }

{ −90   84  −72   53  −30    4   21  −45   66  −80   88  −89   82  −68   50  −26 }

{  −4   21  −38   53  −66   77  −84   88  −90   88  −82   74  −63   50  −34   17 }

{  90  −89   88  −85   82  −78   74  −68   63  −56   50  −42   34  −26   17   −9 }

},