| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0289 |
| --- | --- |

| Title: | CE4: Parallel Merge Estimation for VVC (CE4.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao Semih Esenlik Biao Wang Anand Meher Kotra Jianle Chen | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

Three methods for parallel estimation of merge mode of neighboring blocks are proposed, which takes into account the binary tree and ternary tree partitions of VVC. The proposed method allows estimation of the merge mode and affine merge mode of two coding blocks if any part of the two blocks are in the same merge estimation region (MER). The proposed methods take into account the historical merge candidates as well as sub-block based merge modes. In solution 1, the MER is defined as fixed square grid. In solution 2, the MER is defined by QT depth. In solution 3, MER is defined as rectangular block with same amount of luma samples. Simulation results for different MER size in different solution are presented in the contribution document.

# Introduction

Parallel merge estimation tool was part of HEVC and in the current VTM software an implementation similar to that of HEVC is present. On the other hand it is currently not in the specification text. This input contribution aims at implementation of parallel merge estimation into VVC, taking into account the new binary tree and ternary tree partitions.

# Problem Statement

In HEVC, if two coding blocks are in the same merge estimation region (MER), the first one is set unavailable for prediction during the merge list construction process of the second block. In VTM, due to binary and ternary splits, two problems are identified that prevent application of the HEVC parallel merge estimation rule directly to VVC. The problems are:

## A block can extend over more than one MER and cover each MER only partially

This problem is exemplified in the following figure.

Figure 1

Block A extends over 2 MERs and it does not cover both MERs completely, which was not the cases in HEVC. The left and bottom-left merge candidate of block B are both inside the block A. In this kind of partition scenarios block A should be marked either available or unavailable for both candidate positions, there shouldn’t be different decisions for the left and the bottom-left merge candidates of block B.

## The block coding order does not necessarily follow z-scan order

This problem is exemplified in the following figure.

Figure 2

In the figure above the coding order of blocks are ABCDE. Blocks A and C are inside the same MER, similarly the blocks B and D are in the same MER. However disabling MV prediction between (marking unavailable for merge) A and C is not enough to enable parallel merge estimation. Block B, which is coded between A and C in coding order, can predict from A and the block C can predict from B. As a result the encoding of block C still needs to wait block A, which means A and C cannot be processed in parallel.

# Proposed Method

## Solution 1

In the solution 1, the MER is defined as a fixed non-overlapped square grid.

According to the proposal the rule for setting a neighbor unavailable for merge list construction is changed as follows.

New parallel merge estimation rule: “A neighbor coding block is marked as unavailable if its bottom right coordinate falls in the extended merge estimation region of the current block”. The extended MER is depicted in the following figure:

Figure 3

Assume that the bottom right coordinate a neighbor block is given by (XNb, YNb) and the top-left coordinate of the current block is given by (XCt, YCt). Moreover assume that the binary logarithm of MER size is given by N. The neighbor block is marked as unavailable for prediction if;

(XNb >> N is equal to XCt >> N) AND (YNb >> N smaller than or equal to YCt >> N)

OR

(YNb >> N is equal to YCt >> N) AND (XNb >> N smaller than or equal to XCt >> N)

## Solution 2

In the solution 2, the MER is defined as a fixed non-overlapped square grid by QT depth.

According to the proposal the rule for setting a neighbor unavailable for merge list construction is changed as follows.

New parallel merge estimation rule: “A neighbor coding block is marked as unavailable if its bottom right coordinate falls in the estimation region of the current block and the QT depth of the current block is equal to or greater than the value of CTU size divided by MER size ”.

Assume that the CTU size is given by CTUSize, the bottom right coordinate a neighbor block is given by (XNb, YNb), the top-left coordinate of the current block is given by (XCt, YCt) and the QT Depth of the current block is given by CuQtDepth. Moreover assume that the binary logarithm of MER size is given by N. The neighbor block is marked as unavailable for prediction if;

(CTUSize >> CuQtDepth smaller than or equal to 1<< N)

AND

(YNb >> N is equal to YCt >> N) AND (XNb >> N equal to XCt >> N)

## Solution 3

### Definition of rectangular MER

In the solution 3, the MER is defined rectangular block with same number of luma samples.

According to the proposal the rule for setting a neighbor unavailable for merge list construction is changed as follows.

New parallel merge estimation rule: “A neighbor coding block is marked as unavailable if its bottom right coordinate falls in the rectangular merge estimation region of the current block”. The MER of the current block is defined in the follow:

Assuming the MER number of luma samples is the MER threshold, an ancestor block of the current block is equal to the threshold. Then the region of the ancestor is defined as the MER of the current block.

For example in the Figure 1, if the current block is block D with size 4x8, the MER threshold is 256, based on the definition of the MER, the whole Figure 1 is defined as the MER of block D. Therefore, block A, B, C and E are marked as unavailable for merge estimation.

### Worst Case Guarantee

Worst case of parallel merge estimation is defined as how many sets of parallel processing merge per MER threshold number of luma samples.

For example if the current block is 4x4, the MER threshold is 32 samples and the parent of the current block is 4x8. Then the 4x8 region is defined as MER based on 3.3.1, and per 32 samples 1 set of parallel processing merge.

However, based on the MER definition in 3.3.1, in some special case, the worst case is not guaranteed by 1 set of parallel processing merge.

For example, if the current block is 4x4, the MER threshold is 32 and the current block is split from QT which means the parent block size is 8x8. Then MER is not defined, and per 64 samples 4 sets of (parallel processing) merge, which equivalents to per 32 samples 2 sets of (parallel processing) merge

In another example, if the current block is 4x4, the MER threshold is 32 and the current block is split from vertical TT which means the parent block size is 4x16. Then MER is not defined, and per 64 samples 3 sets of (parallel processing) merge, which equivalents to per 32 samples 1.5 sets of (parallel processing) merge

To guarantee the worst case, on top of 3.3.1, the MER of current block is additional defined as follow:

Assuming the MER number of luma samples is the MER threshold, an ancestor block of the current block is equal to the threshold multiplied by 2 and the current block is split from QT or TT. Then the region of the ancestor is defined as the MER of the current block.

## Application of the proposal to neighbor blocks with sub-pu based inter prediction

According to the proposal, if a neighbor block is predicted with affine or ATMVP, the bottom right coordinate of the coding block is used (instead of the sub-PUs that result from further partitioning of the CB) to check neighbor block availability. In other words the whole CU is considered as available or unavailable by the current block, not the individual sub-PUs.

## Application of the proposal to history based and long distance merge candidates

It is noted that if the proposed algorithm applies to history based and long distance merge candidates without any change. The bottom right coordinate of candidate in question is used in the proposed algorithm just like spatially adjacent candidates.

# Simulation Results

## Solution 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.07% | 0.06% | 100% | 101% |
| Class A2 | 0.12% | 0.22% | 0.26% | 100% | 101% |
| Class B | 0.12% | 0.28% | 0.17% | 100% | 101% |
| Class C | 0.33% | 0.43% | 0.46% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.27% | 0.25% | 100% | 101% |
| Class D | 0.34% | 0.29% | 0.40% | 100% | 101% |
| Class F (optional) | 0.16% | 0.20% | 0.22% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.12% | -0.05% | 0.30% | 100% | 101% |
| Class C | 0.28% | 0.48% | 0.44% | 100% | 102% |
| Class E | -0.03% | -0.05% | 0.37% | 100% | 101% |
| Overall | 0.14% | 0.13% | 0.36% | 100% | 101% |
| Class D | 0.33% | 0.14% | 0.97% | 100% | 103% |
| Class F (optional) | 0.51% | 0.68% | 0.39% | 102% | 102% |

Table 1: Solution 1 with 8x8 MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.21% | 0.30% | 100% | 101% |
| Class A2 | 0.51% | 0.69% | 0.98% | 101% | 100% |
| Class B | 0.48% | 0.64% | 0.55% | 101% | 101% |
| Class C | 0.95% | 1.03% | 1.27% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.55% | 0.67% | 0.78% | 101% | 101% |
| Class D | 1.02% | 0.97% | 1.13% | 102% | 102% |
| Class F (optional) | 0.58% | 0.64% | 0.67% | 103% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.46% | 0.40% | 0.82% | 100% | 102% |
| Class C | 0.92% | 1.22% | 1.29% | 101% | 102% |
| Class E | 0.25% | 0.36% | -0.27% | 100% | 101% |
| Overall | 0.56% | 0.66% | 0.70% | 101% | 102% |
| Class D | 0.97% | 1.51% | 1.11% | 101% | 103% |
| Class F (optional) | 0.98% | 0.84% | 1.10% | 102% | 103% |

Table 2: Solution 1 with 16x16 MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.54% | 0.64% | 0.70% | 101% | 101% |
| Class A2 | 1.38% | 1.90% | 2.15% | 104% | 100% |
| Class B | 1.34% | 1.48% | 1.28% | 103% | 100% |
| Class C | 2.16% | 2.38% | 2.61% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall | 1.41% | 1.64% | 1.69% | 104% | 100% |
| Class D | 2.07% | 2.32% | 2.43% | 105% | 101% |
| Class F (optional) | 1.18% | 1.29% | 1.38% | 105% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.26% | 0.90% | 1.41% | 103% | 101% |
| Class C | 2.09% | 2.45% | 2.71% | 104% | 101% |
| Class E | 0.75% | 0.74% | 0.18% | 101% | 99% |
| Overall | 1.41% | 1.38% | 1.54% | 103% | 100% |
| Class D | 2.07% | 2.64% | 2.65% | 104% | 101% |
| Class F (optional) | 1.99% | 2.21% | 2.17% | 104% | 102% |

Table 3: Solution 1 with 32x32 MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.17% | 1.15% | 1.40% | 103% | 100% |
| Class A2 | 2.73% | 3.26% | 3.70% | 107% | 99% |
| Class B | 2.65% | 2.48% | 2.49% | 106% | 100% |
| Class C | 3.57% | 3.80% | 4.21% | 109% | 101% |
| Class E |  |  |  |  |  |
| Overall | 2.62% | 2.72% | 2.97% | 106% | 100% |
| Class D | 3.27% | 3.40% | 3.80% | 108% | 101% |
| Class F (optional) | 1.92% | 2.05% | 2.13% | 108% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.52% | 2.20% | 2.46% | 105% | 101% |
| Class C | 3.28% | 3.82% | 3.98% | 106% | 100% |
| Class E | 1.48% | 1.48% | 1.03% | 101% | 100% |
| Overall | 2.51% | 2.56% | 2.61% | 104% | 100% |
| Class D | 3.09% | 3.60% | 3.63% | 106% | 102% |
| Class F (optional) | 2.95% | 3.68% | 3.79% | 106% | 101% |

Table 4: Solution 1 with 64x64 MER

## Solution 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.07% | 99% | 101% |
| Class A2 | 0.00% | 0.00% | 0.08% | 99% | 101% |
| Class B | 0.02% | -0.04% | -0.01% | 98% | 99% |
| Class C | 0.03% | 0.10% | 0.00% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.01% | 0.03% | 99% | 100% |
| Class D | -0.01% | -0.16% | 0.05% | 100% | 101% |
| Class F (optional) | 0.02% | 0.01% | 0.01% | 102% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.23% | 0.19% | 100% | 104% |
| Class C | 0.01% | 0.23% | 0.14% | 99% | 100% |
| Class E | -0.01% | 0.38% | -0.33% | 99% | 98% |
| Overall | 0.01% | 0.08% | 0.04% | 100% | 101% |
| Class D | 0.00% | -0.08% | -0.38% | 100% | 100% |
| Class F (optional) | 0.16% | 0.14% | 0.16% | 101% | 102% |

Table 5: Solution 2 with 8x8 MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.08% | 99% | 101% |
| Class A2 | 0.07% | 0.24% | 0.34% | 101% | 101% |
| Class B | 0.10% | 0.22% | 0.03% | 99% | 98% |
| Class C | 0.42% | 0.42% | 0.61% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.24% | 0.26% | 100% | 99% |
| Class D | 0.54% | 0.56% | 0.63% | 102% | 101% |
| Class F (optional) | 0.33% | 0.33% | 0.40% | 103% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | -0.21% | 0.10% | 101% | 104% |
| Class C | 0.39% | 0.66% | 0.65% | 102% | 99% |
| Class E | -0.05% | -0.10% | -0.29% | 99% | 98% |
| Overall | 0.16% | 0.11% | 0.18% | 101% | 101% |
| Class D | 0.52% | 0.40% | 0.44% | 102% | 100% |
| Class F (optional) | 0.62% | 0.47% | 0.39% | 102% | 102% |

Table 6: Solution 2 with 16x16 MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.24% | 0.34% | 0.53% | 100% | 101% |
| Class A2 | 0.99% | 1.46% | 1.83% | 103% | 100% |
| Class B | 0.98% | 1.03% | 0.98% | 102% | 98% |
| Class C | 1.92% | 2.13% | 2.40% | 104% | 98% |
| Class E |  |  |  |  |  |
| Overall | 1.09% | 1.27% | 1.44% | 102% | 99% |
| Class D | 1.93% | 2.19% | 2.35% | 104% | 100% |
| Class F (optional) | 1.05% | 1.12% | 1.25% | 105% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.95% | 0.60% | 0.83% | 103% | 103% |
| Class C | 1.92% | 2.26% | 2.38% | 103% | 98% |
| Class E | 0.39% | 0.77% | 0.23% | 100% | 97% |
| Overall | 1.13% | 1.20% | 1.20% | 102% | 100% |
| Class D | 2.00% | 2.05% | 2.32% | 103% | 99% |
| Class F (optional) | 1.69% | 1.86% | 2.53% | 104% | 101% |

Table 7: Solution 2 with 32x32 MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.11% | 0.95% | 1.29% | 101% | 99% |
| Class A2 | 2.73% | 3.32% | 3.56% | 105% | 98% |
| Class B | 2.63% | 2.36% | 2.41% | 104% | 97% |
| Class C | 3.53% | 3.82% | 4.28% | 108% | 97% |
| Class E |  |  |  |  |  |
| Overall | 2.59% | 2.66% | 2.92% | 105% | 98% |
| Class D | 3.27% | 3.53% | 3.99% | 108% | 97% |
| Class F (optional) | 1.92% | 2.00% | 2.09% | 107% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.47% | 2.36% | 2.40% | 104% | 100% |
| Class C | 3.25% | 3.45% | 3.92% | 104% | 94% |
| Class E | 1.33% | 1.27% | 0.83% | 99% | 94% |
| Overall | 2.44% | 2.45% | 2.51% | 103% | 96% |
| Class D | 3.21% | 3.61% | 3.89% | 105% | 96% |
| Class F (optional) | 2.85% | 3.25% | 3.61% | 105% | 100% |

Table 8: Solution 2 with 64x64 MER

## Solution 3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.11% | 0.07% | 101% | 101% |
| Class A2 | -0.02% | 0.07% | 0.13% | 101% | 102% |
| Class B | 0.00% | 0.11% | 0.02% | 100% | 101% |
| Class C | 0.03% | -0.01% | -0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.07% | 0.04% | 100% | 101% |
| Class D | -0.03% | -0.19% | -0.13% | 100% | 102% |
| Class F (optional) | 0.00% | 0.05% | 0.04% | 101% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.28% | -0.04% | 102% | 105% |
| Class C | -0.01% | 0.33% | 0.07% | 101% | 105% |
| Class E | 0.02% | 0.17% | 0.12% | 101% | 103% |
| Overall | 0.00% | 0.04% | 0.04% | 101% | 104% |
| Class D | -0.12% | -0.06% | -0.10% | 100% | 103% |
| Class F (optional) | -0.01% | 0.13% | -0.06% | 101% | 102% |

Table 9: Solution 3 with 32 luma samples MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.04% | 101% | 102% |
| Class A2 | 0.00% | 0.05% | 0.13% | 101% | 102% |
| Class B | 0.01% | 0.06% | -0.02% | 100% | 101% |
| Class C | 0.09% | 0.11% | 0.12% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.06% | 0.06% | 101% | 102% |
| Class D | 0.10% | 0.09% | -0.04% | 101% | 103% |
| Class F (optional) | 0.10% | 0.11% | 0.11% | 101% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.05% | -0.08% | 102% | 105% |
| Class C | 0.04% | 0.07% | 0.00% | 101% | 105% |
| Class E | -0.05% | 0.16% | -0.55% | 101% | 103% |
| Overall | 0.01% | 0.04% | -0.17% | 101% | 105% |
| Class D | 0.02% | 0.33% | 0.34% | 100% | 102% |
| Class F (optional) | 0.27% | 0.29% | 0.05% | 101% | 101% |

Table 10: Solution 3 with 64 luma samples MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.06% | 0.08% | 101% | 102% |
| Class A2 | 0.08% | 0.20% | 0.30% | 102% | 102% |
| Class B | 0.09% | 0.16% | 0.13% | 101% | 101% |
| Class C | 0.37% | 0.36% | 0.57% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.15% | 0.20% | 0.27% | 101% | 102% |
| Class D | 0.42% | 0.40% | 0.47% | 103% | 102% |
| Class F (optional) | 0.26% | 0.26% | 0.31% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.02% | -0.13% | 102% | 105% |
| Class C | 0.32% | 0.46% | 0.35% | 102% | 104% |
| Class E | 0.04% | 0.29% | 0.00% | 101% | 104% |
| Overall | 0.15% | 0.22% | 0.06% | 102% | 104% |
| Class D | 0.36% | 0.82% | 0.81% | 103% | 103% |
| Class F (optional) | 0.54% | 0.59% | 0.64% | 102% | 101% |

Table 11: Solution 3 with 128 luma samples MER

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.00% | 0.27% | 101% | 102% |
| Class A2 | 0.27% | 0.50% | 0.62% | 102% | 102% |
| Class B | 0.27% | 0.38% | 0.27% | 101% | 101% |
| Class C | 0.73% | 0.93% | 1.04% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.35% | 0.47% | 0.55% | 102% | 101% |
| Class D | 0.80% | 0.83% | 1.03% | 102% | 102% |
| Class F (optional) | 0.42% | 0.43% | 0.50% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 with VTM configuration |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.24% | -0.10% | 0.37% | 102% | 104% |
| Class C | 0.69% | 0.96% | 0.85% | 102% | 105% |
| Class E | 0.15% | 0.11% | 0.26% | 101% | 103% |
| Overall | 0.37% | 0.31% | 0.50% | 102% | 104% |
| Class D | 0.82% | 1.04% | 1.08% | 103% | 102% |
| Class F (optional) | 0.82% | 1.22% | 0.81% | 102% | 102% |

Table 12: Solution 3 with 256 luma samples MER

# Conclusion

Three methods for estimation of merge mode of neighboring blocks in parallel are proposed, which takes into account the binary tree and ternary tree partitions of VVC. It is asserted that the increased picture resolution necessitates the usage of block based parallelization tools such as parallel merge estimation. The contribution is proposed to be included in the VVC.

# Reference

[1] “ Versatile Video Coding (Draft 3)”, B. Bross, J. Chen, S. Liu, doc. JVET-L1001, Oct. 2018

[2] “Non-CE4: Parallel Merge Estimation for VVC”, S. Esenlik, H. Gao, B. Wang, A.M. Kotra, J. Chen, doc. JVET-L0216, Oct. 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).