| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M300-v1 |
| --- | --- |

| Title: | CE4-related: HMVP and parallel processing with tiles and tile groups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Jianle Chen, Semih Esenlik, Biao Wang, Han Gao, | Tel: Email: | anand.meher.kotra@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

In VTM-3.0, History-based motion vector prediction (HMVP) FIFO table is reset for each CTU Row to enable parallel processing for tools such as Wavefronts (WPP). From VTM-3.1 onwards the concept of slices is replaced with tile groups. Therefore this contribution extends the technique of re-setting the HMVP table for Tiles and Tile groups. Specifically, the HMVP table is re-set at the beginning of a new Tile group or a new Tile and also at the beginning of each CTU row inside a Tile.

# Introduction

VTM-3.0 includes a new motion vector prediction tool called HMVP [3].  To enable parallel processing of tools such as wavefront parallel processing (WPP) with HMVP tool, the HMVP FIFO table is re-set at the beginning of each CTU row inside a given slice. During the 12th JVET meeting in Macau, the concept of slices has been replaced with Tile groups. An initial version of the syntax specification with Tile groups is provided in the document JVET-L0686 [6]. The current contribution extends the solution of “re-setting the HMVP table” for the case of Tiles and Tile groups.

# Proposed Solution

As per the VVC working draft text [2], the syntax for re-setting the HMVP FIFO table at the beginning of each CTU row inside a slice is as given below.

if (slice_type != I && (CtbAddrInRs % PicWidthInCtbsY = = 0 ) ¦¦ ( CtbAddrInRs = = slice_address ) )

NumHmvpCand = 0

The expression CtbAddrInRs % PicWidthInCtbsY == 0, gives the raster scan address of the first CTU for each CTU row inside a slice and the HMVP table is then re-set by setting the variable NumHmvpCand to 0.

During the 12th JVET meeting in Macau, the concept of Slices was replaced by Tile groups. Fig. 1 below shows an example of 12 tiles divided into 3 tile groups.

Figure 1. Picture divided into 12 tiles and 3 tile groups.

Current proposal extends the solution of “re-setting HMVP table” for tiles and tile groups. HMVP table is re-set at the beginning of each tile group and tile and also at the beginning of each CTU row inside a Tile.

Fig. 2 below shows an example, where the HMVP list is re-set at the beginning of each CTU-row inside a Tile.

Figure 2. HMVP list re-set at the beginning of tile groups, tiles and beginning of each CTU row inside Tile

The corresponding syntax specification to re-set the HMVP table is as given below

| tile_group_data( ) { | Descriptor |
| --- | --- |
| tileIdx = tile_group_address |  |
| for( i = 0; i <= num_tiles_in_tile_group_minus1; i++, tileIdx++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ tileIdx ] |  |
| for( j = 0; j < NumCtusInTile[ tileIdx ]; j++, ctbAddrInTs++ ) { |  |
| firstCtbInCtuRowInTile = j % ColWidth[tileIdx] == 0) ? 1 : 0 if ( ( tile_group_type ! = I ) && ( firstCtbInCtuRowInTile ) ) HMVPCandNum = 0 |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < num_tiles_in_tile_group_minus1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |

FirstCtbInCtuRowInTile=CtbAddrInTs-FirstCtbAddrInTstileIdx-NumCtusInTiletileIdx% ColWidthtileIdx==0?1 :0

# Conclusion

This contribution extends the concept of “re-setting HMVP table” at the beginning of each CTU row to tiles and tile groups. The HMVP table is re-set at the beginning of a new tile group or a new tile and also at the beginning of each CTU row inside a tile. The corresponding syntax is also provided in the document.

# References

- Spec text for the agreed starting point on slicing and tiling, JVET-L0686, Ye-Kui, Hendry, et al.

- Versatile Video Coding (Draft 3), Document JVET-L01001_v10, B. Bross, J. Chen, S. Liu

- CE4: History-based Motion Vector Prediction (Test 4.4.7),  JVET-L0226

- CE4-related: Modified History-based MVP to support parallel processing, JVET-L0106

- CE4-related: History-Based Motion Vector Prediction considering parallel processing, JVET-L0158

- Spec text for the agreed starting point on slicing and tiling, JVET-L0686

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).