| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0301-v2 |
| --- | --- |

| Title: | Non-CE: Loop filter line buffer reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Semih Esenlik, Jianle Chen, Biao Wang, Han Gao, | Tel: Email: | anand.meher.kotra@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

The current contribution proposes a mechanism of reducing the line buffer requirement of ALF (adaptive loop filter). The contribution uses the concept of virtual boundaries (VBs) which are upward shifted horizontal CTU boundaries by “N” samples. Modified ALF block classification and modified ALF filtering are applied for the samples which are near the virtual boundary to reduce the number of line buffers required. Modified ALF block classification only uses the samples which are above the VB to classify the given 4 x 4 block which is above VB. Similarly for the classification of the 4 x 4 block below VB, samples belonging to the lines below the VB are used.   Modified ALF filtering uses a combination of conditional disabling and truncated versions of the original ALF filter.

Objective results are as follows:

Over VTM 3.0 Anchor, for the case when “N” is 4 (AI, RA, LDB, LDP): Luma BD-Rate of 0.07%, 0.09%, 0.08%, 0.07% with no increase in EncT and DecT.

Over VTM 3.0 Anchor, for the case when “N” is 6 (AI, RA, LDB, LDP): Luma BD-Rate of 0.10%, 0.12%, 0.16%, 0.16% with no increase in EncT and DecT.

Subjective evaluation showed no visual artifacts.

# Introduction

VTM 3.0 [2] consists of three cascaded in-loop filter which are namely Deblocking filter (DBF), Sample adaptive offset (SAO), and Adaptive loop filter (ALF).  The concept of virtual boundary (VB) as introduced in [3] is re-used in this contribution. Virtual boundaries (VBs) are upward shifted horizontal CTU boundaries by “N” samples.  As shown in Fig. 1 the total number of line buffers required is 11.25 lines for the Luma component.

Figure 1. Loop filter line buffer requirement in VTM-3.0 for Luma component

The explanation of the line buffer requirement is as follows: The deblocking of horizontal edge overlapping with CTU edge cannot be performed as the decisions and filtering require lines K,L,M,M from the first CTU and Lines O,P.. from the bottom CTU. Therefore the deblocking of the horizontal edges overlapping with the CTU boundary is postponed until the lower CTU comes. Therefore for the lines K, L, M, N  reconstruncted luma samples have to be stored in the line buffer (4 lines). Then the SAO filtering can be performed for lines A till J. The line J can be SAO filtered as deblocking does not change the samples in line K. For SAO filtering of line K, the edge offset classification decision is only stored in the line buffer (which is 0.25 Luma lines). The ALF filtering can only be performed for lines A-F. As shown in Fig. 1, the ALF classification is performed for each 4 x 4 block. Each 4 x 4 block classification needs an activity window of size 8 x 8 which in turn needs a 9 x 9 window to compute the 1d Laplacian to determine the gradient.

Therefore for the block classification of the 4 x 4 block overlapping with lines G, H, I, J needs, SAO filtered samples below the Virtual boundary. Also the SAO filtered samples of lines D, E, F are required for ALF classification. Also ALF filtering of Line G needs three SAO filtered lines D, E, F from Above. Therefore the total line buffer requirement is as follows:

Lines K-N (Horizontal DF pixels) – 4

Lines D-J (SAO filtered pixels) – 7

SAO Edge offset classifier values between line J and line K – 0.25

Therefore the total number of Luma lines required is 7 + 4 + 0.25 = 11.25.

Fig. 2 shows the line buffer requirement of the Chroma component.  The line buffer requirement for Chroma component is evaluated to be 6.25 lines

Figure 2. Loop filter line buffer requirement in VTM-3.0 for Chroma component

It is emphasized that 11.25 lines of Luma line buffer and 6.25 lines of Chroma line buffer is considered expensive for hardware implementations of the VVC standard and therefore reduction of line buffer requirement of loop filter is desired.

This contribution proposes modified version of ALF block classification and filtering to ensure that the line buffer requirement of ALF filter is zero. This contribution covers two cases.  Case 1: When virtual boundary is 4 lines above the CTU boundary (N = 4), Case 2: When virtual boundary is 6 lines above the CTU boundary (N = 6).

Longer tap deblocking filters are currently being studied in CE11 [4], and the case with “N= 6” covers the scenario when deblocking filter is allowed to use up to 6 lines for deblocking decisions.

# Proposed Solution

## Modified ALF block classification when VB size N is 4

Fig. 3a and 3b depict modified block classification for the case when the virtual boundary is 4 lines above the CTU boundary (N = 4). As depicted in Fig. 3a, for the 4 x 4 block starting at line G, the block classification only uses the lines E till J.  However Laplacian gradient calculation for the samples belonging to line J requires one more line below (line K). Therefore line K is padded with line J.

Similarly as depicted in Fig. 3b, for the 4 x 4 block starting at line K, the block classification only uses the lines K till P.  However Laplacian gradient calculation for the samples belonging to line K require one more line above (line J). Therefore line J is padded with line K.

Figure 3. Modified ALF block classification at virtual boundary when N = 4

## Modified ALF block classification when VB size N is 6

As depicted in Fig. 4, When N is 6, the 4 x 4 block starting at line G crosses the virtual boundary.  For the 4 x 4 block starting at line C, line H is padded to line I to facilitate the ALF block classification.  For the 4 x 4 block starting at line G, the samples in line G, H, re-use the block classification decision from blocks C, D, E, F and the samples in line I, J re-use the block classification decision from the block overlapping the lines K, L, M, N.

Figure 4. Modified ALF block classification at virtual boundary when N = 6

## Modified ALF block filtering

### Truncated ALF filtering at virtual boundaries

As depicted in Fig.4, truncated version of the filters are used for filtering of the Luma samples belonging to the lines close to the virtual boundaries.  These truncated versions of the filters can be used for both N=4 and N=6 case.

Figure 4. Modified Luma ALF filtering at virtual boundary

Similarly as depicted in Fig. 5, truncated version of the filters are also used for chroma ALF filtering

Figure 5. Modified Chroma ALF filtering at virtual boundary

### Conditional Disabling of ALF filtering at virtual boundaries

In addition to the truncated version of the ALF filtering, conditional disabling of ALF filtering is also considered.

In Figure 4, if absolute sum of the padded coefficients is greater than a threshold (T) times the absolute sum of the non-padded coefficients, then the filter is disabled for those respective lines.

For e.g. with lines M and H, if (2*abs  c0>(T* 2*abs c1+abs c2+absc3+absc4+absc5+absc6+absc7+absc8+absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines M and H.

Similarly for lines L and I, if (2*(abs c0+ abs c1 + abs c2+ abs c3)>(T* 2*absc4+absc5+absc6+absc7+absc8+absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines L and I.

Similarly for lines J and K, if (2*(abs c0+ abs c1 + abs c2+ abs c3+absc4+absc5+absc6+absc7+absc8)>(T* 2*absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines J and K.

Where T is generally a threshold which can take a fractional values ranging between 0 and 1. For example, T value can be 0.5, 0.6 or 0.7

Similarly for Chroma, as depicted in Fig. 5, for lines N and K, if (2*abs  c0>(T* 2*abs c1+abs c2+absc3+absc4+absc5)+absc6), then the ALF filtering is disabled for lines N and K.

for lines M and L, if ((2*(abs c0+ abs c1 + abs c2+ abs c3)>(T* 2*absc4+absc5)+absc6), then the ALF filtering is disabled for lines M and L.

the function abs(x) is defined as follows;Abs( x ) =  x;x  >=  0-x;x < 0

# Simulation Results

The proposed methods are implemented on top of VTM-3.0 [2] and the simulation is performed under common test conditions as described in JVET-L1010 [5] with ALF SIMD turned off in both the Anchor and Test.

## Results for the case when N=4, without conditional disabling of ALF filtering

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.15% | 0.14% | 100% | 102% |
| Class A2 | 0.08% | 0.07% | 0.07% | 100% | 101% |
| Class B | 0.05% | 0.05% | 0.05% | 100% | 101% |
| Class C | 0.06% | 0.04% | 0.06% | 100% | 100% |
| Class E | 0.08% | 0.06% | 0.02% | 100% | 101% |
| Overall | 0.06% | 0.07% | 0.06% | 100% | 101% |
| Class D | 0.03% | 0.04% | 0.05% | 100% | 100% |
| Class F | 0.07% | 0.04% | 0.04% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.03% | 0.02% | 101% | 103% |
| Class A2 | 0.12% | 0.13% | 0.13% | 101% | 102% |
| Class B | 0.08% | 0.15% | 0.07% | 101% | 102% |
| Class C | 0.10% | 0.12% | 0.21% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.12% | 0.11% | 101% | 102% |
| Class D | 0.05% | -0.14% | 0.02% | 101% | 100% |
| Class F | 0.06% | 0.01% | 0.01% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | 0.03% | -0.12% | 101% | 102% |
| Class C | 0.11% | 0.43% | 0.38% | 101% | 101% |
| Class E | 0.12% | 0.48% | 0.21% | 101% | 101% |
| Overall | 0.11% | 0.27% | 0.13% | 101% | 101% |
| Class D | 0.07% | 0.01% | 0.24% | 100% | 100% |
| Class F | 0.21% | -0.09% | 0.21% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.26% | 0.29% | 100% | 103% |
| Class C | 0.10% | 0.26% | 0.09% | 101% | 101% |
| Class E | 0.27% | -0.30% | -0.60% | 101% | 100% |
| Overall | 0.14% | 0.12% | 0.00% | 101% | 102% |
| Class D | 0.06% | -0.33% | -0.14% | 101% | 101% |
| Class F | 0.18% | 0.02% | -0.30% | 101% | 101% |

## Results for the case when N=6, without conditional disabling of ALF filtering

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.15% | 0.14% | 100% | 102% |
| Class A2 | 0.13% | 0.07% | 0.07% | 100% | 101% |
| Class B | 0.09% | 0.05% | 0.05% | 100% | 102% |
| Class C | 0.08% | 0.04% | 0.06% | 100% | 100% |
| Class E | 0.15% | 0.06% | 0.02% | 100% | 102% |
| Overall | 0.11% | 0.07% | 0.06% | 100% | 101% |
| Class D | 0.05% | 0.05% | 0.05% | 100% | 100% |
| Class F | 0.12% | 0.05% | 0.05% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.04% | 0.08% | 101% | 103% |
| Class A2 | 0.16% | 0.04% | 0.15% | 101% | 102% |
| Class B | 0.12% | 0.16% | 0.02% | 101% | 101% |
| Class C | 0.12% | 0.06% | 0.13% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | 0.08% | 0.09% | 101% | 102% |
| Class D | 0.07% | -0.03% | 0.14% | 101% | 100% |
| Class F | 0.12% | 0.02% | 0.04% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.15% | -0.21% | -0.17% | 101% | 102% |
| Class C | 0.14% | 0.29% | 0.29% | 101% | 101% |
| Class E | 0.33% | 0.22% | 0.05% | 102% | 100% |
| Overall | 0.19% | 0.06% | 0.04% | 101% | 101% |
| Class D | 0.09% | 0.38% | 0.00% | 101% | 101% |
| Class F | 0.31% | 0.28% | 0.07% | 101% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.18% | 0.26% | 0.14% | 101% | 102% |
| Class C | 0.15% | 0.25% | 0.07% | 101% | 101% |
| Class E | 0.26% | 0.22% | -0.54% | 102% | 101% |
| Overall | 0.19% | 0.25% | -0.05% | 101% | 102% |
| Class D | 0.09% | -0.48% | -0.03% | 101% | 101% |
| Class F | 0.25% | -0.32% | -0.19% | 101% | 101% |

## Results for the case when N=4, with conditional disabling of ALF filtering

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.22% | 0.20% | 100% | 100% |
| Class A2 | 0.09% | 0.09% | 0.08% | 100% | 100% |
| Class B | 0.05% | 0.06% | 0.06% | 100% | 100% |
| Class C | 0.04% | 0.05% | 0.07% | 100% | 100% |
| Class E | 0.08% | 0.12% | 0.10% | 100% | 99% |
| Overall | 0.07% | 0.10% | 0.09% | 100% | 100% |
| Class D | 0.03% | 0.05% | 0.05% | 100% | 99% |
| Class F | 0.05% | 0.04% | 0.04% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.07% | 0.06% | 101% | 100% |
| Class A2 | 0.14% | 0.15% | 0.18% | 101% | 100% |
| Class B | 0.07% | 0.12% | 0.06% | 101% | 100% |
| Class C | 0.05% | 0.12% | 0.16% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.12% | 0.11% | 101% | 100% |
| Class D | 0.03% | -0.15% | 0.13% | 101% | 99% |
| Class F | 0.03% | 0.01% | 0.07% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.09% | 0.28% | 101% | 99% |
| Class C | 0.07% | 0.51% | 0.30% | 101% | 100% |
| Class E | 0.10% | 0.19% | 0.22% | 101% | 99% |
| Overall | 0.08% | 0.18% | 0.27% | 101% | 99% |
| Class D | 0.04% | 0.26% | -0.05% | 100% | 99% |
| Class F | 0.18% | 0.30% | 0.54% | 101% | 99% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.22% | 0.03% | 100% | 100% |
| Class C | 0.03% | 0.18% | 0.10% | 100% | 98% |
| Class E | 0.08% | 0.04% | -0.46% | 101% | 98% |
| Overall | 0.07% | 0.16% | -0.07% | 101% | 99% |
| Class D | -0.04% | -0.14% | 0.22% | 100% | 98% |
| Class F | 0.30% | -0.12% | -0.23% | 101% | 99% |

## Results for the case when N=6, with conditional disabling of ALF filtering

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.22% | 0.20% | 100% | 98% |
| Class A2 | 0.15% | 0.09% | 0.08% | 100% | 98% |
| Class B | 0.09% | 0.06% | 0.06% | 100% | 98% |
| Class C | 0.06% | 0.05% | 0.07% | 100% | 99% |
| Class E | 0.14% | 0.12% | 0.09% | 100% | 99% |
| Overall | 0.10% | 0.10% | 0.09% | 100% | 99% |
| Class D | 0.05% | 0.05% | 0.05% | 100% | 100% |
| Class F | 0.10% | 0.05% | 0.05% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.04% | 0.06% | 101% | 101% |
| Class A2 | 0.18% | 0.03% | 0.12% | 100% | 101% |
| Class B | 0.11% | 0.17% | -0.01% | 101% | 100% |
| Class C | 0.10% | 0.11% | 0.15% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.10% | 0.07% | 100% | 101% |
| Class D | 0.07% | -0.03% | 0.14% | 100% | 101% |
| Class F | 0.08% | 0.00% | 0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.14% | 0.02% | -0.06% | 100% | 101% |
| Class C | 0.09% | 0.29% | 0.36% | 101% | 100% |
| Class E | 0.28% | 0.38% | -0.01% | 101% | 100% |
| Overall | 0.16% | 0.20% | 0.09% | 100% | 101% |
| Class D | 0.09% | -0.16% | -0.36% | 100% | 101% |
| Class F | 0.19% | 0.21% | 0.03% | 100% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.18% | 0.37% | 0.43% | 100% | 101% |
| Class C | 0.07% | 0.39% | 0.09% | 100% | 101% |
| Class E | 0.23% | 0.55% | -0.41% | 100% | 100% |
| Overall | 0.16% | 0.42% | 0.11% | 100% | 101% |
| Class D | 0.07% | 0.01% | 0.15% | 100% | 101% |
| Class F | 0.22% | -0.28% | 0.48% | 100% | 101% |

## Subjective evaluation

Subjective evaluation shows there are no visual boundary artifacts observed near the CTU boundaries.      Fig. 6 below shows a visual example of POC 16, from Johnny sequence, encoded in LDP configuration with QP 37. Fig. 7 shows POC 16 with partitioning information (CTU boundaries are marked).

Figure 6.  Modified ALF filtering with zero line buffer

Figure 7.  Modified ALF filtering with zero line buffer with CTU boundaries marked

# Conclusion

This contribution proposes modified versions of ALF block classification and ALF filtering to completely avoid line buffers for the ALF filtering. ALF block classification only uses samples above the virtual boundary (VB) to classify the blocks which are above the virtual boundary and vice versa. Truncated version of the ALF filter are used for filtering of the samples close to the virtual boundary. Moreover ALF filtering is conditionally disabled in certain cases. Experimental results show that an overall Luma BD-Rate loss of around 0.1% is observed with negligible increase in complexity. It is suggested to adopt this technique to the VVC draft specification and the VTM reference software.

# References

- Description of Core Experiment 9 (CE9): Decoder-side motion derivation, Document JVET-L1026, 	X. Xiu, S. Esenlik

- Versatile Video Coding (Draft 3), Document JVET-L01001_v10, B. Bross, J. Chen, S. Liu

- Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9, JCTVC-G212, C.-Y.Chen et al.

- Description of Core Experiment 9 (CE11): Deblocking, Document JVET-L1031, A. Norkin, A.M. Kotra

- “JVET common test conditions and software reference configurations”, JVET-L1010

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).