| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18Jan. 2019 | Document: JVET-M0308 |
| --- | --- |

| Title: | Non-CE4: MMVD simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Xu Chen Jianhua Zheng | Tel: Email: | +86 13480990067 anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution presents simplification method for MMVDbased on VTM-3.0 that remove scaling process. It has no performance impact to VTM-3.0 anchor.

Proposed improvements

- 8.3.2.7 Derivation process for merge motion vector difference

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Outputs of this process are the lumamerge motion vector differences in 1/16 fractional-sample accuracymMvdL0 andmMvdL1.

The variable currPic specifies the current picture.

The lumamerge motion vector differencesmMvdL0 andmMvdL1 are derived as follows:

- If both predFlagL0 and predFlagL1 are equal to 1, the following applies:

currPocDiffL0  =  DiffPicOrderCnt( currPic, RefPicList0[ refIdxL0 ] )	(8-272)

currPocDiffL1  =  DiffPicOrderCnt( currPic, RefPicList1[ refIdxL1 ] )	(8-273)

- If −currPocDiffL0 * currPocDiffL1 is greater than 0, the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ 0 ]		(8-274)

mMvdL0[ 1 ]  =  MmvdOffset[ 1 ]		(8-275)

mMvdL1[ 0 ]  =  −MmvdOffset[ 0 ]		(8-276)

mMvdL1[ 1 ]  =  −MmvdOffset[ 1 ]		(8-277)

- Otherwise ( −currPocDiffL0 * currPocDiffL1 is less than 0 ), the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ 0 ]		(8-278)

mMvdL0[ 1 ]  =  MmvdOffset[ 1 ]		(8-279)

mMvdL1[ 0 ]  =  MmvdOffset[ 0 ]		(8-280)

mMvdL1[ 1 ]  =  MmvdOffset[ 1 ]		(8-281)

- If Abs( currPocDiffL0 ) is greater than Abs( currPocDiffL1 ), the following applies:

td = Clip3( −128, 127, currPocDiffL0 )		(8-282)

tb = Clip3( −128, 127, currPocDiffL1 )		(8-283)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-284)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-285)

mMvdL1[ 0 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL1[ 0 ] )  *  	(8-286)
	( ( Abs( distScaleFactor * mMvdL1[ 0 ] ) + 127 )  >>  8 ) )

mMvdL1[ 1 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL1[ 1 ] )  *  	(8-287)
	( ( Abs( distScaleFactor * mMvdL1[ 1 ] ) + 127 )  >>  8 ) )

- Otherwise if Abs( currPocDiffL0 ) is less than Abs( currPocDiffL0 ), the following applies:

td = Clip3( −128, 127, currPocDiffL1 )		(8-288)

tb = Clip3( −128, 127, currPocDiffL0 )		(8-289)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-290)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-291)

mMvdL0[ 0 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL0[ 0 ] )  *  	(8-292)
	( ( Abs( distScaleFactor * mMvdL0[ 0 ] ) + 127 )  >>  8 ) )

mMvdL0[ 1 ] = Clip3( −215, 215 − 1, Sign( distScaleFactor * mMvdL0[ 1 ] )  *  	(8-293)
	( ( Abs( distScaleFactor * mMvdL0[ 1 ] ) + 127 )  >>  8 ) )

- Otherwise( predFlagL0 or predFlagL1 are equal to 1 ), the following applies for X being 0 and 1:

mMvdLX[ 0 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ 0 ]  :  0	(8-294)

mMvdLX[ 1 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ 1 ]  :  0	(8-295)

Experimental Results

The proposed method has been implemented on VTM-3.0[1], and allthe tests were conducted under the common testconditions[2]. The experimental results of themerge offset extensioncompared to VTM-3.0 are shown in Table1.

Table 1. Experimental results of Solution

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.03% | 97% | 99% |
| Class A2 | 0.00% | -0.01% | 0.03% | 97% | 99% |
| Class B | -0.01% | 0.03% | 0.02% | 97% | 96% |
| Class C | -0.01% | -0.01% | 0.03% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | 0.03% | 97% | 98% |
| Class D | -0.02% | -0.03% | 0.01% | 97% | 99% |
| Class F (optional) | -0.01% | -0.01% | -0.02% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.14% | -0.16% | 99% | 96% |
| Class C | 0.09% | 0.21% | 0.10% | 100% | 101% |
| Class E | 0.03% | 0.15% | -0.05% | 100% | 100% |
| Overall | 0.08% | 0.05% | -0.05% | 100% | 99% |
| Class D | 0.15% | 0.30% | 0.19% | 100% | 99% |
| Class F (optional) | 0.10% | -0.04% | 0.07% | 100% | 100% |

Reference

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/VVCSoftware_VTM.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, China, Oct. 2018.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).