| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0310 |
| --- | --- |

| Title: | CE2-related: Using shorter-tap filter for 4x4 sized partition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Ru-ling Liao Chong Soon Lim | Tel: Email: | +65-6550-5423 jingya.li@sg.panasonic.com Chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution uses shorter tap filter is used for 4x4 sized partitions including 4x4 CU and sub-block of affine. For test 1, it reduces worst case memory bandwidth of 4x4 sized CU by 67% and complexity of affine to 65% regarding number of multiplications and 62% regarding number of additions. Experimental result shows 0.08% luma BD-rate change in RA and 0.14% luma BD-rate change in LD_B. Ignorable change in encoding and decoding time are observed.

# Algorithm description

In this contribution shorter tap filter is used for 4x4 sized partitions including 4x4 CU and sub-block of affine CU. There are in total 2 tests as below:

Test1: use 6-tap filter (table 4).

Test2: use 4-tap filter (table 5).

The first purpose is to reduce memory bandwidth of 4x4 sized CU (uni-pred) to 67% (test1) and 40% (test 2) as illustrated in table 1. The second purpose is to reduce computational complexity of affine CU (bi-pred with 4x4 size as sub-block size) to 65% (test1) and 37% (test2) in terms of multiplications and 62% (test1) and 31% (test 2) in terms of additions. Details are given in table 2-3.

Table 1: memory bandwidth using difference sizes of filter tap for 4x4 sized uni-pred CU

| filter size | memory bandwidth | ratio to 8 tap |
| --- | --- | --- |
| 8 | 121 | - |
| 6 | 81 | 67% |
| 4 | 49 | 40% |

Table 2: number of multiplications using difference sizes of filter tap for affine CU

| W | H | 8 tap # multiplications | 6 tap # multiplications | ratio to 8 tap | 4 tap # multiplications | ratio to 8 tap |
| --- | --- | --- | --- | --- | --- | --- |
| 128 | 128 | 983,040 | 638,976 | 65% | 360,448 | 37% |
| 128 | 64 | 491,520 | 319,488 | 65% | 180,224 | 37% |
| 64 | 128 | 491,520 | 319,488 | 65% | 180,224 | 37% |
| 128 | 32 | 245,760 | 159,744 | 65% | 90,112 | 37% |
| 32 | 128 | 245,760 | 159,744 | 65% | 90,112 | 37% |
| 128 | 16 | 122,880 | 79,872 | 65% | 45,056 | 37% |
| 16 | 128 | 122,880 | 79,872 | 65% | 45,056 | 37% |
| 128 | 8 | 61,440 | 39,936 | 65% | 22,528 | 37% |
| 8 | 128 | 61,440 | 39,936 | 65% | 22,528 | 37% |
| 64 | 64 | 245,760 | 159,744 | 65% | 90,112 | 37% |
| 64 | 32 | 122,880 | 79,872 | 65% | 45,056 | 37% |
| 32 | 64 | 122,880 | 79,872 | 65% | 45,056 | 37% |
| 64 | 16 | 61,440 | 39,936 | 65% | 22,528 | 37% |
| 16 | 64 | 61,440 | 39,936 | 65% | 22,528 | 37% |
| 64 | 8 | 30,720 | 19,968 | 65% | 11,264 | 37% |
| 8 | 64 | 30,720 | 19,968 | 65% | 11,264 | 37% |
| 32 | 32 | 61,440 | 39,936 | 65% | 22,528 | 37% |
| 32 | 16 | 30,720 | 19,968 | 65% | 11,264 | 37% |
| 16 | 32 | 30,720 | 19,968 | 65% | 11,264 | 37% |
| 32 | 8 | 15,360 | 9,984 | 65% | 5,632 | 37% |
| 8 | 32 | 15,360 | 9,984 | 65% | 5,632 | 37% |
| 16 | 16 | 15,360 | 9,984 | 65% | 5,632 | 37% |
| 16 | 8 | 7,680 | 4,992 | 65% | 2,816 | 37% |
| 8 | 8 | 3,840 | 2,496 | 65% | 1,408 | 37% |

Table 3: number of additions using difference sizes of filter tap for affine CU

| W | H | 8 tap # multiplications | 6 tap # multiplications | ratio to 8 tap | 4 tap # multiplications | ratio to 8 tap |
| --- | --- | --- | --- | --- | --- | --- |
| 128 | 128 | 860,160 | 532,480 | 62% | 270,336 | 31% |
| 128 | 64 | 430,080 | 266,240 | 62% | 135,168 | 31% |
| 64 | 128 | 430,080 | 266,240 | 62% | 135,168 | 31% |
| 128 | 32 | 215,040 | 133,120 | 62% | 67,584 | 31% |
| 32 | 128 | 215,040 | 133,120 | 62% | 67,584 | 31% |
| 128 | 16 | 107,520 | 66,560 | 62% | 33,792 | 31% |
| 16 | 128 | 107,520 | 66,560 | 62% | 33,792 | 31% |
| 128 | 8 | 53,760 | 33,280 | 62% | 16,896 | 31% |
| 8 | 128 | 53,760 | 33,280 | 62% | 16,896 | 31% |
| 64 | 64 | 215,040 | 133,120 | 62% | 67,584 | 31% |
| 64 | 32 | 107,520 | 66,560 | 62% | 33,792 | 31% |
| 32 | 64 | 107,520 | 66,560 | 62% | 33,792 | 31% |
| 64 | 16 | 53,760 | 33,280 | 62% | 16,896 | 31% |
| 16 | 64 | 53,760 | 33,280 | 62% | 16,896 | 31% |
| 64 | 8 | 26,880 | 16,640 | 62% | 8,448 | 31% |
| 8 | 64 | 26,880 | 16,640 | 62% | 8,448 | 31% |
| 32 | 32 | 53,760 | 33,280 | 62% | 16,896 | 31% |
| 32 | 16 | 26,880 | 16,640 | 62% | 8,448 | 31% |
| 16 | 32 | 26,880 | 16,640 | 62% | 8,448 | 31% |
| 32 | 8 | 13,440 | 8,320 | 62% | 4,224 | 31% |
| 8 | 32 | 13,440 | 8,320 | 62% | 4,224 | 31% |
| 16 | 16 | 13,440 | 8,320 | 62% | 4,224 | 31% |
| 16 | 8 | 6,720 | 4,160 | 62% | 2,112 | 31% |
| 8 | 8 | 3,360 | 2,080 | 62% | 1,056 | 31% |

Table 4: test 1 interpolation filter

| MV | tap filter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 64 | 0 | 0 | 0 |
| 1/16 | 1 | -3 | 63 | 4 | -2 | 1 |
| 1/8 | 1 | -5 | 62 | 8 | -3 | 1 |
| 3/16 | 2 | -8 | 60 | 13 | -4 | 1 |
| 1/4 | 3 | -10 | 58 | 17 | -5 | 1 |
| 5/16 | 3 | -11 | 52 | 26 | -8 | 2 |
| 3/8 | 2 | -9 | 47 | 31 | -10 | 3 |
| 7/16 | 3 | -11 | 45 | 34 | -10 | 3 |
| 1/2 | 3 | -11 | 40 | 40 | -11 | 3 |
| 9/16 | 3 | -10 | 34 | 45 | -11 | 3 |
| 5/8 | 3 | -10 | 31 | 47 | -9 | 2 |
| 11/16 | 2 | -8 | 26 | 52 | -11 | 3 |
| 6/8 | 1 | -5 | 17 | 58 | -10 | 3 |
| 13/16 | 1 | -4 | 13 | 60 | -8 | 2 |
| 7/8 | 1 | -3 | 8 | 62 | -5 | 1 |
| 15/16 | 1 | -2 | 4 | 63 | -3 | 1 |

Table 5: test 2 interpolation filter

| MV | tap filter |  |  |  |
| --- | --- | --- | --- | --- |
| 0 | 0 | 64 | 0 | 0 |
| 1/16 | -2 | 62 | 4 | 0 |
| 1/8 | -2 | 58 | 10 | -2 |
| 3/16 | -4 | 56 | 14 | -2 |
| 1/4 | -4 | 54 | 16 | -2 |
| 5/16 | -6 | 52 | 20 | -2 |
| 3/8 | -6 | 46 | 28 | -4 |
| 7/16 | -4 | 42 | 30 | -4 |
| 1/2 | -4 | 36 | 36 | -4 |
| 9/16 | -4 | 30 | 42 | -4 |
| 5/8 | -4 | 28 | 46 | -6 |
| 11/16 | -2 | 20 | 52 | -6 |
| 6/8 | -2 | 16 | 54 | -4 |
| 13/16 | -2 | 14 | 56 | -4 |
| 7/8 | -2 | 10 | 58 | -2 |
| 15/16 | 0 | 4 | 62 | -2 |

# Experimental Results

Table 6: simulation results of test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.05% | -0.04% | 100% |  |
| Class A2 | 0.09% | 0.09% | 0.20% | 100% |  |
| Class B | 0.05% | 0.08% | -0.10% | 100% |  |
| Class C | 0.16% | 0.10% | 0.09% | 100% |  |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.06% | 0.02% | 100% |  |
| Class D | 0.25% | 0.15% | 0.38% | 100% |  |
| Class F | 0.05% | -0.04% | 0.03% | 100% |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.08% | 0.17% | 100% |  |
| Class C | 0.22% | 0.45% | 0.35% | 100% |  |
| Class E | 0.09% | 0.17% | 0.50% | 100% |  |
| Overall | 0.14% | 0.16% | 0.31% | 100% |  |
| Class D | 0.42% | 0.48% | 0.46% | 100% |  |
| Class F (optional) | 0.19% | 0.02% | -0.17% | 100% |  |

Table 7: simulation results of test 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

# Conclusions

In this contribution, shorter tap filter is used for 4x4 sized partitions including 4x4 CU and sub-block of affine. For test 1, it reduces worst case memory bandwidth of 4x4 sized CU by 67% and complexity of affine to 65% regarding number of multiplications and 62% regarding number of additions. Experimental result shows 0.08% luma BD-rate change in RA and 0.14% luma BD-rate change in LD_B. Ignorable change in encoding and decoding time are observed.

It is suggested to adopt proposed method to the next VVC working draft.

# References

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).