| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0312 |
| --- | --- |

| Title: | CE4: MMVD improvement (test 4.4.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Ru-ling Liao Chong Soon Lim | Tel: Email: | +65-6550-5423 Jingya.li@sg.panasonic.com Chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to improve coding gain of MMVD by using adaptive distance table according to occurrence and resolution. Experimental results show -0.24% (RA) and 0.13% (LDB) luma BD-rate change for test3. 3% change in encoding timing time is observed. There is no much change in encoding and decoding time.

# Algorithm description

This proposal is based on JVET-L408. And adaptive distance table is proposed to improve coding gain of MMVD.

(test1) Use adaptive distance table based on occurrence-based distance table reordering. Distance index is reordered according to the occurrence of usage of each distance in previous coded picture in list 0 and reference index 0.

Details of test1 are as below:

- An optimal reordered distance table is determined after coding each inter picture. If current picture is intra picture, base distance table (shown in Table 1 and Table 2) is used as optimal reordered distance table.

- The optimal reordered distance table of reference picture in list 0 and reference index 0 is used as distance table for current inter picture.

- Weighted table is used for determining optimal reordered distance table. And weighted table is determined using gain table as shown in Table 3 and Table 4. Table 3 is used for picture with resolution non larger than 2K, otherwise Table 4 is used.

In Table 3 and Table 4, each row represents a distance ([1/4-32] in Table 3, [1-128] in Table 4; and each column represents a postion [0-7]

For example, if Table 3 is used,

If order is [1/4, 1/2, 1, 2, 4, 8, 16, 32], weighted table will be [0, 1, 2, 3, 4, 5, 6, 8]

If order is [1/2, 1/4, 1, 2, 4, 8, 16, 32], weighted table will be [2, -1, 3, 4, 5, 6, 8]

(test2) Use adaptive distance table based on picture resolution, i.e., if picture resolution is not larger than 2K, i.e., 1920×1080, Table 1 is used as base distance table; Otherwise, Table 2 is used as base distance table.

(test3) test1+test2.

Table 1: MMVD distance table candidate

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Table 2: MMVD distance table

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel | 64-pel | 128-pel |

Table 3: weighted table for distance table reordering

gain_table[8][8] = {

{ 0, -1, -2, -3, -4, -5, -6, -6},

{ 2,  1,  0, -1, -2, -3, -4, -4},

{ 4,  3,  2,  1,  0, -1, -2, -2},

{ 6,  5,  4,  3,  2,  1,  0,  0},

{ 8,  7,  6,  5,  4,  3,  2,  2},

{10,  9,  8,  7,  6,  5,  4,  4},

{12, 11, 10,  9,  8,  7,  6,  6},

{14, 13, 12, 11, 10,  9,  8,  8} };

Table 4: weighted table for distance table reordering

gain_table_4K[8][8] = {

{ 4,  3,  2,  1,  0, -1, -2, -2},

{ 6,  5,  4,  3,  2,  1,  0,  0},

{ 8,  7,  6,  5,  4,  3,  2,  2},

{10,  9,  8,  7,  6,  5,  4,  4},

{12, 11, 10,  9,  8,  7,  6,  6},

{14, 13, 12, 11, 10,  9,  8,  8},

{16, 15, 14, 13, 12, 11, 10, 10},

{18, 17, 16, 15, 14, 13, 12, 12} };

# Experimental Results

Results of test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.43% | -0.41% | 99% | 99% |
| Class A2 | -0.31% | -0.53% | -0.45% | 101% | 98% |
| Class B | -0.17% | -0.23% | -0.40% | 104% | 98% |
| Class C | -0.15% | -0.23% | -0.17% | 107% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.33% | -0.35% | 103% | 98% |
| Class D | -0.19% | -0.44% | -0.38% | 108% | 98% |
| Class F | -0.15% | -0.20% | -0.23% | 108% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.51% | -0.08% | 104% | 100% |
| Class C | -0.17% | 0.15% | -0.02% | 106% | 100% |
| Class E | -0.06% | 0.25% | -0.03% | 104% | 100% |
| Overall | -0.13% | -0.10% | -0.04% | 105% | 100% |
| Class D | -0.17% | -0.60% | -0.18% | 107% | 100% |
| Class F | -0.30% | -0.80% | -0.61% | 105% | 100% |

Results of test 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.46% | -0.51% | -0.74% | 99% | 98% |
| Class A2 | -0.29% | -0.82% | -0.69% | 101% | 97% |
| Class B | -0.10% | -0.12% | -0.26% | 104% | 97% |
| Class C | -0.11% | -0.12% | -0.14% | 107% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | -0.34% | -0.41% | 103% | 98% |
| Class D | -0.16% | -0.43% | -0.26% | 108% | 98% |
| Class F | -0.12% | -0.17% | -0.17% | 108% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.31% | 0.06% | 104% | 104% |
| Class C | -0.13% | -0.06% | -0.30% | 106% | 108% |
| Class E | -0.04% | 0.16% | -0.12% | 105% | 107% |
| Overall | -0.08% | -0.11% | -0.10% | 105% | 106% |
| Class D | -0.12% | -0.22% | -0.30% | 107% | 106% |
| Class F | -0.18% | -0.28% | -0.24% | 106% | 63% |

Results of test 3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.46% | -0.64% | -0.72% | 99% | 98% |
| Class A2 | -0.28% | -0.82% | -0.69% | 101% | 97% |
| Class B | -0.17% | -0.23% | -0.40% | 104% | 98% |
| Class C | -0.15% | -0.23% | -0.17% | 107% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.24% | -0.43% | -0.46% | 103% | 98% |
| Class D | -0.19% | -0.44% | -0.38% | 108% | 98% |
| Class F | -0.15% | -0.20% | -0.23% | 108% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.51% | -0.08% | 104% | 100% |
| Class C | -0.17% | 0.15% | -0.02% | 106% | 100% |
| Class E | -0.06% | 0.25% | -0.03% | 104% | 100% |
| Overall | -0.13% | -0.10% | -0.04% | 105% | 100% |
| Class D | -0.17% | -0.60% | -0.18% | 107% | 100% |
| Class F | -0.30% | -0.80% | -0.61% | 105% | 100% |

# Conclusions

In this contribution, adaptive distance table is proposed to improve coding gain of MMVD by using adaptive distance table according to occurrence and resolution. Experimental results show -0.24% (RA) and 0.13% (LDB) luma BD-rate change for combined test. There is no much change in encoding and decoding time.

We recommend to adopt the proposed solution into the next version of VVC.

# References

- M. W. Park, Y. Piao, M. Park, K. Choi, “CE4 Ultimate motion vector expression,” JVET-L0054, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed Working Draft Changes

### Updating process for distance_idx base table

Inputs to this process are:

- A variable W specifying the width of the current picture in luma samples,

- A variable H specifying the height of the current picture in luma samples,

- The array MMVDDistance1, MMVDDistance2.

Outputs of this process are:

- The array baseMMVDDistance.

The array MMVDDirection is derived as follows:

- If W is greater than 1920 and H is greater than 1080

Set baseMMVDDistance to MMVDDistance1 = {4, 8, 16, 32, 64, 128, 256, 512} (table2);

- Else,

Set baseMMVDDistance to MMVDDistance2 = {1, 2, 4, 8, 16, 32, 64, 128} (table1);

#### Updating process for distance_idx table

Inputs to this process are:

- The array CntMMVDDistance[dist_idx], wherein dist_dx = 0…7 and CntMMVDDistance[dist_idx] is the occurrence of distance of MMVDDistance[dist_idx]

- The array baseMMVDDistance [dist_idx].

Outputs of this process are:

- The array MMVDDistance[dist_idx].

The array MMVDDistance is updated as follows:

Calculating gain process for baseMMVDDistance is invoked and output is named as bestGain;

Set array MMVDDistanceBest same as baseMMVDDistance.

For each distance dist in baseMMVDDistance, the following ordered steps are repeated

For each index idx = 0…7, the following ordered steps are repeated

Set array MMVDDistanceTmp same as MMVDDistanceBest.

Find index for dist in MMVDDistanceTmp as idx2,

If idx > idx2

    - set MMVDDistanceTmp[dist_idx3] = MMVDDistanceTmp[dist_idx3+1] for dist_idx3 = idx2…idx-1.

    - set MMVDDistanceTmp[idx] = distance.

Else if idx < idx2

- set MMVDDistanceTmp[dist_idx3] = MMVDDistanceTmp[dist_idx3-1] for dist_idx3 = idx2…idx+1.

- set MMVDDistanceTmp[idx] = distance.

Calculating gain process for MMVDDistanceTmp is invoked and output is named as gain;

If(gain > bestGain)

- set bestGain as gain

- set MMVDDistanceBest as MMVDDistanceTmp

Set array MMVDDistance to MMVDDistanceBest.

#### Calculating gain process for MMVDDistanceX

Inputs to this process are:

- The array CntMMVDDistance[dist_idx], dist_idx =0 … 7.

- The array MMVDDistance[dist_idx].

- The array MMVDDistanceX[dist_idx].

Outputs of this process are:

- gain.

Gain is calculated as follows:

Set sum to 0;

For each distance dist in MMVDDistanceX[0…7],  the following ordered steps are repeated

- Find index of dist in array MMVDDistance as idx1;

- Find index of dist in array MMVDDistanceX as idx2;

- Find index of dist in array baseMMVDDistance as idx3;

- Add CntMMVDDistance[idx1] *gain_table[idx3][idx2] to sum

Set gain as sum;

General decoding process

Updating process for distance_idx base table is invoked before slice decoding

### Slice decoding process

This process is invoked when the current slice is a intra slice:

Set baseMMVDDistance to as current slice’s MMVDDistance

This process is invoked when the current slice is a P or B slice:

Set current slice’s MMVDDistance as MMVDDistance of reference picture idx 0 in reference list 0

After decoding slice

#### Updating process for distance_idx table is invoked