| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0327_v1 |
| --- | --- |

| Title: | CE8-related: A new CPR syntax scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurui Ye Fangdong Chen Li Wang | Email: | yeshurui@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This document describes a new CPR syntax scheme. The reported BD rate changes from the VTM3.0 with CPR enabled are as follows:

Scheme 1(Using I initial context values for CPR-P frame):

- In AI, -0.05%/-0.05%/-0.02% for CTC/Class F/SCC 1080p classes, separately.

Scheme 2(Remain the frame type as I frame if the original frame type is I):

- In AI, -0.05%/-0.08%/-0.05% for CTC/Class F/SCC 1080p classes, separately.

Scheme 3(Take CPR as an intra mode in IBP frame):

- In AI, -0.09%/-0.17%/-0.40% for CTC/Class F/SCC 1080p classes, separately.

Introduction

Current picture referencing (CPR) was adopted into VTM in last JVET meeting. In the VTM3.0, the current (partially) decoded picture is considered as a reference picture. This current picture is put in the last position of reference picture list 0. Therefore, for a frame using the current picture as the only reference picture, it is a P frame. The bitstream syntax follows the same syntax structure for inter coding while the decoding process is unified with inter coding. The only outstanding difference is that the block vector (which is the motion vector pointing to the current picture) always uses integer-pel resolution.

Proposed scheme

Using I initial context values for CPR-P frame

For a frame using the current picture as the only reference picture, it is denoted as CPR-P frame. For this frame, it is proposed to use I initial context values to initialize the syntax which it is already set for I frame. For new syntax, it will be initialized by P initial context values.

Remain the frame type as I frame if the original frame type is I

It is proposed to remain the frame type as I frame if the original fame type is I. In other words, for a frame using the current picture as the only reference picture, it's remain as I frame. Besides that, the syntax structure is the same as VTM3.0.

Take CPR as an intra mode in IBP frame

It is proposed to take CPR as an intra prediction mode and remain the frame type as I frame if the original frame type is I. Based on this, the following will apply:

- Using CU level CPR flag to indicate CPR block rather than with prediction direction and the POC of the reference frame. And when dual-tree is enabled, only luma block parsing this CPR flag. For chroma block, we enlarge the range of DM (direct mode) mode to include CPR mode.

- What’s more, in PB frame, the ref_idx binarization will not be influenced when CPR is enabled, which means current frame will not insert to reference picture list 0.

Simulation results

The proposed method is integrated on VTM3.0. All tests were conducted under the common test condition [1]. Results are shown in the following three tables:

Table 1. Using I initial context values for CPR-P frame

|  | Over VTM3.0 with CPR enabled (AI configuration) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | -0.03% | 100% | 99% |
| Class A2 | -0.01% | 0.02% | 0.02% | 100% | 101% |
| Class B | -0.03% | 0.11% | 0.02% | 100% | 101% |
| Class C | -0.08% | 0.20% | 0.01% | 100% | 100% |
| Class E | -0.12% | -0.07% | -0.09% | 100% | 102% |
| Overall | -0.05% | 0.08% | -0.01% | 100% | 100% |
| Class D | -0.16% | -0.01% | -0.40% | 100% | 99% |
| Class F | -0.05% | 0.03% | 0.04% | 100% | 101% |
| Class SCC | -0.02% | 0.01% | 0.02% | 100% | 104% |
|  | Over VTM3.0 with CPR enabled (RA configuration) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.07% | -0.08% | 100% | 101% |
| Class A2 | -0.01% | -0.04% | -0.04% | 100% | 102% |
| Class B | 0.00% | -0.05% | -0.10% | 100% | 99% |
| Class C | -0.03% | 0.05% | 0.08% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | -0.04% | 100% | 100% |
| Class D | -0.01% | 0.25% | 0.15% | 100% | 102% |
| Class F | 0.06% | 0.16% | 0.15% | 100% | 102% |
| Class SCC | -0.04% | -0.07% | -0.06% | 100% | 100% |
|  | Over VTM3.0 with CPR enabled (LDB configuration) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.25% | 0.21% | 101% | 105% |
| Class C | 0.01% | 0.10% | 0.09% | 101% | 104% |
| Class E | -0.20% | -0.28% | -0.13% | 100% | 105% |
| Overall | -0.05% | 0.07% | 0.08% | 100% | 104% |
| Class D | 0.02% | 0.25% | -0.79% | 100% | 100% |
| Class F | -0.06% | 0.18% | 0.20% | 101% | 103% |
| Class SCC | 0.13% | 0.10% | 0.07% | 100% | 110% |

Table 2. Remain the frame type as I frame if the original frame type is I

Over VTM3.0 with CPR enabled (AI configuration). Y; U; V; EncT; DecT. Class A1; -0.01%; 0.06%; 0.01%; 101%; 99%. Class A2; -0.01%; -0.04%; -0.03%; 101%; 99%. Class B; -0.04%; 0.09%; 0.09%; 101%; 102%. Class C; -0.07%; 0.05%; -0.11%; 101%; 99%. Class E; -0.12%; -0.10%; -0.17%; 100%; 100%. Overall; -0.05%; 0.02%; -0.03%; 101%; 100%. Class D; -0.16%; -0.06%; -0.43%; 101%; 103%. Class F; -0.08%; 0.03%; 0.04%; 101%; 101%. Class SCC; -0.05%; -0.01%; 0.02%; 102%; 101%. Over VTM3.0 with CPR enabled (RA configuration). Y; U; V; EncT; DecT. Class A1; -0.01%; 0.08%; -0.05%; 100%; 102%. Class A2; 0.01%; 0.00%; -0.08%; 101%; 101%. Class B; 0.00%; -0.03%; -0.08%; 100%; 99%. Class C; -0.02%; 0.02%; 0.11%; 100%; 100%. Class E. Overall; 0.00%; 0.01%; -0.02%; 100%; 100%. Class D; 0.04%; 0.25%; 0.15%; 100%; 102%. Class F; 0.10%; 0.18%; 0.36%; 100%; 101%. Class SCC; 0.01%; -0.01%; 0.00%; 100%; 99%. Over VTM3.0 with CPR enabled (LDB configuration). Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.01%; 0.18%; -0.05%; 100%; 101%. Class C; 0.01%; 0.05%; 0.19%; 100%; 104%. Class E; -0.19%; -0.17%; 0.80%; 99%; 102%. Overall; -0.04%; 0.05%; 0.24%; 100%; 102%. Class D; 0.03%; 0.12%; -0.52%; 100%; 100%. Class F; 0.20%; 0.26%; -0.19%; 100%; 101%. Class SCC; -0.08%; -0.05%; -0.13%; 100%; 106%.

Table 3. Take CPR as an intra mode in IBP frame

Over VTM3.0 with CPR enabled (AI configuration). Y; U; V; EncT; DecT. Class A1; -0.05%; 0.01%; -0.04%; 101%; 100%. Class A2; -0.07%; -0.08%; -0.05%; 101%; 98%. Class B; -0.06%; 0.01%; -0.08%; 101%; 99%. Class C; -0.12%; 0.11%; -0.11%; 101%; 103%. Class E; -0.15%; -0.01%; -0.25%; 101%; 101%. Overall; -0.09%; 0.01%; -0.10%; 101%; 100%. Class D; -0.20%; -0.19%; -0.66%; 103%; 101%. Class F; -0.17%; -0.20%; -0.05%; 101%; 101%. Class SCC; -0.40%; -0.37%; -0.39%; 101%; 99%. Over VTM3.0 with CPR enabled (RA configuration). Y; U; V; EncT; DecT. Class A1; 0.00%; 0.12%; -0.02%; 100%; 100%. Class A2; 0.02%; -0.10%; -0.11%; 100%; 98%. Class B; 0.03%; -0.05%; 0.10%; 101%; 99%. Class C; 0.00%; -0.07%; -0.24%; 100%; 101%. Class E. Overall; 0.01%; -0.03%; -0.06%; 100%; 100%. Class D; 0.02%; -0.10%; -0.25%; 100%; 102%. Class F; 0.02%; 0.10%; 0.03%; 100%; 101%. Class SCC; 0.13%; 0.11%; 0.15%; 100%; 98%. Over VTM3.0 with CPR enabled (LDB configuration). Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -0.03%; 0.19%; 0.03%; 100%; 96%. Class C; -0.03%; 0.16%; -0.03%; 100%; 98%. Class E; -0.06%; -0.92%; -0.21%; 99%; 98%. Overall; -0.04%; -0.10%; -0.05%; 100%; 97%. Class D; -0.02%; 0.33%; -0.47%; 100%; 101%. Class F; 0.01%; 0.30%; 0.74%; 100%; 98%. Class SCC; -0.06%; -0.02%; -0.07%; 99%; 96%.

Conclusion

In this contribution, a new syntax scheme of CPR is proposed. Each stage of modification is tested. It is reported that by the new CPR syntax scheme, in AI configuration, it brings -0.09%/-0.17%/-0.40% for CTC/Class F/SCC 1080p classes, separately.

- Reference

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Hikvision may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).