| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0328_v4 |
| --- | --- |

| Title: | CE10-related: Simplified Triangle Prediction Unit Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

Although the triangle prediction unit mode has been adopted in the VTM 3.0, it still has some redundant processes. In this contribution, several redundancies are proposed to be removed. First, the candidate list construction is simplified from two aspects: the redundant pruning for spatial candidates is removed, and some unnecessary candidates are also removed. Second, the two weight groups during the motion compensation are reduced to one group. Third, the redundant triangle prediction flag is not signaled when MMVD or MHIntra mode is enabled.

It is reported that the first simplification on the candidate list construction only brings 0.00% and 0.02% loss for RA and LB compared to VTM-3.0, and the second simplification on the candidate list construction only brings 0.00% and 0.03% loss for RA and LB compared to VTM-3.0. And when only keeping one weight group, it only brings 0.01% and 0.03% loss for RA and LB compared to VTM-3.0. Moreover, when the redundant triangle prediction flag is not signaled, it can provide 0.07% and 0.14% gains for RA and LB compared to VTM-3.0.

Introduction

In [1], the triangular prediction unit mode is proposed, and it is adopted in VTM 3.0. In the triangular prediction unit mode, a CU can be split using two triangular prediction units, in either diagonal or inverse diagonal direction. Each triangular prediction unit in the CU has its own uni-prediction motion vector and reference frame index which are derived from a uni-prediction candidate list. Triangular partitioning is only applied to motion compensated prediction, which means that the transform and quantization process is applied to the whole CU formed by combining the two triangles together. The triangular prediction unit mode is only applied to a CU which block size is larger than or equal to 8×8, and its frame type must be B frame.

The uni-prediction candidate list consists of five uni-prediction motion vector candidates. It is derived from seven neighboring blocks including five spatial neighboring blocks and two temporal co-located blocks. The motion vectors of the seven neighboring blocks are collected and put into the uni-prediction candidate list according in the order of uni-prediction motion vectors, L0 motion vector of bi-prediction motion vectors, L1 motion vector of bi-prediction motion vectors, and averaged motion vector of the L0 and L1 motion vectors of bi-prediction motion vectors. If the number of candidates is less than five, zero motion vector is added to the list.

After predicting each triangular prediction unit, an adaptive weighting process is applied to the diagonal edge between the two triangular prediction units to derive the final prediction for the whole CU. Two weighting factor groups are listed as follows:

- 1st weighting factor group: {7/8, 6/8, 4/8, 2/8, 1/8} and {7/8, 4/8, 1/8} are used for the luminance and the chrominance samples, respectively;

- 2nd weighting factor group: {7/8, 6/8, 5/8, 4/8, 3/8, 2/8, 1/8} and {6/8, 4/8, 2/8} are used for the luminance and the chrominance samples, respectively.

One weighting factor group is selected based on the comparison of the motion vectors of two triangular prediction units. The 2nd weighting factor group is used when the reference pictures of the two triangular prediction units are different from each other or their motion vector difference is larger than 16 pixels. Otherwise, the 1st weighting factor group is used.

Proposed Simplification

Remove redundant pruning

In the list construction, there are two pruning processes. One happens when collecting the five spatial candidates, the other happens when putting the motion information into the uni-prediction candidate list. The first one may have 1+2+3+4=10 times of pruning at most, and it seems to be redundant. Hence, it is proposed to remove the first redundant pruning process. The related test is Test A.

Remove redundant candidates

In the list construction, the motion vectors of the seven neighboring blocks are collected and put into the uni-prediction candidate list according in the order of uni-prediction motion vectors, L0 motion vector of bi-prediction motion vectors, L1 motion vector of bi-prediction motion vectors, and averaged motion vector of the L0 and L1 motion vectors of bi-prediction motion vectors. The averaged motion vector of the L0 and L1 motion vectors of bi-prediction motion vectors seems to be redundant, and it is proposed to remove it. The related test is Test B.

Remove redundant weighting group

During the motion compensation, there are two weighting factor groups, which seem to be redundant. It is proposed to keep only one weighting factor group. Here, we have two tests:

Test C: Only keep the 1st weighting factor group.

Test D: Only keep the 2nd weighting factor group.

For Test D, it has two benefits:

- The comparison of the motion information of two parts to decide weight group set index can be skipped, thus the related content in the draft can be removed:

The weighting set variable wS is derived as follows:

- The variables refPicLA and refPicLB are derived as follows:

refPicLA = ( predFlagLA = = 0 )  ?  RefPicList0[ refIdxLA ]  :  RefPicList1[ refIdxLA ]	(8-745)

refPicLB = ( predFlagLB = = 0 )  ?  RefPicList0[ refIdxLB ]  :  RefPicList1[ refIdxLB ] 	(8-746)

- If DiffPicOrderCnt( refPicLA, refPicLB ) is not equal to 0, wS is set equal to 1.

- Otherwise ( DiffPicOrderCnt( refPicLA, refPicLB ) is equal to 0),

wS = ( Abs( mvLA[ 0 ] − mvLB[ 0 ] ) > 256  | |  	Abs( mvLA[ 1 ] − mvLB[ 1 ] ) > 256 )  ?  1  :  0	(8-747)

- Since in the second weight group, the weight values are just the same as weight index, if there is only the second weight group, there is no need to keep such a nearly 100 Byte table, which can also help to save memory. The related table can be also removed:

–	The variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

wValue = ( cIdx < 1 ) ? ( Clip3( 0, 8, wIdx ) : ( Clip3( 0, 8, wIdx*2 ) )	(8-750)

Table 8-14 – Specification of wValue using cIdx, wS and wIdx

| cIdx | wS | wIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 0 | 0 | 0 | 1 | 2 | 4 | 6 | 7 | 8 | 8 |
|  | 1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1, 2 | 0 | 0 | 1 | 4 | 7 | 8 | 0 | 0 | 0 | 0 |
|  | 1 | 0 | 2 | 4 | 6 | 8 | 0 | 0 | 0 | 0 |

Remove redundant flag signalling

In VTM 3.0, when MMVD or MHIntra mode is enabled, the triangular prediction unit mode must be disabled. However, its flag is still signaled in this case. Hence, the triangular prediction unit flag is proposed to be not signaled when MMVD or MHIntra mode is enabled. The related test is Test E.

The related syntax change is given as follows.

| … |  |
| --- | --- |
| if( sps_triangle_enabled_flag && tile_group_type = = B && cbWidth * cbHeight >= 64 && ciip_flag[ x0 ][ y0 ] = = 0 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| … |  |

It is noted that in the latest draft the signaling of the merge_triangle_flag is already excluded when MMVD mode is enabled.

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-3.0 is served as the experimental platform. The CTC specified in [2], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

Results of removing redundant pruning

The results of Test A are given in Table I. It is reported that removing redundant pruning on the candidate list construction only brings 0.00% and 0.02% loss for RA and LB compared to VTM-3.0, respectively.

Table I Results of Test A

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.00% | 100% | 101% |
| Class A2 | -0.01% | 0.00% | 0.03% | 100% | 99% |
| Class B | 0.00% | -0.02% | -0.04% | 100% | 101% |
| Class C | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class D | 0.00% | -0.04% | 0.03% | 100% | 100% |
| Class F | 0.00% | 0.01% | 0.02% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.21% | 0.05% | 100% | 96% |
| Class C | 0.01% | -0.01% | 0.03% | 101% | 108% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 106% |
| Overall | 0.02% | -0.09% | 0.03% | 100% | 102% |
| Class D | -0.01% | 0.02% | -0.07% | 101% | 100% |
| Class F | -0.04% | -0.17% | -0.25% | 101% | 106% |

Results of removing redundant candidates

The results of Test B are given in Table II. It is reported that removing redundant candidates of the candidate list only brings 0.00% and 0.03% loss for RA and LB compared to VTM-3.0, respectively.

Table II Results of Test B

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | 0.03% | 100% | 102% |
| Class A2 | -0.01% | -0.02% | 0.03% | 100% | 103% |
| Class B | 0.01% | 0.07% | -0.03% | 100% | 101% |
| Class C | 0.00% | 0.05% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.00% | 100% | 101% |
| Class D | 0.04% | -0.04% | -0.01% | 100% | 99% |
| Class F | -0.01% | 0.00% | 0.00% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.19% | 0.07% | 101% | 98% |
| Class C | 0.02% | 0.15% | 0.37% | 101% | 102% |
| Class E | 0.07% | 0.32% | -0.05% | 100% | 101% |
| Overall | 0.03% | 0.05% | 0.14% | 101% | 100% |
| Class D | 0.05% | 0.32% | 0.01% | 100% | 96% |
| Class F | -0.01% | -0.02% | 0.36% | 100% | 102% |

The results of the combination of Test A and Test B are given in Table III. It is reported that these two simplifications on candidate list construction only bring 0.00% and 0.03% loss for RA and LB compared to VTM-3.0, respectively.

Table III Results of the combination of Test A and Test B

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.09% | 100% | 101% |
| Class A2 | 0.00% | 0.03% | 0.05% | 100% | 101% |
| Class B | 0.00% | 0.02% | -0.03% | 100% | 100% |
| Class C | 0.00% | 0.06% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.04% | 100% | 100% |
| Class D | 0.03% | -0.02% | -0.01% | 100% | 98% |
| Class F | 0.00% | 0.00% | -0.01% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.32% | 0.03% | 101% | 95% |
| Class C | 0.03% | 0.28% | 0.31% | 101% | 104% |
| Class E | 0.06% | 0.49% | 0.29% | 101% | 99% |
| Overall | 0.03% | 0.09% | 0.19% | 101% | 99% |
| Class D | 0.00% | 0.23% | 0.03% | 100% | 95% |
| Class F | -0.03% | 0.43% | -0.38% | 100% | 98% |

Results of removing redundant weighting group

The results of Test C and Test D are given in Table IV and Table V, respectively. It is reported that only keeping the 1st weighting factor group brings 0.02% and 0.03% loss for RA and LB compared to VTM-3.0, respectively. And keeping the 2nd weighting factor group only brings 0.01% and 0.03% loss for RA and LB compared to VTM-3.0, respectively. Based on the results, it is suggested to only keep the 2nd weighting factor group.

Table IV Results of Test C

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.12% | 0.04% | 100% | 101% |
| Class A2 | 0.00% | 0.10% | 0.08% | 101% | 103% |
| Class B | 0.01% | 0.12% | 0.06% | 99% | 100% |
| Class C | 0.04% | 0.01% | 0.06% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.09% | 0.06% | 100% | 100% |
| Class D | 0.02% | -0.10% | -0.06% | 99% | 99% |
| Class F | 0.02% | 0.01% | 0.03% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.21% | 0.09% | 100% | 96% |
| Class C | 0.03% | 0.26% | 0.16% | 101% | 102% |
| Class E | 0.03% | -0.13% | -0.51% | 100% | 98% |
| Overall | 0.03% | -0.03% | -0.04% | 100% | 99% |
| Class D | -0.01% | -0.15% | 0.38% | 100% | 92% |
| Class F | 0.10% | 0.20% | -0.22% | 101% | 101% |

Table V Results of Test D

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.06% | -0.02% | 100% | 102% |
| Class A2 | 0.01% | 0.00% | 0.06% | 100% | 103% |
| Class B | 0.00% | 0.10% | 0.03% | 100% | 100% |
| Class C | 0.04% | -0.02% | 0.10% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.05% | 100% | 101% |
| Class D | -0.01% | 0.14% | 0.01% | 100% | 98% |
| Class F | 0.01% | -0.02% | 0.02% | 101% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.13% | 0.01% | 101% | 98% |
| Class C | 0.06% | 0.10% | 0.23% | 101% | 101% |
| Class E | 0.01% | 0.06% | -0.37% | 101% | 100% |
| Overall | 0.03% | -0.01% | -0.01% | 101% | 100% |
| Class D | 0.11% | 0.25% | 0.02% | 100% | 96% |
| Class F | 0.11% | -0.04% | 0.38% | 101% | 100% |

Results of removing redundant flag signalling

The results of Test E are given in Table VI. It is reported that removing redundant flag signalling brings 0.07% and 0.14% gains for RA and LB compared to VTM-3.0, respectively.Table VI Results of Test E

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.08% | -0.09% | 100% | 103% |
| Class A2 | -0.10% | -0.09% | -0.02% | 99% | 97% |
| Class B | -0.07% | -0.02% | -0.18% | 99% | 95% |
| Class C | -0.07% | -0.06% | -0.09% | 99% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.05% | -0.11% | 99% | 97% |
| Class D | -0.10% | -0.20% | -0.07% | 99% | 104% |
| Class F | -0.03% | -0.01% | -0.03% | 100% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.32% | 0.00% | 100% | 98% |
| Class C | -0.19% | 0.03% | -0.14% | 100% | 96% |
| Class E | -0.13% | 0.03% | -0.17% | 99% | 101% |
| Overall | -0.14% | -0.12% | -0.09% | 100% | 98% |
| Class D | -0.17% | 0.13% | 0.13% | 99% | 98% |
| Class F | 0.00% | -0.14% | -0.54% | 99% | 99% |

Conclusion

In this proposal, we propose several simplified methods for triangle prediction unit mode. These simplified methods bring negligible coding loss, and one even provides some coding gains. Based on the above results, it is suggested to adopt them into the VTM.

Reference

- R.-L. Liao, C. S. Lim, “CE10.3.1.b: Triangular prediction unit mode,” JVET-L0124, Macao, CN, 3–12 Oct. 2018.

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).