| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0336 |
| --- | --- |

| Title: | Non-CE11: Considering boundary strength on CPR coded block boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Seung-Hwan Kim Jayhyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This proposal presents boundary strength design on CPR coded block boundary. Specifically, following two methods for boundary strength design are suggested.

- Boundary strength is set to 0 when at least one of block is coded as CPR across the block boundary.

- Boundary strength is set to 0 when both block are coded as CPR and set to 1 when only one block is coded as CPR across the block boundary

The proposed method has been shown to provide visual quality improvement on computer generated video sequence where CPR mode occurs frequently.

Problem Statement

12th JVET meeting CPR is adopted in VTM. The characteristics on this block boundary is different from block boundary of conventional intra and inter coded block. When the block boundary strength on the CPR coded block is decided based on conventional BS decision process which is designed based on natural video characteristics, the edge could be over blurred.

Proposed method

- Boundary strength is set as 0 when at least one of block is coded as CPR across the block boundary.

- Boundary strength is set as 0 when both block are coded as CPR and is set as 1 when only one block is coded as CPR across the block boundary

Experimental result

Subjective quality

Anchor result shows excessive blurring around block boundary which is captured in Figure 1. In contrast, the proposed method shows subjective quality improvement around object boundary as shown in Figure 2.

Figure 1. Anchor FlyingGraphics QP37 1st frame

Figure 2. Proposed FlyingGraphics QP37 1st frame

Figure 3. Anchor console QP37 1st frame

Figure 4. Proposed console QP37 1st frame

Objective quality

Experimental result under common test condition are shown as following. Noticeable BD-rate change is not observed.

Method 1

Boundary strength is set to 0 when at least one of block is coded as CPR across the block boundary.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR-off |  |  |  |  | Over VTM-3.0-CPR-on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.10% | 0.06% | 126% | 100% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | -0.47% | -0.50% | -0.54% | 128% | 100% | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class B | -0.29% | -0.39% | -0.36% | 135% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.08% | -0.24% | -0.25% | 139% | 101% | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class E | -0.61% | -0.81% | -0.60% | 139% | 100% | 0.03% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.27% | -0.40% | -0.33% | 134% | 100% | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.10% | -0.23% | 0.22% | 136% | 101% | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class F (optional) | -12.05% | -12.04% | -12.09% | 154% | 98% | 0.05% | 0.00% | 0.00% | 100% | 99% |
| Class SCC 1080p | -36.89% | -36.11% | -36.36% | 109% | 80% | 0.03% | 0.00% | 0.00% | 100% | 99% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR-off |  |  |  |  | Over VTM-3.0-CPR-on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -0.01% | 0.24% | 99% | 100% | -0.01% | 0.03% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.12% | 0.22% | 97% | 100% | 0.01% | 0.05% | -0.02% | 100% | 100% |
| Class B | 0.02% | 0.00% | -0.24% | 99% | 100% | 0.00% | 0.00% | -0.04% | 100% | 100% |
| Class C | 0.19% | -0.08% | 0.16% | 102% | 100% | 0.00% | -0.02% | 0.08% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.08% | 0.00% | 0.05% | 99% | 100% | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.21% | -0.33% | 0.12% | 102% | 99% | 0.06% | -0.05% | -0.04% | 100% | 100% |
| Class F (optional) | -9.87% | -10.00% | -10.00% | 107% | 99% | 0.02% | -0.09% | -0.05% | 100% | 99% |
| Class SCC 1080p | -22.02% | -21.69% | -22.27% | 103% | 95% | 0.00% | -0.14% | -0.19% | 100% | 99% |

Method 2

Boundary strength is set as 0 when both block are coded as CPR and is set as 1 when only one block is coded as CPR across the block boundary

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR-off |  |  |  |  | Over VTM-3.0-CPR-on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.10% | 0.06% | 126% | 100% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | -0.47% | -0.50% | -0.54% | 128% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | -0.29% | -0.39% | -0.35% | 135% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.09% | -0.25% | -0.25% | 139% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | -0.63% | -0.81% | -0.59% | 139% | 101% | 0.01% | 0.00% | 0.00% | 99% | 100% |
| Overall | -0.27% | -0.40% | -0.33% | 134% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.10% | -0.23% | 0.21% | 136% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | -12.08% | -12.04% | -12.10% | 154% | 97% | 0.01% | 0.00% | 0.00% | 100% | 99% |
| Class SCC 1080p | -36.91% | -36.11% | -36.36% | 109% | 80% | 0.01% | 0.00% | 0.00% | 100% | 99% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR-off |  |  |  |  | Over VTM-3.0-CPR-on |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | -0.03% | 0.23% | 99% | 100% | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class A2 | 0.01% | 0.03% | 0.22% | 97% | 100% | 0.01% | -0.04% | -0.02% | 100% | 100% |
| Class B | 0.02% | -0.05% | -0.24% | 98% | 99% | 0.00% | -0.05% | -0.03% | 100% | 100% |
| Class C | 0.17% | -0.02% | 0.19% | 102% | 99% | -0.02% | 0.04% | 0.10% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.08% | -0.02% | 0.06% | 99% | 99% | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.20% | -0.16% | 0.22% | 102% | 99% | 0.05% | 0.12% | 0.06% | 100% | 100% |
| Class F (optional) | -9.90% | -9.90% | -9.98% | 107% | 99% | -0.01% | 0.02% | -0.03% | 100% | 99% |
| Class SCC 1080p | -22.08% | -21.77% | -22.32% | 103% | 96% | -0.06% | -0.22% | -0.25% | 100% | 99% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).