| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0340_v4 |
| --- | --- |

| Title: | CE6-related: Simplification on MTS for intra residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoqiang Cao Fangdong Chen Li Wang | Tel: Email: | caoxiaoqiang@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution presents two simplified MTS methods for intra residual coding in VTM3.0. Three transform pairs are considered at most when the MTS_CU_FLAG is equal to 1. The first simplification was based on block shape, it is reported that the BD-rate are 0.07%, 0.02%, 0.06% for AI, RA, and LB respectively with 21%, 3% and 1% encoding time reduced. The second simplification was based on both block shape and intra mode, it is reported that the BD-Rate are 0.00%, -0.02%, 0.02% for AI, RA, and LB respectively with 21%, 3% and 1% encoding time reduced.

Introduction

The current MTS design in VTM3[1] is show as the Table 1, five transform pairs are going to be checked and selected the best one for the eventually transform pair. MTS is a tool providing interesting gain, but the complexity of encoding time which seems very high even though MTS has a fast algorithm in the encoder.

Table 1 Transform and signalling mapping table

| MTS_CU_flag | Transform pair(H,V) | Transform pair index | Codeword |
| --- | --- | --- | --- |
| 0 | (DCT2, DCT2) | - | - |
| 1 | (DST7,DST7) | 0 | 00 |
|  | (DCT8,DST7) | 1 | 10 |
|  | (DST7,DCT8) | 2 | 01 |
|  | (DCT8,DCT8) | 3 | 11 |

Proposed Method

In this contribution, the proposed methods introduced two simplified version of MTS for intra coding, 3 transform pairs are considered at most when the MTS_CU_FLAG is equal to 1. The first version of simplified MTS is based on block shape, and the second version of simplified MTS is based on both intra mode and block shape. Both two version of simplification on MTS reduced encoding time with the number of transform pairs to be signalled reduced.

simplified MTS based on block shape

Table 2 shows the first version of the proposed simplified MTS method for intra residual coding. 3 transform pairs are considered at most when the MTS_CU_FLAG is equal to 1. When the TU’s width is equal to the TU’s height, 3 transform pairs are to be checked, otherwise, only 2 transform pairs are to be checked and the shorter side of TU will be fixed with DST7. compared with the current MTS, the number of transform pairs are reduced from 5 to 4 when width is equal to height, and the number of transform pairs are reduced from 5 to 3 when width is not equal to height.

Table 2 Transform pair and codeword of the first proposed simplified MTS method

| MTS_CU_flag | TU shape | Transform pair(H,V) | Codeword |
| --- | --- | --- | --- |
| 0 | - | (DCT2, DCT2) | - |
| 1 | W == H | Index 0: (DST7, DST7) | 0 |
|  |  | Index 1: (DCT8, DST7) | 10 |
|  |  | Index 2: (DST7, DCT8) | 11 |
|  | W > H | Index 0/1 | 0/1 |
|  | W < H | Index 0/2 | 0/1 |

simplified MTS based on block shape and intra mode

Table 3 Transform pair and codeword of the second proposed simplified MTS method

| MTS_CU_flag | TU shape | Intra mode | Transform pair(H,V) | Codeword |
| --- | --- | --- | --- | --- |
| 0 | - | - | (DCT2, DCT2) | - |
| 1 | - | 0 ~ 1 | Set0: Index 0/1/2 | 0/10/11 |
|  | W == H | 2 ~ 34 | Set1: Index 0/2 | 0/1 |
|  |  | 35 ~ 66 | Set2: Index 0/1 |  |
|  | W > H | (B+1) ~ 34 | Set1 |  |
|  |  | 2 ~ B, 35 ~ 66 | Set2 |  |
|  | W < H | 2 ~ 34, B ~ 66 | Set1 |  |
|  |  | 35 ~ (B-1) | Set2 |  |

Table 3 shows the second version of the proposed simplified MTS method for intra residual coding. 3 transform pairs are considered at most when the MTS_CU_FLAG is equal to 1. If current block’s intra mode is DC or Planar, 3 transform pairs are to be checked with set0, otherwise, only 2 transform pairs are to be checked with set1 or set2. Figure 1 shows the transform sets for the proposed simplification when intra mode is range from 2 to 66. Due to the wide angle intra prediction, if current block’s intra mode is not DC and Planar mode, the selection of 2 transform pairs is not only determined with the ratio of width and height, but also determined with intra angular mode, when width is greater than height, wide angle mode are from 2 to B; when width is less than height, wide angle mode are from B to 66. B is defined by the ratio of width and height (r = W/H). For wide angle intra prediction, when r is 2, 4, 8 and 16, mode B is 7, 11, 13 and 15 respectively. When r is 1/2, 1/4, 1/8 and 1/16, mode B is 61, 57, 55 and 53 respectively.

Figure 1 transform sets for angular intra mode based on shape

Experimental Results

The proposed methods has been implemented based on VTM-3.0 reference software, and simulations are performed using VTM configurations under common test condition defined in JVET-L1010[2] and Core Experiment description in JVET-L1026[3].

Results for simplified MTS based on block shape

Results for simplified MTS based on block shape and intra mode

Conclusion

This contribution presents two simplified MTS method for intra residual coding. The first simplified version is based on the shape of current TU, 3 transform pairs are considered to check and signal at most when the MTS_CU_FLAG is equal to 1. The second simplified version is based on both intra mode and the ratio of width and height, 3 transform pairs are considered to check and signal at most when the MTS_CU_FLAG is equal to 1. Both two version reduced encoding time with neglectable loss. It is recommended of the inclusion of the proposed transform versions in the next reference software.

Reference

[1] Jianle Chen, Yan Ye, Seung Hwan Kim, “Algorithm description for Versatile Video Coding and Test Model 3”, JVET-L1002, Oct. 2018.

[2] Frank Bossen, Jill Boyce, Karsten Suehring, et al, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Oct. 2018.

[3] Amir Said, Xin Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, JVET-L1026, JVET-L1026, Oct. 2018.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).