| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0343 |
| --- | --- |

| Title: | Non-CE2: Simplified subblock motion derivation for SbTMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Seung-Hwan Kim Jayhyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This proposal presents a simplified subblock motion vector derivation process for SbTMVP in VTM-3.0. In order to reduce worst case scaling operations, this contribution suggests motion vector of the subblock set to default motion information when subblock after motion shifting point outside of motion fetch area of current CTU. Proposed method reduces worst case 256 times scaling operation of SbTMVP without noticeable BD-rate change. The experimental results show 0.00%, 0.00% BD-rate change for RA and LDB, respectively.

Problem Statement

At 11-th JVET meeting, JVET-K0346[1] was proposed and adopted. JVET-K0346 suggests temporal motion fetch area where same as conventional TMVP should be defined as shown as Figure 1.

Figure 1. Temporal motion fetch area for TMVP and SbTMVP

As depicted in Figure 2, Subblock is clipped when subblock is located in outside of this area after motion shifting from this suggestion.

Figure 2. Case of subblock clipping of SbTMVP

Motion information for subblock is derived based on scaling operation even though motion information might be not correct because the subblock is clipped inside fetch area base on subblock motion vector derivation process of SbTMVP in Figure 3. In worst case, a number of 256 subblock of CTU is clipped and derived motion vector by scaling operation.

Figure 3. subblock motion derivation process of current SbTMVP

Proposed method

This contribution suggests motion vector of the subblock set to default motion information when subblock after motion shifting require to fetch motion information outside motion fetch area to reduce worst case scaling operation. The proposed method is depicted in Figure 4.

Figure 3. Proposed subblock motion derivation process for SbTMVP

Experimental result

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.02% | -0.04% | 100% | 100% |
| Class C | 0.02% | -0.01% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class F | 0.00% | -0.01% | 0.01% | 100% | 100% |

Reference

- X. Xiu, Y. Hem Y. Ye, H. Jang, J. Nam, S. Kim, J. Lim, H. Chen, H. Yang, J. Chen, “CE4-related: one simplified design of advanced temporal motion vector prediction(ATMVP),” JVET-K0346, Ljubljana, SI, July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).