| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0345 |
| --- | --- |

| Title: | CE4-related: Remove redundancy between TMVP and ATMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Suhong Wang1, Xiaozhen Zheng2, Jing Cui1, Shanshe Wang1, Siwei Ma1 Peking University1, DJI2 | Email: | suhong.wang@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

At the 11th JVET meeting, the granularity of temporal MV compression is set to 8x8 and at the 12th JVET meeting, sub-block size of ATMVP have been fixed to 8x8. In specific cases, there are some redundancy operations between TMVP and ATMVP. Besides, TMVP with smaller block size becomes unnecessary in some cases due to the storage granularity. In order to remove the redundant process of deriving TMVP in merge mode, this contribution reports the experimental results on two cases as follows:

- Test(a): Skip TMVP process in merge mode when CU size is equal to 8x8

- Test(b): Skip TMVP process in merge mode when the width or height of CU is 4 + Test(a)

Simulation results for Test (a), Test (b) show minor coding gain compared to VTM-3.0 and observe minor encoding and decoding time decrease.

Introduction

In merge mode, TMVP derives one temporal candidate MVP from two temporal co-located blocks, which are the blocks locate at the bottom right and at the center of the current block, following a specified order. With the granularity of temporal MV storage has been set to 8x8 and the adoption of 8x8 ATMVP, it exists some redundancy operations in TMVP deriving process:

- In affine merge mode, ATMVP derives multiple motion for 8x8 sub-blocks of one CU based on the motion information of the collocated blocks from temporal reference picture. Both ATMVP and TMVP derive motion information from temporal domain. When the size of current CU is 8x8, both of these two tools will derive one MVP for current CU. In principal, the motion vectors derivation process for these two tools have redundancy at 8x8 coding block.

- Since the granularity of temporal MV storage has been set to 8x8, TMVP with smaller block size might become unnecessary in some cases, e.g. the width or height of CU is 4, when some adjacent CUs derive MVP from the same MV storage units.

Proposed Method

In order to remove the redundant process of deriving TMVP in merge mode, two methods have been proposed to the construction of current merge candidate list.

Skip TMVP process in merge mode when CU size is equal to 8x8

In current design, sub-block size of ATMVP is fixed to 8x8. For 8x8 CUs, only one sub-block is consist in the CU during ATMVP process. In this case, only one MVP will be derived for the CU. The derivation of MV when CU is 8x8 can be described as followed:

- Form a MVP list by adding the motion information of spatial neighboring CUs in 4 certain positions;

- Check whether the reference picture of the first candidate(if exists) in the above list is same as the co-located picture of current CU, if so, take this MV as temporal MV to derive co-located CU in temporal reference picture; otherwise, take (0,0) as temporal MV;

- Set the MV of the co-located CU (if exists) as the MV of the 8x8 CU.

Compared with TMVP, which derives MVP from two certain positions, the above ATMVP process for 8x8 CUs is more reasonable and flexible, leading to higher prediction accuracy. Therefore, the proposed method suggests skip TMVP when CU size is equal to 8x8. The construction process for merge candidate list with proposed method is shown in Fig.1.

- (b)

Figure. 1 Construction process for merge list (a) current design (b) proposed method

Skip TMVP process in merge mode when the width or height of CU is 4

Since the granularity of temporal MV storage has been set to 8x8, TMVP process might be redundant in some cases. As shown in Fig.2, when the width or height of current CU is smaller than 8(equal to 4), the MV of current CU derived from temporal domain can be the same as its adjacent CUs, which will make these kind of partition meaningless. Therefore, in this contribution, it is proposed to skip TMVP in merge mode when the width or height of CU is 4.

Figure. 2 Illustration of unnecessary cases for TMVP

Experimental Results

The experiments were performed under JVET common test conditions.

BD-Rate results of the two tests on top of the VTM-3.0 compared to the anchor with RA and LDB configurations are shown in following tables. The full results are provided in the attached excel sheets.

Coding performance and complexity of Test (a) (Skip TMVP process in merge mode when CU size is equal to 8x8)

Coding performance and complexity of Test (b) (Skip TMVP process in merge mode when the width or height of CU is 4) + Test (a)

Conclusion

This contribution proposed redundant removal for TMVP in merge mode, which skips TMVP process in merge mode when CU size is equal to 8x8 and when the width or height of CU is 4.

Simulation results for all tests shows minor coding gain on average compared to VTM 3.0 anchor, while minor encoding and decoding time decrease are observed. Therefore, it is recommended to adopt the proposed redundant removal for TMVP method to the next version of VVC working draft.

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).