| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0352-v3 |
| --- | --- |

| Title: | CE10-related: Simplification of triangular partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dohyeon Park Yong-Uk Yoon Jae-Gon Kim Jinho Lee Jungwon Kang | Tel: Email: | +82-2-300-0414 dhpark@kau.kr yuyoon@kau.kr jgkim@kau.ac.kr jinosoul@etri.re.kr jungwon@etri.re.kr |
| Source: | Korea Aerospace University (KAU), ETRI |  |  |

_____________________________

- Abstract

This contribution presents simplifications of triangular partitions based on VTM-3.0. The first simplification is to remove the reference picture mapping process in the motion vector storage of triangular partitions. The second one is to reduce the number of combinations of motion information for each partition along with modified binarization methods for the reduced combination indications. In this contribution, the performances on the 3 tests as shown follows are reported

- Test 1: Simplification of motion vector storage skipping reference picture mapping process

  - (RA) 0.00% BD-rate with 100% encoding time and 100% decoding time

  - (LDB) 0.17% BD-rate with 100% encoding time and 100% decoding time

- Test 2: Reduction of the number of motion information combinations

  - (RA) 0.01% BD-rate with 96% encoding time and 100% decoding time

  - (LDB) 0.03% BD-rate with 94% encoding time and 100% decoding time

- Test 3: Test1 + Test2

  - (RA) 0.02% BD-rate with 96% encoding time and 100% decoding time

  - (LDB) 0.20% BD-rate with 94% encoding time and 100% decoding time

Introduction

At the last meeting, the triangular partitions mode has been adopted in VTM-3.0 software [1], [2]. A CU can be split into two partitions with diagonal or inverse diagonal direction. Each triangular partition in the CU is inter-predicted using its own uni-predicted motion information which is derived from an uni-prediction candidate list constructed by motion information of neighboring positions [3].

The motion vectors (Mv1 and Mv2 shown in Figure 1) of the triangular partitions are stored in a 4×4 grid. For each 4×4 grid, either uni-prediction or bi-prediction motion vector is stored depending on the position of the 4×4 grid in the CU. At the 4x4 grid on the boundary areas of partitions in which both predictions obtained by each partition’s motion vector are combined as a final prediction, the motion vectors used for inter-prediction are stored as a bi-prediction motion vector. The bi-prediction motion vector to be stored is derived from Mv1 and Mv2 according to the following rules of reference picture mapping process:

- In the case that Mv1 and Mv2 have motion vector from different directions (L0 or L1), Mv1 and Mv2 are simply combined to form a bi-prediction motion vector.

- In the case that both Mv1 and Mv2 are from the same L0 (or L1) direction,

  - If the reference picture of Mv2 is the same as a picture in the L1 (or L0) reference picture list, Mv2 is scaled to the picture. Mv1 and the scaled Mv2 are combined to form a bi-prediction motion vector.

  - If the reference picture of Mv1 is the same as a picture in the L1 (or L0) reference picture list, Mv1 is scaled to the picture. The scaled Mv1 and Mv2 are combined to form the bi-prediction motion vector.

  - Otherwise, only Mv1 is stored for the boundary area.

For a CU coded in a skip or merge mode, a CU level flag is signaled to indicate whether the triangular partitions mode is applied or not to the current CU. When the CU level flag is 1, an index indicating the split direction and the motion vectors of two partitions are signaled. The index ranges from 0 to 39.

Figure 1: An example of motion vector storage

Proposed simplifications

There are two aspects for simplification of triangular partitions in this proposal. The first aspect is related to simplification of motion vector storage of triangular partitions. A bi-prediction motion vector is stored for a 4x4 grid located on the boundary areas of partitions. When a bi-directional motion vector is derived, the reference picture mapping process is performed to convert the direction of one motion vector to the other direction when two partitions’ motion vector have the same direction (L0 or L1). In the proposed method, the reference picture mapping process is removed, and only Mv1 is stored for the boundary area when two partitions have the same direction.

The second aspect is focused on the reduction of the number of motion vector combinations for both partitions. In the current the triangular partitions, 40 combinations are defined to indicate the split direction and the motion vectors of two partitions. In the proposed method, it is restricted to 16 combinations. Likewise, the size of the LUT for combinations is reduced. In addition, the binarization of the combination index is modified with fixed length coding and truncated binary coding as shown in Table 1. The regular coding for the first bin is used. While, the bypass coding for remaining bins is used.

Table 1. Binarization for the reduced combinations (the number of combinations is 16)

| IDX | Bins |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |  |  |
| 1 | 0 | 0 | 1 |  |  |
| 2 | 0 | 1 | 0 |  |  |
| 3 | 0 | 1 | 1 |  |  |
| 4 | 1 | 0 | 0 | 0 |  |
| 5 | 1 | 0 | 0 | 1 |  |
| 6 | 1 | 0 | 1 | 0 |  |
| 7 | 1 | 0 | 1 | 1 |  |
| 8 | 1 | 1 | 0 | 0 | 0 |
| 9 | 1 | 1 | 0 | 0 | 1 |
| 10 | 1 | 1 | 0 | 1 | 0 |
| 11 | 1 | 1 | 0 | 1 | 1 |
| 12 | 1 | 1 | 1 | 0 | 0 |
| 13 | 1 | 1 | 1 | 0 | 1 |
| 14 | 1 | 1 | 1 | 1 | 0 |
| 15 | 1 | 1 | 1 | 1 | 1 |

Experimental results

The proposed methods have been implemented into VTM-3.0 software, and three tests below were conducted under the common test conditions [4].

- Test 1: Simplification of motion vector storage

- Test 2: Reduction of the number of motion information combinations

- Test 3: Test 1 + Test 2

The experimental results of the simplifications of triangular partitions compared to VTM-3.0 with VTM configuration are shown in Table 2, 3, and 4.

Table 2. Summary of experimental results (Test 1 over VTM-3.0)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | -0.01% | 0.02% |  | 0.07% |  | 100% |  | 100% |  |
| Class A2 | 0.00% | 0.21% |  | 0.11% |  | 100% |  | 100% |  |
| Class B | 0.00% | 0.01% |  | -0.02% |  | 100% |  | 100% |  |
| Class C | 0.02% | -0.12% |  | -0.06% |  | 100% |  | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.02% |  | 0.01% |  | 100% |  | 100% |  |
| Class D | -0.05% | -0.03% |  | 0.03% |  | 100% |  | 100% |  |
| Class F (optional) | -0.01% | 0.00% |  | 0.01% |  | 100% |  | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.11% |  | -0.05% |  | 0.03% |  | 100% |  | 100% |
| Class C | 0.18% |  | 0.37% |  | 0.34% |  | 100% |  | 100% |
| Class E | 0.26% |  | 0.71% |  | -0.09% |  | 100% |  | 99% |
| Overall | 0.17% |  | 0.28% |  | 0.10% |  | 100% |  | 100% |
| Class D | 0.06% |  | -0.22% |  | 0.27% |  | 100% |  | 100% |
| Class F (optional) | 0.08% |  | 0.13% |  | 0.19% |  | 100% |  | 99% |

Table 3. Summary of experimental results (Test 2 over VTM-3.0)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.00% | -0.04% |  | 0.08% |  | 96% |  | 100% |  |
| Class A2 | 0.01% | 0.08% |  | 0.10% |  | 95% |  | 100% |  |
| Class B | 0.01% | 0.02% |  | 0.01% |  | 96% |  | 100% |  |
| Class C | 0.03% | 0.14% |  | 0.17% |  | 95% |  | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.05% |  | 0.09% |  | 96% |  | 100% |  |
| Class D | 0.03% | 0.02% |  | -0.04% |  | 95% |  | 101% |  |
| Class F (optional) | 0.04% | 0.02% |  | 0.01% |  | 94% |  | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% |  | -0.25% |  | -0.17% |  | 95% |  | 100% |
| Class C | 0.07% |  | 0.25% |  | 0.19% |  | 95% |  | 101% |
| Class E | 0.00% |  | 0.12% |  | -0.48% |  | 92% |  | 100% |
| Overall | 0.03% |  | 0.01% |  | -0.13% |  | 94% |  | 100% |
| Class D | 0.04% |  | 0.18% |  | 0.46% |  | 94% |  | 101% |
| Class F (optional) | 0.09% |  | 0.31% |  | 0.21% |  | 93% |  | 100% |

Table 4. Summary of experimental results (Test 3 over VTM-3.0)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.01% | -0.09% |  | 0.02% |  | 96% |  | 100% |  |
| Class A2 | 0.00% | 0.06% |  | 0.16% |  | 96% |  | 100% |  |
| Class B | 0.03% | 0.05% |  | 0.07% |  | 96% |  | 100% |  |
| Class C | 0.05% | 0.14% |  | 0.13% |  | 95% |  | 100% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.02% | 0.05% |  | 0.10% |  | 96% |  | 100% |  |
| Class D | 0.03% | 0.04% |  | -0.10% |  | 95% |  | 100% |  |
| Class F (optional) | 0.02% | 0.04% |  | 0.01% |  | 94% |  | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.14% |  | -0.04% |  | 0.20% |  | 95% |  | 100% |
| Class C | 0.25% |  | 0.45% |  | 0.51% |  | 95% |  | 100% |
| Class E | 0.26% |  | 0.80% |  | 0.26% |  | 93% |  | 100% |
| Overall | 0.20% |  | 0.33% |  | 0.32% |  | 94% |  | 100% |
| Class D | 0.19% |  | 0.41% |  | 0.84% |  | 94% |  | 100% |
| Class F (optional) | 0.19% |  | 0.43% |  | 0.20% |  | 93% |  | 99% |

Conclusions

This contribution presents two simplifications of triangular partitions. Test 1 reduces the complexity of encoder and decoder by removing the reference picture mapping process, and it is reported that 0.00% BD-rate loss with 100% encoding time and 100% decoding time for RA. Test 2 has the minor BD-rate loss of 0.01% with the significant encoding time reduction of 4% for RA.

References

- R. Liao and C. Lim, “CE10.3.1.b: Triangular prediction unit mode,” JVET document, JVET-L0124, Oct. 2018

- VVC Test Model (VTM), https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3),” JVET document, JVET-L1001, Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Suhring, “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-L1010, Oct. 2018.

Patent rights declaration(s)

Korea Aerospace University and ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).