| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0362 |
| --- | --- |

| Title: | CE13: In-loop filters disabled across face discontinuities (Test 1.1.a and Test1.1.b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sheng-Yen Lin, Lin Liu, Jian-Liang Lin, Yung-Chang Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Philippe Hanhart, Yuwen He 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {Sheng-Yen.Lin, Lin.Liu, jl.lin, yc.zhang,cc.ju}@mediatek.com Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com |
| Source: | MediaTek Inc. InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of Test 1.1.a/b in CE13: in-loop filter disabled across face discontinuities on HEC/PHEC. In terms of end-to-end WS-PSNR-Y/U/V, the experimental results reportedly demonstrate that Test 1.1.a achieves an average -0.57%/-0.58%/-0.37% and -0.27%/-0.93%/-0.80% BD-rate saving compared to PHEC and HEC, respectively, and Test 1.1.b achieves an average -0.01%/-0.29%/-0.13% and 0.29%/-0.63%/-0.56% BD-rate saving compared to PHEC and HEC, respectively. Moreover, in terms of codec PSNR, Test 1.1.b achieves an average -0.32%/-1.41%/-1.23% BD-rate saving compared to PHEC. In addition, the subjective quality of Test 1.1.a and Test 1.1.b is improved by alleviating the seam artifacts compared to HEC and PHEC, respectively.

# Introduction

In the last meeting, Core Experiment 13 (CE13) [1] was established to study removing subjective artifacts at face boundaries for the 360° video category. This contribution reports the results of in-loop filters disabled across face discontinuities implemented on HEC and PHEC proposed in JVET-L-0232 [2], JVET-L0347 [3], JVET-L0348 [4] and JVET-L0350 [5].

The in-loop filters in a conventional video coding system is used to remove undesired artifacts and enhance encoding quality. For a 360° video, the layout for a specific projection format usually has one or multiple discontinuous edges. Applying the in-loop filters on these discontinuous edges directly may result in poor visual quality and coding efficiency because the accessed pixels (to be referenced and/or filtered) cross discontinuous edges are jointly processed (filtered). To address this problem, the conventional in-loop filters are modified such that the discontinuous edge can be properly processed.

# Proposed Method

In this contribution, the in-loop filters including the de-blocking filter, sample adaptive offset (SAO) filter and the adaptive loop filter (ALF), are performed necessarily changes to avoid inappropriately filtering at the discontinuous edge.

## De-blocking Filter

The proposed de-blocking filtering method is a straightforward approach that disables the de-blocking filtering process when the filtered pixels cross the discontinuous edge as illustrated in Fig. 1.

Fig. 1. The de-blocking filtering is disabled when the filtered pixels cross the discontinuous edge

## SAO Filter

As proposed in JVET-L0348 [4], when the referenced pixels of current pixel in the SAO filtering process cross the discontinuous edge, the proposed modification disables the SAO filtering process of the current pixel to avoid inadequate filtering process.

Fig. 2. The SAO filtering process is disabled when the referenced pixels cross the discontinuous edge

## Adaptive Loop Filter

For ALF, when filtering a CU, reconstructed sample positions used in filtering are clipped to be within the same side of the face discontinuity to which the current CU belongs.

# Experimental Results

The simulations of the Test 1.1.a and Test 1.1.b are conducted on HEC and PHEC, respectively. The software is implemented on 360Lib-8.0 [6] and VTM-3.0 [7] followed by the JVET common test conditions and evaluation procedures for 360° video [8]. The resolution of each face is set to 1280x1280, which yields the frame resolution is 3840x2560. As compared to PHEC and HEC, the BD-rate performance, in terms of end-to-end WS-PSNR, provided by Test 1.1.a and Test 1.1.b are listed in Table I and Table II, respectively. In addition, the BD-rate performance of codec PSNR for Test 1.1.b is listed in Table III.

Table I. BD-rate performance for Test 1.1.a compared to PHEC and HEC under VTM-3.0 in terms of end-to-end WS-PSNR

|  | Anchor : PHEC |  |  | Anchor : HEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -0.22% | -0.33% | 0.54% | -0.22% | -1.21% | -0.69% |
| ChairLift | -0.47% | -0.36% | -0.39% | -0.36% | -0.54% | -0.41% |
| KiteFlite | -0.65% | -0.32% | -0.35% | -0.18% | -0.37% | -0.37% |
| Harbor | -0.57% | -0.17% | -0.29% | -0.70% | -1.30% | -1.69% |
| Trolley | -0.84% | -0.44% | -0.30% | -0.16% | -0.78% | -0.56% |
| GasLamp | -0.73% | -0.52% | -0.60% | -0.26% | -1.01% | -0.91% |
| Balboa | -0.37% | -0.57% | 0.15% | -0.31% | -0.60% | -0.33% |
| Broadway | -0.32% | -0.56% | -0.21% | -0.27% | -0.60% | -0.40% |
| Landing2 | -0.70% | -1.98% | -1.66% | -0.23% | -2.78% | -2.34% |
| BranCastle2 | -0.86% | -0.57% | -0.55% | -0.06% | -0.11% | -0.28% |
| Class S1 | -0.58% | -0.36% | -0.23% | -0.31% | -0.87% | -0.77% |
| Class S2 | -0.56% | -0.92% | -0.57% | -0.22% | -1.02% | -0.83% |
| Overall | -0.57% | -0.58% | -0.37% | -0.27% | -0.93% | -0.80% |

Table II. BD-rate performance for Test 1.1.b compared to PHEC and HEC under VTM-3.0 in terms of end-to-end WS-PSNR

|  | Anchor : PHEC |  |  | Anchor : HEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | 0.13% | -0.49% | 0.35% | 0.13% | -1.37% | -0.88% |
| ChairLift | -0.07% | -0.12% | -0.10% | 0.05% | -0.29% | -0.12% |
| KiteFlite | -0.03% | 0.02% | -0.13% | 0.43% | -0.03% | -0.15% |
| Harbor | -0.10% | -0.32% | -0.09% | -0.23% | -1.45% | -1.49% |
| Trolley | -0.04% | -0.12% | -0.25% | 0.65% | -0.46% | -0.50% |
| GasLamp | -0.06% | -0.16% | -0.19% | 0.41% | -0.64% | -0.49% |
| Balboa | 0.04% | -0.42% | 0.06% | 0.10% | -0.44% | -0.41% |
| Broadway | 0.06% | -0.06% | -0.13% | 0.11% | -0.09% | -0.31% |
| Landing2 | -0.04% | -1.36% | -0.99% | 0.43% | -2.17% | -1.67% |
| BranCastle2 | 0.02% | 0.16% | 0.11% | 0.82% | 0.63% | 0.39% |
| Class S1 | -0.03% | -0.20% | -0.07% | 0.24% | -0.71% | -0.61% |
| Class S2 | 0.02% | -0.42% | -0.24% | 0.36% | -0.52% | -0.50% |
| Overall | -0.01% | -0.29% | -0.13% | 0.29% | -0.63% | -0.56% |

Table III. BD-rate performance for Test 1.1.b compared to PHEC under VTM-3.0 in terms of codec PSNR

|  | Anchor : PHEC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| SkateboardInLot | -0.14% | -2.02% | -1.49% |
| ChairLift | -0.41% | -0.80% | -0.48% |
| KiteFlite | -0.25% | -0.49% | -0.59% |
| Harbor | -0.78% | -1.92% | -2.37% |
| Trolley | -0.21% | -1.11% | -0.81% |
| GasLamp | -0.30% | -1.49% | -1.45% |
| Balboa | -0.38% | -1.20% | -0.63% |
| Broadway | -0.35% | -0.91% | -0.92% |
| Landing2 | -0.29% | -3.82% | -3.21% |
| BranCastle2 | -0.08% | -0.31% | -0.34% |
| Class S1 | -0.35% | -1.30% | -1.20% |
| Class S2 | -0.27% | -1.56% | -1.27% |
| Overall | -0.32% | -1.41% | -1.23% |

# Subjective Quality Testing

In Fig. 3 to Fig. 10, the cropped viewport images of HEC, PHEC, Test 1.1.a and Test 1.1.b are demonstrated (under VTM-3.0 with QP = 37) in order to evaluate the seam artifacts appeared at  discontinuous edges.

Fig. 3. The subjective quality of Broadway (frame 231) in HEC. The image is cropped from the viewport centered in degree (45, -35).

Fig. 4. The subjective quality of Broadway (frame 231) in Test 1.1.a. The image is cropped from the viewport centered in degree (45, -35).

Fig. 5. The subjective quality of Broadway (frame 231) in PHEC. The image is cropped from the viewport centered in degree (45, -35).

Fig. 6. The subjective quality of Broadway (frame 231) in Test 1.1.b. The image is cropped from the viewport centered in degree (45, -35).

Fig. 7. The subjective quality of Landing2 (frame 59) in HEC. The image is cropped from the viewport centered in degree (45, -35).

Fig. 8. The subjective quality of Landing2 (frame 59) in Test 1.1.a. The image is cropped from the viewport centered in degree (45, -35).

Fig. 9. The subjective quality of Landing2 (frame 59) in PHEC. The image is cropped from the viewport centered in degree (45, -35).

Fig. 10. The subjective quality of Landing2 (frame 59) in Test 1.1.b. The image is cropped from the viewport centered in degree (45, -35).

# Conclusion

In this contribution, the performance of the in-loop filters disabled across face discontinuities on HEC and PHEC is investigated. The conventional in-loop filters are modified to avoid improper filtering at the discontinuous edge. The experimental results demonstrate that Test 1.1.a and Test 1.1.b reduce the visible seam artifacts with slightly improved compression efficiency compared to HEC and PHEC, respectively.

# References

- P. Hanhart, J.-L. Lin, and C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1033, Oct. 2018, Macau, CN.

- P. Hanhart, Y. He and Y. Ye, “CE13: HEC with deblocking and ALF disabled across face discontinuities (Test 4.1.c),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0232, Oct. 2018, Macao, CN.

- S.-Y. Lin, J-L Lin, H-C Lin, S-K Chang, Y-C Chang, C-C Ju, “CE13: De-blocking filter disabled across face discontinuities (Test 4.1.a),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0347, Oct. 2018, Macao, CN.

- S.-Y. Lin, L. Liu, J.-L. Lin, H.-C. Lin, S.-K. Chang, Y-C Chang and C.-C. Ju, “CE13: In-loop filters disabled across face discontinuities (Test 4.1.b and Test 4.1.d),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0348, Oct. 2018, Macau, CN.

- S.-Y. Lin, L. Liu, J.-L. Lin, S.-K. Chang, Y-C Chang and C.-C. Ju, “CE13: Padding and in-loop filters disabled across face discontinuities (Test 7.1),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0350, Oct. 2018, Macau, CN.

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- Versatile Video Coding Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- P. Hanhart, J. Boyce, K. Choi and J.-L. Lin, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1012, Oct. 2018, Macau, CN.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).