| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0364 |
| --- | --- |

| Title: | CE13: Test 1.1.a with face row based geometry padding of reference pictures (Test 2.1.a, Test 2.1.c and Test 2.1.d) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng-Hsuan Shih, Jian-Liang Lin, Yung-Chang Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {ch.shih, jl.lin, yc.zhang, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of Test 2.1.a, Test 2.1c, and Test 2.1.d in CE13: Test 1.1.a with face row based geometry padding of reference pictures. The simulation results reportedly show that, in terms of end-to-end WS-PSNR-Y/U/V, Test 1.1.a with face row based geometry padding of reference pictures using projection with Lanczos 3/2 interpolation (Test 2.1.a) achieves the average -2.00%/-2.48%/-2.38%, and -1.99%/-2.20%/-2.25% BD-rate reduction compared to PHEC and PHEC with in-loop filters disabled across face discontinuities, respectively. For Test 2.1.c (Test 1.1.a with face row based geometry padding of reference pictures using projection with nearest-neighbor interpolation), the results reportedly show the average -1.75%/-2.17%/-2.08% and -1.74%/-1.89%/-1.95% BD-rate reduction compared to PHEC and PHEC with in-loop filters disabled across face discontinuities, respectively. For Test 2.1.d (Test 1.1.a with face row based geometry padding of reference pictures using unfolding-based padding without interpolation), the results reportedly show the average -1.32%/-1.81%/-1.65% and -1.31%/-1.53%/-1.52% BD-rate saving compared to PHEC and PHEC with in-loop filters disabled across face discontinuities, respectively.

# Introduction

In the last meeting, Core Experiment 13 (CE13) [1] was established to study removing subjective artifacts at face boundaries for the 360° video category. This contribution reports the results of face row based geometry padding of reference pictures proposed in JVET-E0057 [2], JVET-J0019 [3], JVET-K0141 [4], and JVET-L0346 [5].

In conventional video coding systems, the replication padding for reference pixel data is used in inter prediction when the reference region outside the frame boundary. Fig. 1 shows a typical implementation for replication padding which extends the boundary pixels in the border region.

Fig. 1 Replication padding in conventional video system

In this contribution, the spherical-based inter prediction for cubemap projection is proposed to use the spherical neighboring pixel data for efficient prediction based on the spherical continuity in 360° videos. In the proposed method, the inter prediction replaces the replication padding with the geometry padding when the reference pixels are located outside the frame boundary (layout boundary) or across the discontinuous edge.

# Proposed method

## Test 1.1.a with face row based geometry padding of reference pictures using projection with Lanczos 3/2 interpolation (Test 2.1.a)

The geometry padding in Fig. 2 demonstrates a pixel position qb qboutside the Bottom face is represented by its spherical corresponding pixel qa, which is located at the Front face. This spherical corresponding pixel is obtained by using the HEC mapping functions [6]. When the derived coordinate qa is fractional, 6-tap and 4-tap Lanczos interpolation filters [7][8] are applied for the luminance and chrominance components, respectively.

Fig. 2 Geometry padding process

To achieve the geometry padding [9][10] in inter prediction, the proposed implementation maximally utilizes face continuity by applying geometry padding on extended padding areas of top and bottom face row for each reference frame data. The padding size is the LCU size with additional 16 pixels.

## Test 1.1.a with face row based geometry padding of reference pictures using projection with nearest neighbor interpolation (Test 2.1.c)

The same geometric padding process in Section 2.1 is conducted in this test except that the interpolation method is replaced by nearest neighbor (NN) rounding [7].

## Test 1.1.a with face row based geometry padding of reference pictures using unfolding-based padding without interpolation (Test 2.1.d)

In this test, the padding areas of face rows are filled by duplicating spherical adjacent faces as illustrated in Fig. 3.

Fig. 3 Adjacent faces of target face

# Experiment results

The experiments are conducted on 360Lib-8.0 [11] and VTM-3.0 [12] followed by the JVET common test conditions (CTC) and evaluation procedures [13] under the VTM-3.0 [14] configuration setting. Following the CTC, the frame size is set to 3840x2560 with the face size of 1280x1280. Compared to PHEC and PHEC with in-loop filters disabled across face discontinuities, Table I demonstrates the average BD-rate performance in terms of end-to-end WS-PSNR for the proposed face row based geometry padding of reference pictures using projection with Lanczos 3/2 interpolation (Test 2.1.a). Table II demonstrates the average BD-rate performance of the proposed face row based geometry padding of reference pictures using projection with nearest neighbor (NN) interpolation (Test 2.1.c). Table III demonstrates the average BD-rate performance of the proposed face row based geometry padding of reference pictures using unfolding-based padding without interpolation (Test 2.1.d).

Table I BD-rate performance of Test 2.1.a

|  | CTC Anchor: PHEC |  |  | CE Anchor: PHEC with in-loop filters disabled across face discontinuities |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -3.42% | -3.83% | -4.10% | -3.54% | -3.35% | -4.44% |
| ChairLift | -3.47% | -3.68% | -3.43% | -3.41% | -3.56% | -3.33% |
| KiteFlite | -0.69% | -0.41% | -0.40% | -0.65% | -0.43% | -0.27% |
| Harbor | -0.60% | -0.21% | -0.27% | -0.50% | 0.11% | -0.18% |
| Trolley | -1.05% | -0.75% | -0.77% | -1.01% | -0.64% | -0.52% |
| GasLamp | -0.81% | -0.63% | -0.70% | -0.74% | -0.47% | -0.52% |
| Balboa | -2.78% | -4.07% | -3.74% | -2.82% | -3.66% | -3.81% |
| Broadway | -2.14% | -3.55% | -2.79% | -2.20% | -3.48% | -2.68% |
| Landing2 | -2.18% | -4.16% | -3.81% | -2.14% | -2.82% | -2.83% |
| BranCastle2 | -2.85% | -3.49% | -3.81% | -2.87% | -3.65% | -3.91% |
| Class S1 | -1.67% | -1.58% | -1.61% | -1.64% | -1.39% | -1.54% |
| Class S2 | -2.49% | -3.82% | -3.54% | -2.51% | -3.40% | -3.31% |
| Overall | -2.00% | -2.48% | -2.38% | -1.99% | -2.20% | -2.25% |

Table II BD-rate performance of Test 2.1.c

|  | CTC Anchor: PHEC |  |  | CE Anchor: PHEC with in-loop filters disabled across face discontinuities |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -2.99% | -3.34% | -3.33% | -3.12% | -2.86% | -3.68% |
| ChairLift | -2.99% | -3.25% | -3.10% | -2.92% | -3.13% | -3.00% |
| KiteFlite | -0.65% | -0.31% | -0.38% | -0.62% | -0.33% | -0.25% |
| Harbor | -0.61% | -0.26% | -0.36% | -0.50% | 0.07% | -0.28% |
| Trolley | -1.02% | -0.72% | -0.69% | -0.99% | -0.61% | -0.44% |
| GasLamp | -0.80% | -0.61% | -0.67% | -0.73% | -0.46% | -0.49% |
| Balboa | -2.35% | -3.69% | -3.15% | -2.40% | -3.28% | -3.22% |
| Broadway | -1.71% | -3.04% | -2.48% | -1.77% | -2.97% | -2.36% |
| Landing2 | -1.92% | -3.65% | -3.35% | -1.88% | -2.31% | -2.37% |
| BranCastle2 | -2.49% | -2.85% | -3.32% | -2.51% | -3.01% | -3.42% |
| Class S1 | -1.51% | -1.41% | -1.42% | -1.48% | -1.22% | -1.36% |
| Class S2 | -2.12% | -3.31% | -3.07% | -2.14% | -2.89% | -2.84% |
| Overall | -1.75% | -2.17% | -2.08% | -1.74% | -1.89% | -1.95% |

Table III BD-rate performance of Test 2.1.d

|  | CTC Anchor: PHEC |  |  | CE Anchor: PHEC with in-loop filters disabled across face discontinuities |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -1.75% | -2.83% | -2.43% | -1.88% | -2.35% | -2.77% |
| ChairLift | -2.15% | -2.52% | -2.39% | -2.08% | -2.41% | -2.29% |
| KiteFlite | -0.68% | -0.34% | -0.44% | -0.64% | -0.37% | -0.32% |
| Harbor | -0.60% | -0.19% | -0.23% | -0.50% | 0.14% | -0.14% |
| Trolley | -1.02% | -0.68% | -0.60% | -0.98% | -0.56% | -0.35% |
| GasLamp | -0.82% | -0.60% | -0.71% | -0.76% | -0.45% | -0.52% |
| Balboa | -1.80% | -3.18% | -2.49% | -1.84% | -2.77% | -2.55% |
| Broadway | -1.33% | -2.47% | -1.90% | -1.39% | -2.41% | -1.78% |
| Landing2 | -1.20% | -2.95% | -2.81% | -1.16% | -1.60% | -1.83% |
| BranCastle2 | -1.87% | -2.37% | -2.53% | -1.88% | -2.53% | -2.64% |
| Class S1 | -1.17% | -1.20% | -1.13% | -1.14% | -1.00% | -1.07% |
| Class S2 | -1.55% | -2.74% | -2.43% | -1.57% | -2.33% | -2.20% |
| Overall | -1.32% | -1.81% | -1.65% | -1.31% | -1.53% | -1.52% |

# Subjective quality testing

In Fig. 4 to Fig. 11, the resulted viewport images under VTM-3.0 encoder configuration with QP = 37 of Test 1.1.a (HEC with in-loop filters disabled across face discontinuities), Test 2.1.a, Test 2.1.c and Test 2.1.d are demonstrated to evaluate the seam artifacts appeared at face discontinuities.

Fig. 4 The cropped image of Broadway frame 218 (Test 1.1.a)

Fig. 5 The cropped image of Broadway frame 218 (Test 2.1.a)

Fig. 6 The cropped image of Broadway frame 218 (Test 2.1.c)

Fig. 7 The cropped image of Broadway frame 218 (Test 2.1.d)

Fig. 8 The cropped image of Landing2 frame 74 (Test 1.1.a)

Fig. 9 The cropped image of Landing2 frame 74 (Test 2.1.a)

Fig. 10 The cropped image of Landing2 frame 74 (Test 2.1.c)

Fig. 11 The cropped image of Landing2 frame 74 (Test 2.1.d)

# Conclusion

In this contribution, the subjective and objective performance of the proposed face row based geometry padding of reference pictures is investigated. The experimental data reports that proposed methods achieve noticeable BD-rate saving and remarkably reduce the visible seam artifacts compared to HEC with in-loop filters disabled across face discontinuities.

# Reference

- P. Hanhart, J.-L. Lin, and C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1033, Oct. 2018, Macau, CN.

- C.-H. Shih, H.-C. Lin, J.-L Lin, S.-K. Chang, “AHG8: Face-based padding scheme for cube projection”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-E0057, Jan. 2017, Geneva, CH.

- J.-L. Lin, Y.-H. Lee, C.-H. Shih, S.-Y. Lin, H.-C. Lin, S.-K. Chang, P. Wang, L. Liu, and C.-C. Ju, “Description of 360° video coding technology proposal by MediaTek,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J0019, Apr. 2018, San Diego, USA

- C.-H. Shih, J.-L. Lin, H.-C. Lin, S.-K. Chang, C.-C. Ju, “AHG8: 360°-based inter/intra prediction for cubemap projection”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K0141, July 2018, Ljubljana, SI.

- C.-H. Shih, J.-L. Lin, H.-C. Lin, S.-K. Chang, Y.-C. Chang, C.-C. Ju, “CE13: Face row based geometry padding of reference pictures (Test 3.1.a and Test 3.1.c)”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L0346, Oct. 2018, Macau, CN.

- Y.-H. Lee, J.-L. Lin, S.-K. Chang and C.-C. Ju,”CE13: Modified Cubemap Projection in JVET-J0019 (Test 5),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0131, July 2018, Ljubljana, SI.

- Y. Ye, J. Boyce, "Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 7", Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K1004, July 2018, Ljubljana, SI.

- Lanczos resampling, https://en.wikipedia.org/wiki/Lanczos_resampling

- J. Sauer and M.Wien, “Geometry correction for motion compensation of planar-projected 360VR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-D0067, Oct. 2016, Chengdu, China

- Y. He, Y. Ye, P. Hanhart, and X. Xiu , “AHG8: Geometry padding for 360 video coding,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-D0075, Oct. 2016, Chengdu, China

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- VVC software Benchmark Set, https://vcgit.hhi.fraunhofer.de/JVET-L-CE13/VVCSoftware_VTM/

- P. Hanhart, J. Boyce, K. Choi, and J.-L. Lin, “JVET common test conditions and evaluation procedures for 360° video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1012, Oct. 2018, Macau, CN.

- Versatile Video Coding Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).