| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0366 |
| --- | --- |

| Title: | CE-6 related: Transform Simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Davood Saffar, Per Wennersten, Jacob Ström Torshamnsgatan 23 164 80 Stockholm, Sweden | Tel: Email: | +46107146437 {christopher.hollmann, davood.saffar, per.wennersten, jacob.strom} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The Multiple Transform Selection (MTS) tool adopted into the VVC Test Model VTM evaluates DCT-2 in horizontal and vertical direction, and four combinations of DCT-8 and DST-7 transforms. Among all the combinations, DCT-8 in horizontal and vertical direction is used only about 2% of the time. This contribution proposes to exclude this combination from the MTS tool. As this removal comes with the expense of compression deficiency, the following changes are proposed. First, DCT-2 is employed instead of DCT-8 for block edges of size 32. Second, the coding of mts_tu_idx is removed and replaced by two new flags: mts_dst_flag, which signals the usage of DST-7 in both directions, and mts_tu_flag, which signals which of the other combinations of DCT-X (DCT-2 or DCT-8) and DST-7 is used.

Each of the new proposed flags mts_dst_flag and mts_tu_flag use one context each for intra and one context each for inter predicted blocks. For the mts_tu_flag, the value states the selection of a more probable or less probable combination on the decoder side. For intra predicted blocks, the selection is made based on the intra mode and for the inter predicted blocks, it is made based on the block size.

In this proposal, the coding of the mts_cu_flag has been revised. For intra predicted blocks, the context is selected based on the block size and the intra direction mode. On the other hand, for inter predicted blocks, the context is designated based on the block height and width.

It is reported that, compared to the VTM-3.0, this proposal achieves -0.03% and -0.01% luma BD-rates for AI and RA, respectively, while the encoding time decreases by 12% for AI and by 2% for RA. The decoder runtime decreases by 1% for AI and remains the same for RA. The contribution also states that the proposal leads to a simplification, since transform coefficients for DCT-8 of size 32 are no longer needed.

# Introduction

At the July 2018 meeting of the JVET in Ljubljana the Multiple Transform Selection (MTS) tool was adopted into the VVC Test Model VTM [1]. A close inspection done in AhG 13 between the Ljubljana and Macao meetings showed that the tool improves compression efficiency for All Intra (AI) by about -2.8% while increasing the encoder run time to over 220%. The corresponding Random Access (RA) BD rate number is around -1.3% and gets an encoder run time of 120% [2].

The tool, which is only enabled for intra coded blocks in the Common Test Conditions (CTC) [3], tests five different combinations of transforms:

- DCT-2 horizontally and DCT-2 vertically,

- DST-7 horizontally and DST-7 vertically,

- DST-7 horizontally and DCT-8 vertically,

- DCT-8 horizontally and DST-7 vertically,

- DCT-8 horizontally and DCT-8 vertically.

These combinations are encoded in the bit stream as shown in Figure 1.

mts_cu_flag

mts_tu_idx_hor

DCT-2 hor

DCT-2 ver

mts_tu_idx_ver

mts_tu_idx_ver

DCT-8 hor

DCT-8 ver

DCT-8 hor

DST-7 ver

DST-7 hor

DCT-8 ver

DST-7 hor

DST-7 ver

0

0

0

0

1

1

1

1

Figure 1 Coding tree of the MTS tool

However, the likelihood of a combination being used is not equal for all five combinations. While there are small differences depending on the block size, the following list summarizes the usage of transforms quite well.

- DCT-2 horizontally and DCT-2 vertically: ~20%,

- DST-7 horizontally and DST-7 vertically: ~60%,

- DST-7 horizontally and DCT-8 vertically: ~10%,

- DCT-8 horizontally and DST-7 vertically: ~10%,

- DCT-8 horizontally and DCT-8 vertically: ~2%.

As the combination of using DCT-8 both horizontal and vertical is only used in rare cases, our proposal removes this option both for the encoder and for the decoder. This reduces the run time for the encoder, as one less combination has to be evaluated. However, disallowing one combination comes with a decrease in compression performance, as this combination was still chosen for a few cases where it performed better than the other combinations.

To avoid the loss in compression performance, we propose the following changes:

- Replacing the existing DCT-8 with the existing DCT-2 for blocks with edges of size 32. The DCT-2 of size 32 is implemented using a partial butterfly structure, which gives an additional speed-up compared to the DCT-8, that uses a straight-forward matrix multiplication. It also simplifies hardware implementations, since DCT-8 coefficients do not need to be stored for transforms of size 32.

- Removing the mts_tu_idx (both horizontal and vertical) and replacing it with two new flags:

  - mts_dst_flag to signal whether to use the DST-7 in both directions

  - mts_tu_flag to signal in which direction to use the DCT-X (i.e., DCT-8 or DCT-2) and DST-7. As change 1) allows the codec to have two different DCTs at this stage based on the block size, the DCT is marked as DCT-X in this contribution.

These changes are visualized in Figure 2.

mts_cu_flag

mts_dst_flag

DCT-2 hor

DCT-2 ver

DST-7 hor

DST-7 ver

mts_tu_flag

DCT-X hor

DST-7 ver

DST-7 hor

DCT-X ver

0

0

0

1

1

1

Figure 2 Coding tree after proposed changes. DCT-X is either DCT-8 or DCT-2.

Regarding the CABAC encoding, the flags use the following context configurations:

- mts_dst_flag: 1 context each for intra predicted blocks and inter predicted blocks.

- mts_tu_flag: 1 context each for intra predicted blocks and inter predicted blocks. The value that is encoded represents whether the decoder should take a more probable or less probable combination of transforms.

For intra predicted blocks, the determination of which combination is more likely is made based on the direction of the intra prediction. In the proposed solution, the combination of DST-7 in horizontal and DCT-X in vertical directions is deemed more probable if the direction of the intra prediction is from a direction closer to horizontal than to vertical. If the direction of the intra prediction is closer to vertical than to horizontal, the combination of DCT-X horizontal and DST-7 vertical is considered more probable.

For blocks using inter prediction a different determination has to be made, as there is no intra direction available. Here the decision of which combination is more probable is made based on the shape of the block. If the block has a larger height than width, the combination of DST-7 horizontal and DCT-X vertical is considered more probable. If the block has a larger width than height, the combination of DCT-X horizontal and DST-7 vertical is deemed more probable.

Regarding the coding of the mts_cu_flag, we also propose a change in the context selection. For intra coded blocks, the context selection is made based on the following table:

|  |  | Type of intra direction |  |
| --- | --- | --- | --- |
|  |  | Closer to horizontal or vertical | Closer to diagonal |
| Block sizes | 32xN or Nx32 | id 0 | id 1 |
|  | 16xN or Nx16 | id 2 | id 3 |
|  | 4xN, 8xN, Nx4 or Nx8 | id 4 | Id 5 |

“Closer to horizontal or vertical” is used if the intra direction is one of the modes 10-22 or 46-57, whereas “closer to diagonal” is used if the intra direction is one of the modes 0-9, 23-45 or 58-66.

For blocks using inter prediction, the context of the mts_cu_flag is chosen based on the size of the block. This is visualized in the following table:

|  |  | Block width |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 4 | 8 | 16 | 32 |
| Block height | 4 | id 5 | id 2 | id 1 | id 0 |
|  | 8 | id 2 | id 5 | id 3 | id 1 |
|  | 16 | id 1 | id 3 | id 4 | id 3 |
|  | 32 | id 0 | id 1 | id 3 | id 4 |

# Test Results

In the following test results the changes described above are made:

- Not allowing DCT-8 in both the horizontal and vertical direction,

- DCT-8 for size 32 replaced by DCT-2,

- The coding tree as depicted in Figure 2 is used,

- The above-mentioned changes to the CABAC context selection are made.

Due to a bug in the code, which was discovered shortly before the meeting, the test results had to be adjusted. We would like to thank HHI for cross-checking the results of the fixed version. Due to the same bug, some test results were still missing at the time of the update.

## Tests according to CTC

These are the results for All Intra according to CTC:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,07% | 0,00% | 0,02% | 87% | 98% |
| Class A2 | -0,05% | 0,07% | -0,01% | 89% | 99% |
| Class B | -0,05% | -0,09% | -0,04% | 87% | 99% |
| Class C | -0,02% | -0,07% | -0,18% | 88% | 99% |
| Class E | -0,09% | 0,03% | -0,01% | 88% | 100% |
| Overall | -0,05% | -0,03% | -0,05% | 88% | 99% |
| Class D | -0,02% | 0,02% | -0,09% | 88% | 100% |
| Class F | -0,01% | -0,08% | 0,02% | 88% | 101% |

These are the results for Random Access according to CTC:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | 0,13% | 0,06% | 98% | 99% |
| Class A2 | -0,06% | -0,10% | 0,05% | 98% | 101% |
| Class B | -0,03% | 0,20% | -0,11% | 98% | 101% |
| Class C | 0,01% | -0,38% | 0,00% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,02% | -0,03% | -0,01% | 98% | 100% |
| Class D | 0,00% | -0,64% | -0,06% | 97% | 101% |
| Class F | -0,03% | 0,11% | 0,04% | 97% | 102% |

These are the results for Low Delay B according to CTC:

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  | 98% |  | 100% |  |
| Class C | 0,03% | 0,33% |  | 0,43% |  | 99% |  | 101% |  |
| Class E | -0,05% | 0,96% |  | -0,88% |  | 99% |  | 95% |  |
| Overall |  |  |  |  |  | 99% |  | 99% |  |
| Class D | 0,00% | 0,33% |  | 0,57% |  | 99% |  | 102% |  |
| Class F |  |  |  |  |  | 99% |  | 99% |  |

## Results for CTC with Inter MTS enabled

These are the results for Random Access:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 with MTS Inter On |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  | 96% | 99% |
| Class A2 |  |  |  | 95% | 99% |
| Class B | 0,05% | 0,06% | -0,08% | 97% | 100% |
| Class C | 0,04% | -0,20% | -0,15% | 96% | 99% |
| Class E |  |  |  |  |  |
| Overall |  |  |  | 96% | 99% |
| Class D | 0,04% | -0,73% | -0,18% | 96% | 99% |
| Class F | -0,03% | 0,02% | 0,05% | 96% | 99% |

These are the results for Low Delay B:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 with MTS Inter On |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  | #DIV/0! | #DIV/0! |
| Class C | 0,12% | -0,15% | 0,10% | 95% | 100% |
| Class E | 0,08% | 0,47% | -0,81% | 96% | 99% |
| Overall |  |  |  | #DIV/0! | #DIV/0! |
| Class D | 0,11% | -0,51% | -0,34% | 97% | 100% |
| Class F | -0,07% | 0,16% | -0,34% | 97% | 99% |

# Recommendation

We recommend the group to adopt the changes as proposed in this contribution.

## Changes to Specification

All changes are relative to version 7 of the third draft of the VVC specification [4].

In 7.3.4.12 Residual coding syntax (deletions in red, additions in green):

| } |  |
| --- | --- |
| if( tu_mts_flag[ x0 ][ y0 ] && ( cIdx = = 0 ) ) { |  |
| mts_idx[ x0 ][ y0 ][ cIdx ] | ae(v) |
| mts_dst_flag[ x0 ][ y0][ cIdx ] | ae(v) |  |
| if( mts_dst_flag[ x0 ][ y0 ] = = 1 ) |  |  |
| mts_tu_idx[ x0 ][ y0 ][ cIdx ] | ae(v) |  |
| } |  |  |
| } |  |  |

In 7.4.5.12 Residual coding semantics:

- Remove the paragraphs

mts_idx[ x0 ][ y0 ][ cIdx ] specifies which transform kernels are applied to theresidual samples along the horizontal and vertical direction of the current transform block. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture the array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb and equal to 2 for Cr.

When mts_idx[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to −1.

Replace with:

mts_dst_flag[ x0 ][ y0 ][ cIdx ] and mts_tu_idx[ x0 ][ y0 ][ cIdx ] specify which transform kernels are applied to the residual samples along the horizontal and vertical direction of the current transform block. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture the array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb and equal to 2 for Cr.

When mts_dst_flag[ x0 ][ y0 ][ cIdx ] and mts_tu_idx[ x0 ][ y0 ][ cIdx ] are not present, they are inferred to be equal to -1.

In 8.4.4.1 General:

- Add additional input:

  - a variable predMode specifying the prediction mode of the current transform block,

  - a variable IntraPredMode specifying the intra direction of the current transform block,

- Remove the paragraph

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-16 depending on mts_idx[ xTbY ][ yTbY ][ cIdx ].

Replace with:

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-16a and Table 8-16b depending on mts_dst_flag[ xTbY ][ yTbY ][ cIdx ], mts_tu_flag[ xTbY ][ yTbY ][ cIdx ], the prediction mode CuPredMode, and the intra direction.

- Replace Table 8-16 with:

Table 8-16a – Specification of trTypeHor and trTypeVer depending on mts_dst_flag[ x ][ y ][ cIdx ], mts_tu_flag[ x ][ y ][ cIdx ] and IntraPredMode, if the block is using intra prediction (CuPredMode == INTRA)

| mts_dst_flag [ xTbY ][ yTbY ][ cIdx ] |  | mts_tu_flag [ xTbY ][ yTbY ][ cIdx ] |  | IntraPredMode |  | trTypeHor |  | trTypeVer |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| −1 |  | −1 |  | 0-66 |  | 0 |  | 0 |  |
| 0 |  | −1 |  | 0-66 |  | 1 |  | 1 |  |
| 1 |  | 0 |  | 0-34 |  | 1 |  | 2 |  |
| 1 | 0 | 35-66 | 2 | 1 |
| 1 | 1 | 0-34 | 2 | 1 |
| 1 |  | 1 |  | 35-66 |  | 1 |  | 2 |  |

Table 8-16b – Specification of trTypeHor and trTypeVer depending on mts_dst_flag[ x ][ y ][ cIdx ], mts_tu_flag[ x ][ y ][ cIdx ] and the dimensions nTbW and nTbH of the block, if the block is using inter prediction (CuPredMode == INTER)

| mts_dst_flag [ xTbY ][ yTbY ][ cIdx ] | mts_tu_flag [ xTbY ][ yTbY ][ cIdx ] | nTbH > nTbW | trTypeHor | trTypeVer |
| --- | --- | --- | --- | --- |
| −1 | −1 | does not matter | 0 | 0 |
| 0 | −1 | does not matter | 1 | 1 |
| 1 | 0 | true | 1 | 2 |
| 1 | 0 | false | 2 | 1 |
| 1 | 1 | true | 2 | 1 |
| 1 | 1 | false | 1 | 2 |

In 9.5.3.1 General, in table 9-4:

| residual_coding( ) | mts_idx[ ][ ][ ] | FL | cMax = 3 |
| --- | --- | --- | --- |

Replace with:

| residual_coding( ) | mts_dst_flag[ ][ ][ ] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | mts_tu_flag[ ][ ][ ] | FL | cMax = 1 |

In 9.5.4.2.1 General, in table 9-10:

| tu_mts_flag[ ][ ] | cqtDepth | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |

Replace with

| tu_mts_flag[ ][ ] | 0..5 (clause 9.5.4.2.8) | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |

In the same table:

| mts_idx[ ][ ][ ] CuPredMode[ x0 ][ y0 ] = = MODE_INTRA | 0 | 1 | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |
| mts_idx[ ][ ][ ] CuPredMode[ x0 ][ y0 ] ! = MODE_INTRA | 2 | 3 | na | na | na | na |

Replace with

| mts_dst_flag[ ][ ][ ] CuPredMode[ x0 ][ y0 ] = = MODE_INTRA | 0 | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |
| mts_dst_flag[ ][ ][ ] CuPredMode[ x0 ][ y0 ] ! = MODE_INTRA | 1 | na | na | na | na | na |
| mts_tu_flag[ ][ ][ ] CuPredMode[ x0 ][ y0 ] = = MODE_INTRA | 0 | na | na | na | na | na |
| mts_tu_flag[ ][ ][ ] CuPredMode[ x0 ][ y0 ] ! = MODE_INTRA | 1 | na | na | na | na | na |

Add section 9.5.4.2.8 Derivation process of ctxInc for the syntax element tu_mts_flag:

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture, the colour component cIdx, the width and the height of the current coding block in luma samples cbWidth and cbHeight, the prediction mode CuPredMode of the current luma block, and, if CuPredMode is equal to MODE_INTRA, the direction of the intra prediction IntraPredMode.

Output of this process is ctxInc.

If the prediction of the current block CuPredMode is equal to MODE_INTRA, ctxInc is determined based on Table 9-12:

Table 9-12 – Specification of ctxInc for tu_mts_flag for intra predicted blocks

|  |  |  | Intra direction IntraPredMode |  |
| --- | --- | --- | --- | --- |
|  |  | Allowed values for N | 10-22 or 46-57 | 0-9 or 23-45 or 58-66 |
| Block sizes (cbWidth x cbHeight) | 32xN or Nx32 | 4, 8, 16 or 32 | ctxInc = 0 | ctxInc = 1 |
|  | 16xN or Nx16 | 4, 8 or 16 | ctxInc = 2 | ctxInc = 3 |
|  | 4xN or 8xN | 4 or 8 | ctxInc = 4 | ctxInc = 5 |

If the prediction of the current block CuPredMode is not equal to MODE_INTRA, ctxInc is determined based on Table 9-13:

Table 9-13 – Specification of ctxInc for tu_mts_flag for inter predicted blocks

|  |  | Block width (cbWidth) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 4 | 8 | 16 | 32 |
| Block height (cbHeight) | 4 | ctxInc = 5 | ctxInc = 2 | ctxInc = 1 | ctxInc = 0 |
|  | 8 | ctxInc = 2 | ctxInc = 5 | ctxInc = 3 | ctxInc = 1 |
|  | 16 | ctxInc = 1 | ctxInc = 3 | ctxInc = 4 | ctxInc = 3 |
|  | 32 | ctxInc = 0 | ctxInc = 1 | ctxInc = 3 | ctxInc = 4 |

# References

[1] G. J. Sullivan, J.-R. Ohm. Meeting Report of the 11th JVET Meeting (Ljubljana, 10–18 July 2018), section 6.6, JVET-K1000, Ljubljana, July 2018

[2] W-J Chien, J. Boyce, et al. JVET AHG report: Tool reporting procedure (AHG13), JVET-L0013, Macao, October 2018

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring (editors). JVET common test conditions and software reference configurations for SDR video. JVET-L1010, Macau, October 2018

[4] B. Bross, J. Chen, S. Liu (editors). Versatile Video Coding (Draft 3), JVET-L1001-v7, Macao, October 2018

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).