| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 January 2019 | Document: JVET-M0374-v2 |
| --- | --- |

| Title: | AHG12: Flexible tiles to support MCTS use cases |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Mitra Damghanian Martin Pettersson | Email: | rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to include support for tile boundaries that do not span across the entire picture from left to right or top to bottom. Tile sizes are proposed to be signaled individually, either by copying the tile size from the previous tile size in decoding order or by using one tile width and one tile height code word.

It is proposed to keep the uniform tile partitioning option (uniform_tile_spacing_flag=1) but to use the following signaling when tiles are not uniformly spaced:

- Signal the number of tiles in a picture in the PPS

- Signal a subtile height and a subtile width in CTU units using one UVLC code word each

- Specify individual tile sizes, in subtile raster scan order either by copying the tile size from the previous tile size in decoding order or by one tile width and one tile height UVLC code word each using the subtile sizes

It is claimed by the proponents that the current VVC tile design doesn’t support some tile partitions that are supported by HEVC by splitting HEVC tiles using slices. It is claimed that one such partition is included into clauses D.6.3 and D.6.4 of OMAF and adopted in the VR Industry Forum Guidelines.

It is claimed that using MCTSs and mixing tiles from sources of different spatial resolutions is not currently well supported in VVC. The proponents believe that the proposed design provides support for such use cases, including the OMAF example.

A new version of this contribution showing proposed draft text on top of a VVC draft containing all adoptions from Macau is planned when such a draft is made available.

# Introduction and problem statement

A tile design including a segmentation of tiles into VCL NAL units consisting of a tile group header was adopted at the 13th meeting in Macau. The design only supports NAL units consisting of an integer number of tiles.

Tiles in its current form in VVC (and HEVC) are restricted by the use of tile rows and tile columns. This means that all tile boundaries are required to span across the entire picture; from left to right or from top to bottom. This limits the flexibility of tiles. For example, the left tile partition in Figure 1 is not supported in VVC.

Figure 1 – An example flexible Tile partition

To virtually support such a tile structure using VVC, one may specify the tile partition shown to the right in Figure 1. However, that tile structure contains a lot more tile boundaries which harms compression efficiency and may cause unnecessary tiling artifacts. Also, if the tiles are to be completely spatially independent, VVC only allows for either enabling or disabling in-loop filtering dependencies for all tile boundaries of a picture using the loop_filter_across_tiles_enabled_flag. Therefore, a lot of unnecessary in-loop filtering restrictions may occur.

Figure 2 shows a CMP partitioning example from section 2.5 of JVET-K0300-v1. The JVET-K0300-v1 document reports that the example is included into clauses D.6.3 and D.6.4 of OMAF and adopted in the VR Industry Forum Guidelines. This and other 360 video streaming solutions use segments of different resolutions which in HEVC requires a mix of tiles and slices. As can be seen to the right of Figure 2, the rightmost tile column consisting of small tiles is realized by splitting tiles using HEVC slices.

Since a mix of slices and tiles is not supported in VVC, the example in Figure 2 is not supported.

Figure 2 – Example MCTS-based RWMR viewport-dependent 360° streaming

# Proposal

We propose a tile design for VVC where rectangular shaped tiles of varying sizes are supported. Both the example shown to the left in Figure 1 as well as the OMAF example in Figure 2 is supported by the proposal.

We propose the following additions and modifications to the current VVC tile design:

- That tile boundaries are not required to span across the entire picture.

- Keep the uniform tile partitioning option.

- Add the option to use flexible tiles where:

  - the number of tiles in a picture is signaled in the PPS

  - the tile sizes are specified in subtile units to reduce the signaling bit cost

  - tiles are specified individually in subtile raster scan order by their individual tile size either by copying the tile size from the previous tile size in decoding order or by one tile width and one tile height UVLC code word each

We do not propose to add support for slices in VVC for the following reasons:

- Slices are more expensive in terms of bit cost compared to the proposed flexible tile design.

- We believe that it is cleaner to use tiles as the only partition tool instead of tiles and slices in combination.

- Supporting reposition of MCTS without VCL re-writing seems impossible if slice headers need to be inserted during rewriting. By allowing tiles of different sizes, re-writing without touching VCL data only requires a solution for the problem of modifying tile group addresses outside the VCL. Such a solution is proposed in JVET-M0373.

# Proposed syntax and semantics and level constraints

Table 1 – Proposed Tile syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| } else { |  |
| number_of_tiles_in_picture_minus2 | ue(v) |
| subtile_width_minus1 | ue(v) |
| subtile_height_minus1 | ue(v) |
| for ( i=0; i < number_of_tiles_in_picture_minus2+2 ; i++ ) { |  |
| if ( i > 0 ) |  |
| use_previous_tile_size_flag | u(1) |
| if ( use_previous_tile_size_flag == 0) { |  |
| tile_width_minus1[ i ] | ue(v) |
| tile_height_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

single_tile_in_pic_flag equal to 1 specifies that there is only one tile in each picture referring to the PPS. single_tile_in_pic_flag equal to 0 specifies that there is more than one tile in each picture referring to the PPS.

It is a requirement of bitstream conformance that the value of single_tile_in_pic_flag shall be the same for all PPSs that are activated within a CVS.

If single_tile_in_pic_flag is equal to 1, the value of the variable tile_id and the value of the two-dimentional array TileId are specified as follows:

tile_id = 0

for( y= 0; y < PicHeightInCtbsY; y++ )

for( x = 0; x < PicWidthInCtbsY; x++ )

TileId[ x ][ y ] = “no tile”

uniform_tile_spacing_flag equal to 1 specifies that tile column boundaries and likewise tile row boundaries are distributed uniformly across the picture. uniform_tile_spacing_flag equal to 0 specifies that tile column boundaries and likewise tile row boundaries are not distributed uniformly across the picture but signalled explicitly using the syntax elements tile_column_width_minus1[ i ] and tile_row_height_minus1[ i ]. When not present, the value of uniform_tile_spacing_flag is inferred to be equal to 1.

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture. num_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of num_tile_columns_minus1 is inferred to be equal to 0.

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture. num_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of num_tile_rows_minus1 is inferred to be equal to 0.

The variable NumTilesInPic is set equal to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ).

When single_tile_in_pic_flag is equal to 0, NumTilesInPic shall be greater than 1.

number_of_tiles_in_picture_minus2 plus 2 specifies the number of tiles in a picture. number_of_tiles_in_picture_minus2 shall be in the range of 0 to PicSizeInCtbsY − 2, inclusive.

[Editors note: By changing this to minus1, UVLC value 0 would indicate 1 tile and the single_tile_in_pic_flag can be removed]

subtile_width_minus1 plus 1 specifies the width of the subtile unit in coding tree units. subtile_width_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive.

subtile_height_minus1 plus 1 specifies the height of the subtile unit in coding tree units. subtile_height_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive.

use_previous_tile_size_flag equal to 1 specifies that the size of the current tile is equal to the size of the previous tile. use_previous_tile_size_flag equal to 0 specifies that the size of the current tile is not equal to the size of the previous tile. When not present, the value of use_previous_tile_size_flag is inferred to be equal to 0.

When use_previous_tile_size_flag is equal to 1, the value of the i’th element of the arrays tile_height_minus1 and tile_width_minus1 are derived as follows:

tile_height_minus1[ i ]  = tile_height_minus1[ i - 1 ]

tile_width_minus1[ i ] = tile_width_minus1[ i - 1 ]

tile_width_minus1[ i ] plus 1 specifies the width of the i-th tile in tile units.

tile_height_minus1[ i ] plus 1 specifies the height of the i-th tile in tile units.

The value of the variable tile_id and the value of the two-dimentional array CTU_tile_assigment are derived as follows:

for( tile_coordinate_y = 0; tile_coordinate_y < PicHeightInCtbsY; tile_coordinate_y++ )

for( tile_coordinate_x = 0; tile_coordinate_x < PicWidthInCtbsY; tile_coordinate_x++ )

if( TileId[ tile_coordinate_x ][ tile_coordinate_y ] == “no tile” )

break

for( ctu_y = 0 ; ctu_y  < ( tile_height_minus1[ i ] + 1 ) * ( subtile_height_minus1 + 1 ) && (tile_coordinate_y + ctu_y ) < PicHeightInCtbsY ; ctu_y++ )

for( ctu_x = 0 ; ctu_x < ( tile_width_minus1[ i ] + 1 ) * ( subtile_width_minus1 + 1 ) && (tile_coordinate_x + ctu_x ) < PicWidthInCtbsY ; ctu_x++ )

TileId[ tile_coordinate_x + ctu_x ][ tile_coordinate_y + ctu_y ] = tile_id

tile_id++

It is a requirement of bitstream conformance that the break statement is reached for each value of i.

It is a requirement of bitstream conformance that the value of TileId[ tile_coordinate_x + ctu_x ][ tile_coordinate_y + ctu_y ] shall be equal to “no tile” prior to the assignment to the value tile_id.

It is a requirement of bitstream conformance that after all assignments of TileId, TileId[ x ][ y ] shall not be equal to “no tile” for x ranging from 0 to PicWidthInCtbsY-1 inclusive and y ranging from 0 to PicHeightInCtbsY-1 inclusive.

NOTE X – The derivation of the variable tile_id and the value of the two-dimentional array CTU_tile_assigment as specified in the semantics of tile_height_minus1[ i ] are exectuted for each value of i. This means that the derivation is done after each loop of the variable i, including when use_previous tile_size_flag is not present, when use_previous tile_size_flag is equal to 0 and when use_previous tile_size_flag is equal to 1. Likewise, the bitstream conformance condition that the value of TileId[ tile_coordinate_x + ctu_x ][ tile_coordinate_y + ctu_y ] shall be equal to “no tile” prior to the assignment to the value tile_id shall be true for any assignment done in any loop.

Table 2 – Proposed level constraints

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols | Max # of tiles MaxTiles |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 | 4 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 | 6 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 | 25 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 | 25 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 | 110 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 | 110 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 | 110 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 | 440 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 | 440 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 | 440 |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).