| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0379r2 |
| --- | --- |

| Title: | CE6-related: Further Simplification on top of tests CE6-2.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Naser, E. Francois, F. Le Léannec | Tel: Email: | Karam.naser@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

In tests CE6-2.2, simplified implementation of DCT8/DST7 is proposed. The simplification is based on DFT decomposition of DCT8/DST7 transform for the sizes of 32x32 and 16x16 where matrix multiplications are used with reduced number of operations and overall memory.

This contribution studies further simplifications of CE6-2.2 by considering the following:

- Odd basis functions of DCT2 of large sizes are replaced by DCT8 with DFT based implementation

- DCT8/DST7 for small block sizes are replaced by DCT4/DST4 that can be inherited from DCT2

The overall impact is a further reduction of the overall memory and a reduced number of operations for computing DCT2 for large block sizes.

# Introduction

In the context of transform design for VVC, several methods have been proposed for simplifying both the MTS transforms as well as the core DCT2 transform. Specifically, type IV transforms have been proposed in ‎[1] to replace DST7 and DCT8 as these transforms can be inherited from the DCT2 transforms without any additional memory/complexity. Otherwise, DST7/DCT8 are embedded in [2] inside the DCT2 transform so that the overall memory is reduced. The method is known as Compound Orthogonal Transform (COT).

In the same direction, test CE6-2.2 proposes to reduce both the complexity and memory requirement by employing the DFT decomposition such that the smaller matrices are stored, and less operations are required ‎[3]. The method is applicable to transform dimensions of 16 and 32. Therefore, the implementation of DCT8/DST7 for dimensions of 4 and 8 are still required.

This contribution proposes to fill the gap and study the optimal simplification by embedding the DCT8/DST7 design for sizes of 16 and 32 inside the DCT2 transform, as done in COT, and to replace DCT8/DST7 of sizes 8x8 and 4x4 by DCT4/DST4 which are inherited from DCT2 where no extra memory/complexity is required.

# Proposed methods

## Method 1: DFT based COT

As in COT, the odd basis functions of DCT2 are replaced by DCT8 of half size, where DCT8 is implemented by DFT decomposition as in test CE6-2.2. Specifically, the 32x32 DCT2 is implemented as follows:

- The even basis functions (0:2:4:…30) are the same as VTM3.0

- The odd basis functions (1:3:5:…:31) are implemented by DFT decomposition of DCT8 of size 16x16

And the 64x64 is implemented as follows:

- The even basis functions (0:4:8: … :60) are the same as VTM3.0

- The even basis functions (2:6:10: … :62) are implemented by DFT decomposition of DCT8 of size 16x16

- The odd basis functions (1:3:5: … :31) are implemented by DFT decomposition of DCT8 of size 32x32

This way, the implementation of DCT8/DST7 for sizes 32x32 and 16x16 are embedded in DCT2. It should be noted that the implementation of DCT8/DST7 for these sizes requires the following number of matrix coefficients:

| Matrix | Number of Coefficients |
| --- | --- |
| DFT_R_3 | 1 |
| DFT_I_3 | 1 |
| DFT_R_5 | 2 |
| DFT_I_5 | 2 |
| DFT_R_11 | 5 |
| DFT_I_11 | 5 |
| DFT_R_13 | 6 |
| DFT_I_13 | 6 |
| Sum | 28 |

Additionally, the memory requirement for DCT2 of size 32x32 is reduced as follows:

- 16x16 even basis functions (basis number 0:2:4: … :30), which are the same as DCT2 of size 16x16

- Implementation of DCT8 of size 16x16

Similarly, for DCT2 of size 64x64:

- 16x16 even basis function (basis number 0:4:8: … :60) , which are the same as DCT2 of size 16x16

- Implementation of DCT8 of size 16x16

- Implementation of DCT8 of size 32x32

Another important property of DCT2 matrix is that it contains all the transform matrices of sizes of 12nx12n. That is, only DCT2 of size 64x64 is needed for all transform sizes of VTM (2, 4, 8, 16, 32, 64). Therefore, the total memory for DCT2 is 64x64 times the number of bits required to store the matrix elements.

Method 1 requires only the DCT2 of size of 16x16, in addition to the 28 coefficients of DFT matrices. That is, the total number of elements is 256+28 = 284.

## Method 2: type IV transform for small transform sizes

Another simplification is proposed by replacing the remaining DCT8/DST7 of sizes 4x4 and 8x8 by DCT4/DST4 transforms of the same sizes. These transforms can be obtained respectively from the transform matrices of DCT2 of sizes 8x8 and 16x16. Therefore, neither extra memory nor further complexity is required to implement MTS for small transform sizes.

# Simulation results

Method 1 and 2 are implemented on top of VTM 3.0 software. The common testing conditions have been followed to generate the results for AI, RA and LDB configurations. Two testing scenarios where studied (as in test CE6-2.2): 8- and 16-bit representation of the DFT matrices.

## Method 1

Table 1: Results of common testing condition with 8 bits representation of DFT matrices

|  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 | 0.29% | 0.19% | 0.33% | 127% | 133% |  |
|  | Class A2 | 0.30% | 0.25% | 0.26% | 122% | 125% |  |
|  | Class B | 0.14% | 0.20% | 0.14% | 121% | 124% |  |
|  | Class C | 0.04% | 0.08% | -0.12% | 116% | 114% |  |
|  | Class E | 0.15% | 0.16% | 0.11% | 119% | 126% |  |
|  | Overall | 0.17% | 0.17% | 0.13% | 121% | 123% |  |
|  | Class D | 0.05% | -0.10% | 0.07% | 113% | 111% |  |
|  | Class F | 0.04% | 0.09% | 0.03% | 115% | 110% |  |
|  |  | Random Access Main 10 |  |  |  |  |  |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 | 0.20% | 0.30% | 0.20% | 115% | 120% |  |
|  | Class A2 | 0.18% | 0.21% | 0.36% | 110% | 111% |  |
|  | Class B | 0.12% | 0.25% | 0.11% | 111% | 111% |  |
|  | Class C | 0.05% | -0.02% | 0.23% | 108% | 109% |  |
|  | Class E |  |  |  |  |  |  |
|  | Overall | 0.13% | 0.18% | 0.21% | 111% | 112% |  |
|  | Class D | 0.05% | -0.25% | -0.23% | 107% | 103% |  |
|  | Class F | 0.06% | 0.17% | 0.20% | 111% | 107% |  |
|  |  | Low delay B Main10 |  |  |  |  |  |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 |  |  |  |  |  |  |
|  | Class A2 |  |  |  |  |  |  |
|  | Class B | 0.18% | -0.28% | -0.08% | 110% | 120% |  |
|  | Class C | 0.22% | 0.28% | 0.39% | 108% | 113% |  |
|  | Class E | 0.25% | 0.20% | 0.07% | 110% | 108% |  |
|  | Overall | 0.21% | 0.03% | 0.11% | 109% | 115% |  |
|  | Class D | 0.08% | 0.06% | 0.41% | 105% | 102% |  |
|  | Class F | 0.30% | 0.30% | -0.08% | 110% | 108% |  |

Table 2: Results of common testing condition with 16 bits representation of DFT matrices

|  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 | 0.30% | 0.11% | 0.25% | 127% | 133% |  |
|  | Class A2 | 0.28% | 0.17% | 0.19% | 121% | 125% |  |
|  | Class B | 0.13% | 0.09% | 0.12% | 121% | 123% |  |
|  | Class C | 0.03% | 0.05% | -0.11% | 116% | 114% |  |
|  | Class E | 0.15% | 0.16% | 0.14% | 120% | 127% |  |
|  | Overall | 0.16% | 0.11% | 0.11% | 121% | 124% |  |
|  | Class D | 0.04% | 0.04% | -0.07% | 112% | 110% |  |
|  | Class F | 0.04% | 0.05% | 0.18% | 114% | 110% |  |
|  |  | Random Access Main 10 |  |  |  |  |  |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 | 0.24% | 0.24% | 0.27% | 115% | 120% |  |
|  | Class A2 | 0.18% | 0.23% | 0.26% | 110% | 110% |  |
|  | Class B | 0.13% | 0.18% | 0.12% | 110% | 110% |  |
|  | Class C | 0.07% | -0.01% | 0.25% | 108% | 108% |  |
|  | Class E |  |  |  |  |  |  |
|  | Overall | 0.15% | 0.15% | 0.21% | 110% | 111% |  |
|  | Class D | -0.03% | -0.21% | -0.16% | 106% | 102% |  |
|  | Class F | 0.04% | 0.08% | 0.04% | 111% | 105% |  |
|  |  | Low delay B Main10 |  |  |  |  |  |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 |  |  |  |  |  |  |
|  | Class A2 |  |  |  |  |  |  |
|  | Class B | 0.18% | -0.30% | -0.15% | 111% | 120% |  |
|  | Class C | 0.14% | 0.21% | 0.26% | 108% | 113% |  |
|  | Class E | 0.25% | 0.26% | -0.11% | 110% | 110% |  |
|  | Overall | 0.18% | 0.01% | -0.01% | 110% | 115% |  |
|  | Class D | 0.02% | -0.08% | -0.14% | 105% | 102% |  |
|  | Class F | 0.27% | -0.01% | -0.42% | 110% | 109% |  |

## Method 1 + Method 2

Table 3: Results of common testing condition with 8 bits representation of DFT matrices

|  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 | 0.41% | 0.34% | 0.51% | 127% | 133% |  |
|  | Class A2 | 0.37% | 0.23% | 0.20% | 121% | 124% |  |
|  | Class B | 0.10% | -0.07% | -0.04% | 122% | 124% |  |
|  | Class C | -0.26% | -0.23% | -0.37% | 117% | 114% |  |
|  | Class E | -0.15% | -0.20% | -0.27% | 121% | 126% |  |
|  | Overall | 0.08% | -0.01% | -0.02% | 121% | 124% |  |
|  | Class D | -0.30% | -0.28% | -0.40% | 115% | 112% |  |
|  | Class F | -0.15% | -0.13% | -0.12% | 116% | 110% |  |
|  |  | Random Access Main 10 |  |  |  |  |  |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 | 0.28% | 0.40% | 0.38% | 131% | 127% |  |
|  | Class A2 | 0.23% | 0.24% | 0.41% | 123% | 115% |  |
|  | Class B | 0.12% | 0.27% | 0.12% | 123% | 112% |  |
|  | Class C | -0.02% | 0.05% | -0.04% | 119% | 109% |  |
|  | Class E |  |  |  |  |  |  |
|  | Overall | 0.14% | 0.23% | 0.19% | 123% | 115% |  |
|  | Class D | -0.11% | -0.31% | -0.31% | 115% | 98% |  |
|  | Class F | -0.05% | -0.11% | 0.04% | 114% | 91% |  |
|  |  | Low delay B Main10 |  |  |  |  |  |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 |  |  |  |  |  |  |
|  | Class A2 |  |  |  |  |  |  |
|  | Class B | 0.20% | -0.16% | 0.02% | 128% | 126% |  |
|  | Class C | 0.15% | 0.29% | 0.03% | 123% | 120% |  |
|  | Class E | 0.25% | 0.91% | 0.82% | 130% | 98% |  |
|  | Overall | 0.19% | 0.26% | 0.23% | 127% | 116% |  |
|  | Class D | 0.00% | -0.04% | -0.21% | 119% | 105% |  |
|  | Class F | 0.16% | -0.14% | -0.24% | 129% | 98% |  |

Results of common testing condition with 16 bits representation of DFT matrices

|  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 | 0.42% | 0.23% | 0.41% | 126% | 134% |  |
|  | Class A2 | 0.32% | 0.16% | 0.19% | 121% | 124% |  |
|  | Class B | 0.07% | -0.08% | -0.08% | 122% | 123% |  |
|  | Class C | -0.26% | -0.20% | -0.33% | 117% | 114% |  |
|  | Class E | -0.14% | -0.24% | -0.24% | 121% | 125% |  |
|  | Overall | 0.06% | -0.04% | -0.04% | 121% | 123% |  |
|  | Class D | -0.31% | -0.44% | -0.43% | 114% | 111% |  |
|  | Class F | -0.19% | -0.20% | -0.12% | 116% | 110% |  |
|  |  | Random Access Main 10 |  |  |  |  |  |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 | 0.27% | 0.37% | 0.38% | 131% | 127% |  |
|  | Class A2 | 0.23% | 0.32% | 0.33% | 123% | 116% |  |
|  | Class B | 0.11% | 0.29% | 0.18% | 123% | 113% |  |
|  | Class C | -0.06% | -0.13% | -0.17% | 118% | 110% |  |
|  | Class E |  |  |  |  |  |  |
|  | Overall | 0.12% | 0.20% | 0.16% | 123% | 115% |  |
|  | Class D | -0.10% | -0.21% | -0.12% | 115% | 98% |  |
|  | Class F | -0.01% | 0.06% | 0.08% | 114% | 91% |  |
|  |  | Low delay B Main10 |  |  |  |  |  |
|  |  | Over VTM 3.0 |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |  |
|  | Class A1 |  |  |  |  |  |  |
|  | Class A2 |  |  |  |  |  |  |
|  | Class B | 0.21% | -0.09% | 0.05% | 128% | 126% |  |
|  | Class C | 0.16% | 0.47% | 0.24% | 123% | 120% |  |
|  | Class E | 0.27% | 0.07% | 0.05% | 130% | 98% |  |
|  | Overall | 0.21% | 0.14% | 0.11% | 127% | 116% |  |
|  | Class D | 0.01% | 0.02% | 0.24% | 119% | 104% |  |
|  | Class F | 0.14% | 0.18% | 0.08% | 128% | 97% |  |

# Memory Analysis

The overall number of matrix coefficients that are required for the implementation of the transform design of VTM-3.0 software is computed as follows:

- DCT2: 64x64 = 4096 coefficients

- DCT8/DST7 = 4x4 + 8x8 + 16x16 + 32x32 = 1360 coefficients

It should be noted the same matrices for DST7 and DCT8 can be used with a simple reversing of the input samples and negating the output coefficients.

With 8 bits representation of the matrices, 4096+1360=5456 bytes of memory are required for the implementation.

Method 1 proposes to replace the odd basis functions of DCT2 by DCT8 which is implemented by DFT decomposition. As explained in Sec. ‎2.1, it requires 256 coefficients for DCT2 (256 bytes) and 28 coefficients for DCT8/DST7 of sizes 16x16 and 32x32. Additionally, the implementation of 4x4 and 8x8 of DCT8/DST7 remains unchanged, which requires 80 bytes (4x4+8x8). As shown in the tables below, 364 and 392 bytes are required for method 1, which represents 6.7% and 7.2% of the original memory requirement.

Table 4: Memory requirement for method 1 with 8 bits representation of DFT matrices

|  | Memory requirement (Bytes) |
| --- | --- |
| DCT2 | 256 |
| DCT8/DST7 (4x4 and 8x8) | 80 |
| DCT8/DST7 (16x16 and 32x32) | 28 |
| Sum | 364 |

Table 5: Memory requirement for method 1 with 16 bits representation of DFT matrices

|  | Memory requirement (Bytes) |
| --- | --- |
| DCT2 | 256 |
| DCT8/DST7 (4x4 and 8x8) | 80 |
| DCT8/DST7 (16x16 and 32x32) | 56 |
| Sum | 392 |

Method 2 simplifies the design by removing the required memory for the 8x8 and 4x4 DCT8/DST7 transform, where they are replaced by DCT4/DST4 that are obtained from DCT2 transform of sizes 8x8 and 16x16. This removes the 80 bytes required for these transforms and reduces the memory to 5.2% and 5.7% of the original memory when Method 1 and Method 2 are used together with bit depth of 8 and 16 respectively.

# Conclusion

This contribution proposes a simplification of the transform design in VTM software, where DFT decomposition is employed to implement DCT8/DST7 transforms of sizes 16x16 and 32x32 and to replace the odd basis functions of DCT2 by this DCT8. Furthermore, the remaining DCT8/DST7 of sizes 4x4 and 8x8 are replaced by DCT4/DST4 so the implementation of DCT2 can be used to derive these transforms.

It is shown that this can largely reduce the overall memory and the number of operations for both MTS and core transforms. This comes with a reasonable loss on RD performance.

# References

- K. Naser, F. Le Léannec, E. François, “CE6-related: Reduction of the number of core transforms in AMT” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-K0265

- X. Zhao, X. Li, S. Liu, “CE6: Compound Orthonormal Transform (Test 6.1.1 a/b/c/d)” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET- M0496

- K. Naser, E. Francois, F. Le Léannec “CE6: Fast DST-7/DCT-8 based on DFT and matrix multiplication (test 6.2.2)” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET- M0496

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).