| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0384 |
| --- | --- |

| Title: | Non-CE3: LM in the middle |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Guillaume Laroche Jonathan Taquet Patrice Onno | Tel: Email: | christophe.gisquet@crf.canon.fr |
| Source: | Canon Research France |  |  |

_____________________________

# Abstract

This proposal presents a new method to perform the multiple-model linear mode using straight lines. This goes through the definition of a knee point between the two lines, derived from the luma and chroma sample averages and extremum points. The method only operates on larger blocks to not affect the linear model worst case. It is asserted that the results provided by the modifications are respectively W%/X%/Y%/Z% (or W%/X%/Y%/Z%) for the AI/RA configurations.

# Technical content

## Processing stage / dependency problem

Computing averages (first stage) over the samples must be performed prior to any other pass over them to e.g. classify them or update whatever computations (second stage) are needed for deriving the model(s) parameters. Depending on the implementation, this can require additional buffering, and/or several stages in the pipeline.

## Discontinuity problem in unconstrained MMLM

The “MMLM” mode, and derivatives, currently being studied in CE3 determines two linear models, one per segment, for two segments defined by the average of the downsampled reconstructed luma samples over a selection of the block borders. Initially based on LMS solutions, a source of inefficiency is the discontinuity between the two lines. A solution is to determine a point in the middle, where either one or the two models are potentially the most wrong.

Figure 1: knee in the cloud

In the above, that point, T, indicated by an arrow to the knee between the 2 lines, offers a continuity, as well as define the new threshold used to select one model or the other.

This knee can be simply determined by a computation based on the luma and chroma averages together with their minimum and maximum, in a single pass, thus solving both the discontinuity and stage issues.

## Throughput requirements

As the MMLM mode applies to all block sizes, it increases the complexity of its worst case (smallest block size). The above ‘LM in the middle’ method can be applied to any regular CCLM mode solely based on the block size being processed.

Indeed, the above method is more likely to work if indeed CCLM in the middle is wrong, which is more likely the smaller the luma interval is. The method therefore suffers less from block size restrictions. Current smallest number of samples for any CCLM-coded block is 2x2, setting their required throughput. Relatively, the ‘LM in the middle’ method suffers minimally from working only on blocks of larger size, thereby not impacting the throughput worst case. The present results are for blocks of more than 32 samples.

As a result, the average block size for the CCLM modes increases, meaning these modes become more realistic for an encoder that is restricted in the depth of its analysis.

# Results for VTM3.0

Results are tentative and will be updated later.

# Conclusion

A simple way of determining 2 linear models through the use of a knee point has been presented. As particular care has been taken regarding complexity, it seems worthwhile to consider the method in a CE.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).