| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0385 |
| --- | --- |

| Title: | Non-Linear Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Taquet Christophe Gisquet Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 jonathan.taquet@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution introduces an adaptive clipping operation on the input samples values of the Adaptive Loop Filter in VTM 3.0. The goal of this adaptive clipping is to introduce some non-linearities to cap the difference between the input sample value to be filtered and the other neighbor input sample values of the filter.

Compared to VTM-3.0, the average Luma/Cr/Cb BDR gains are reported to be -0.36%/-0.51%/-0.69% for the AI configuration.

# Introduction

## Filtering in Adaptive Loop Filter

In VTM3, the filtering process of the Adaptive Loop Filter, is performed as follows:

Ox,y=i,jwi,j.Ix+i,y+j,	(1)

where samples I(x+i,y+j) are input samples, O(x,y) is the filtered output sample (i.e. filter result), and wi, j denotes the filter coefficients. In practice, in VTM3.0 it is implemented using integer arithmetic for fixed point precision computations:

Ox,y=i=-L2L2j=-L2L2wi, j.Ix+i,y+j+64≫7,	(2)

where L denotes  the filter length, and where wi, j are the filter coefficients in fixed point precision.

# Proposed modifications

## Filtering reformulation

Equation (1)1 can be reformulated, without coding efficiency impact, in the following expression:

Ox,y=Ix,y+(i,j)≠(0,0)wi, j.Ix+i,y+j-Ix,y,	(3)

where wi, j are the same filter coefficients as in equation (1) [excepted w0, 0 which is equal to 1 in equation (3) while it is equal to 1-(i,j)≠(0,0)wi, j in equation (1)].

## Modified filter

Using this above filter formula of (3), we can easily introduce nonlinearity to make ALF more efficient by using a simple clipping function to reduce the impact of neighbor sample values (Ix+i,y+j) when they are too different with the current sample value (Ix,y) being filtered.

In this proposal, The ALF filter is modified as follows:

O'x,y=Ix,y+(i,j)≠(0,0)wi, j.KIx+i,y+j-Ix,y,k(i,j),	(4)

where Kd,b=min⁡(b,max-b,d) is the clipping function, and k(i,j) are clipping parameters, which depends on the(i,j) filter coefficient. The encoder performs the optimization to find the best k(i,j).

In the proposed implementation, the clipping parameters ki,j are specified for each ALF filter, one clipping value is signaled per filter coefficient. It means that 12 clipping values are signalled in the bitstream per Luma filter and 6 clipping values for the Chroma filter.

In order to limit the signaling cost and the encoder complexity, we limit the evaluation of the clipping values to a small set of possible  values. In the proposal, we only use 3 possible values on INTRA slices.

Because the variance of the local differences is often higher for Luma than for Chroma, we use two different sets for the Luma and Chroma filters. We also include the maximum sample value (here 1024 for 10bits bit-depth) in each set, so that clipping can be disabled if it is not necessary.

The proposed sets of clipping values are provided in Table 1.

|  | INTRA |
| --- | --- |
| LUMA | { 10, 102, 1024 } |
| CHROMA | { 4, 24, 1024 } |

Table 1: Allowed clipping values

The clipping values are encoded in the slice header using a Golomb encoding corresponding to  the index of the clipping value in the set

## Encoder modifications

At encoder side, instead of computing one covariance matrix E and one cross-covariance matrix y per CTU and per frame, we compute NK×NK clipping covariance matrices Ecab and NK clipping cross-covariance matrices, yc[c]. NK is the number of allowed clipping values. Ecab is the clipping covariance matrix between inputs using a-th clipping value (same clipping for each input difference, no clipping for center value) and inputs using b-th clipping value. yc[c] is the cross-covariance matrix between expected output and inputs using c-th clipping value (same clipping for each input difference, no clipping for center value).

Thus, if we have fixed the filter clipping values (ki,j in equation (4)), if we use 1-d filtering notations (i.e. ku,v corresponds to the n-th filter coefficient k[n]), if we note c[n] the index of the clipping values in the set of available clipping values for the n-th filter coefficient, then we can extract one covariance matrix E, such that E[i,j]=Eccicj[i,j] for each i<N,j<N and one cross-covariance matrix y, such that yk=ycck[k] for each k<N, where i, j and k are filter coefficients indexes and N is the number of filter coefficients.

Using these matrices E and y, we can compute the Wiener filter as in VTM3.0.

The encoder needs to optimize k[n] for each n<N-1 (no clipping for coefficient N-1 which corresponds to the center of the filter, i.e. k(0,0) in equation (4)).

This optimization is performed using an iterative process: starting for fixed clipping indexes, we iteratively try to increase or decrease the clipping values while it improves the efficiency of the corresponding wiener filter.

The optimization is done each time a filter must be optimized: for each filter combination test on Luma component and for each CTU selection iteration on Luma and Chroma.

# Experimental results

Table 2 gives the results of the proposed nonlinear adaptive loop filter on top of VTM3.0 compared to VTM3.0 and its adaptive loop filter. The proposed modification shows interesting gains for limited encoder complexity increase. We may notice an increase of decoder complexity which is partly introduced by the additional filter operations (one subtraction and clipping per filter input sample), but this is also due to a significant increase of the number of filtered samples by ALF.

Table 2: Results of the proposed non-linear Adaptive Loop Filter compared to VTM3.0

# Conclusion

In this contribution, the results of non-linear adaptive loop filter are reported. We introduced a clipping operation in ALF to limit the impact of neighbooring samples of the currently filtered sample when the difference between these samples is very large.

According to these encouraging BDR results, we recommend to study the proposed modified ALF design in a CE.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).