| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0397 |
| --- | --- |

| Title: | CE6-related: DST-3 based transform kernels derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sandeep Shrestha Ankit Kumar Bumshik Lee Yonggil Lee Junsuk Park | Email: | sandeepshrestha407@chosun.kr ankitku@chosun.kr bslee@chosun.ac.kr yglee@humaxdigital.com parkjs@humaxdigital.com |
| Source: | Chosun University and Humax |  |  |

_____________________________

- Abstract

In Multiple Transform Selection (MTS) in VTM3.0, three transform kernels types (DCT-2, DST-7 and DCT-8) are used. This contribution proposes an efficient and low complexity transform kernels derivation method from a 64×64 DST-3 kernel. In this proposed method, a 64x64 DST-3 integer kernel can be partitioned into 32×32 DST-4, 16×16 DST-4, 8×8 DST-4, 4×4 DST-4, 2×2 DST-4 and 2×2 DST-3 with multiplication of unit matrices consisting of 0, 1 and -1. Any block size of DCT-2, DST-4 and DCT-4 kernel matrices can be derived from the proposed method. The proposed method overcomes the drawback of memory consumption by storing only 1368 elements (1368 bytes for 8-bit precision) instead of total 4096 elements (4096 bytes for 8-bit precision). In this contribution, DST-4 and DCT-4 have been used instead of DST-7 and DCT-8, respectively.

Problem Statement

In MTS in VTM3.0, DCT-2, DST-7 and DCT-8 transform kernels are used. Although it shows overall good results for all test sequences, it is observed that DST-4 and DCT-4 are more beneficial for smaller test sequences [JVET-L0118]. Moreover, DST-4 and DCT-4 have more advantages than DST-7 and DCT-8 in reusability of possible building block form different transform types and sizes. [JVET-L0118]. In this contribution, a kernel derivation scheme from the single largest size of kernel with a 64x64 size is proposed. In the proposed method, 64×64, 32×32, 16×16, 8×8, 4×4 and 2×2 DCT-2 as well as 32×32, 16×16, 8×8, 4×4 DST-4 and DCT-4 can be extracted from the 64x64 DST-3 kernel.

Proposed method

A 64x64 DST-3 matrix can be decomposed with a unified matrix and five-unit matrices consisting of 0, 1 and -1 as

DST-364=UABCDE                                                                (1)

where the U represent the unified matrix and can be expressed as Fig. 1, A, B, C, D and E matrix are the unit matrix consisting of -1, 0 and 1.

Fig. 1. Decomposition of a 64×64 DST-3 matrix (More details in the Excel link)

The proposed method stores only 1368 elements out of 64x64 matrix elements. For DST-4, 32×32, 16×16, 8×8 and 4×4 matrices can easily be extracted from the unified matrix of 64x64 DST-3 matrix as shown in Fig. 1. The kernels for various sizes of DCT-4 can be obtained by

DCT-4=S×DST-4×F                                                         (2)

where S is a sign changing matrix and F is a flipping matrix, given by

Fm,n=1,if n=N-1-m,0,otherwise,                                                          (3)

Sm,n=(-1)m,if n=m,0,otherwise.                                                         (4)

In order to obtain the DCT-2 matrix of various block sizes, DST-3 matrices are first obtained using the unified matrix and unit matrices.

DST-34=T[U64×A64]4                                                  (5)

DST-38=T[U64×M64]8                                                 (6)

DST-316=T[U64×N64]16                                              (7)

DST-332=T[U64×O64]32                                              (8)

DST-364=U64×P64                                                       (9)

where T[ ]N represent the function that takes N×N block of right-bottom part of the matrix,  U indicates the unified matrix as (1) and M, N, O and P matrices can be obtained using unit matrices as

| M=A×B N=A×B×C O=A×B×C×D P=A×B×C×D | (10) |
| --- | --- |

Finally, DCT-2 can readily be derived using (11)

DCT-2=F×DST-3×S                                                     (11)

where F and S are a flipping and sign changing matrices, respectively.

Experimental results

The proposed method has been implemented on VTM-3.0 reference software and experimentally evaluated for VTM test according to common test conditions for both AI, RA and LDB configurations.

Table 1. Simulation results for the RA configuration over VTM-3.0.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Table 2. Simulation results for the AI configuration over VTM-3.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Table 1. Simulation results for the LDB-10 configuration over VTM-3.0.

|  | Lower Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.31% | 0.43% | 0.31% | 100% | 100% |
| Class E | 0.42% | 0.64% | 0.80% | 97% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.27% | -0.10% | 0.98% | 99% | 100% |
| Class F (optional) |  |  |  |  |  |

- 4 Patent rights declaration(s)

Chosun University and Humax may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).