| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0401-v1 |
| --- | --- |

| Title: | CE3: Classification-based mean value for CCLM coefficients derivation (tests 2.5.1-2.5.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma, Alexey Filippov, Vasily Rufitskiy, Haitao Yang, Jianle Chen | Email: | maxiang6@huawei.com alexey.filippov@huawei.com vasily.rufitskiy@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co. |  |  |

_____________________________

# Abstract

This document proposes 4 CCLM-related tests (CE3-2.5.1 – CE3-2.5.4) that reportedly show the following overall results: -0.09% (Y), -0.44% (U), -0.47% (V) and -0.08% (Y), -0.31% (U), -0.39% (V) for test CE3-2.5.1 for AI and RA configurations, respectively; -0.18% (Y), -0.88% (U), -0.90% (V) and -0.11% (Y), - 0.74% (U), -0.84% (V) for test CE3-2.5.2 for AI and RA configurations, respectively; 0.05% (Y), 0.45% (U), 0.48% (V) and 0.02% (Y), 0.51% (U), 0.51% (V) for test CE3-2.5.3 for AI and RA configurations, respectively; -0.14% (Y), -0.51% (U), -0.51% (V) and -0.09% (Y), - 0.41% (U), -0.46% (V) for test CE3-2.5.4 for AI and RA configurations, respectively. For tests CE3-2.5.1-CE3-2.5.2, minor reduction of running time is observed at the decoder side for both AI and RA configurations. The encoding time for all the test is 100% for both AI and RA configurations.

# Introduction

In JVET-L meeting, MaxMin method (JVET-L0191) for CCLM coefficients derivation was adopted, in which both the linear model coefficients α and β are derived by a linear equation directly. In this contribution, we propose a further improvements of CCLM.

# Proposal

In the proposed method, a new coefficients derivation method based on classification-based mean value is proposed. In the proposed method, the luma template samples are classified into two luma classes by the mean value of them. Correspondingly, two chroma classes are obtained. Then two luma mean values of the two luma classes, and two chroma mean values of the two chroma classes are obtained, respectively. The coefficients are derived based on the luma mean values and the chroma mean values.

Figure 1: Classification and mean value for CCLM mode

As shown in Figure 1, Lmean is the mean value of the luma template samples. Cmean is the mean value of the chroma template samples. The luma template samples are classified into two luma classes by Lmean. Then, mean values of the two luma classes LLmean and LRmean are obtained. Correspondingly, CLmean and CRmean are obtained. The CCLM coefficients are derived as:

α=CLmean-CRmeanLLmean-LRmean

β=Cmean-α*Lmean

In the existing method, to get the luma template samples, 2 rows and 3 columns of neighboring luma samples of current luma block are needed for down-sampling, except the CU at CTU top boundary. In the proposed method, only 1 row and 1 column of neighboring luma samples of current luma block are needed for all the CU. It means, only the first top row and the second left column will be used for down-sampling to get the luma template samples as shown in Figure 2.

Figure 2: Luma template samples (1 row, 1 column, with down-sampling).

As described in [1], 4 proposed combinations summarized in Table 1 were tested in this contribution.

Table 1. Summary of 4 proposed test

| 2.5.1 | CCLM + MDLM, using MaxMin method, and β derived by using Cmean and Lmean |
| --- | --- |
| 2.5.2 | CCLM + MDLM, using classification-based mean value method |
| 2.5.3 | CCLM + MDLM, using MaxMin method; using the first top row and the second left column |
| 2.5.4 | CCLM + MDLM, using classification-based mean value method; using the first top row and the second left column |

# Experiment Results

The results reportedly obtained for tests CE3-2.5.1 – CE3-2.5.4 using the common test conditions [2] are presented in Table 2-Table 5 for AI and RA configurations.

Table 2. Experimental results for test CE3-2.5.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.54% | -0.26% | 101% | 99% |
| Class A2 | -0.11% | -0.51% | -0.34% | 100% | 99% |
| Class B | -0.05% | -0.46% | -0.67% | 100% | 100% |
| Class C | -0.05% | -0.39% | -0.59% | 100% | 100% |
| Class E | -0.01% | -0.29% | -0.33% | 100% | 99% |
| Overall | -0.09% | -0.44% | -0.47% | 100% | 99% |
| Class D | -0.02% | -0.31% | -0.43% | 100% | 100% |
| Class F | -0.04% | -0.01% | 0.13% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.50% | -0.47% | 100% | 99% |
| Class A2 | -0.07% | -0.11% | -0.12% | 100% | 98% |
| Class B | -0.04% | -0.17% | -0.56% | 100% | 99% |
| Class C | -0.01% | -0.50% | -0.33% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.31% | -0.39% | 100% | 99% |
| Class D | 0.02% | -0.80% | -0.25% | 100% | 99% |
| Class F | -0.05% | -0.20% | -0.12% | 101% | 101% |

Table 3. Experimental results for test CE3-2.5.2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -1.10% | -0.86% | 100% | 99% |
| Class A2 | -0.21% | -0.96% | -0.53% | 100% | 99% |
| Class B | -0.11% | -1.01% | -1.28% | 100% | 99% |
| Class C | -0.14% | -0.79% | -0.94% | 100% | 100% |
| Class E | -0.08% | -0.45% | -0.62% | 100% | 99% |
| Overall | -0.18% | -0.88% | -0.90% | 100% | 99% |
| Class D | -0.07% | -0.61% | -0.60% | 100% | 100% |
| Class F | -0.13% | -0.56% | -0.07% | 102% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -1.06% | -1.16% | 100% | 99% |
| Class A2 | -0.10% | -0.65% | -0.22% | 100% | 99% |
| Class B | -0.04% | -0.74% | -1.29% | 100% | 99% |
| Class C | -0.02% | -0.57% | -0.51% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.74% | -0.84% | 100% | 99% |
| Class D | 0.03% | -1.01% | -0.41% | 99% | 99% |
| Class F | -0.05% | -0.28% | -0.08% | 101% | 101% |

Table 4. Experimental results for test CE3-2.5.3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 1.11% | 1.01% | 101% | 99% |
| Class A2 | 0.02% | 0.24% | 0.07% | 100% | 100% |
| Class B | 0.02% | 0.22% | 0.43% | 100% | 100% |
| Class C | 0.10% | 0.68% | 0.61% | 100% | 100% |
| Class E | 0.02% | 0.07% | 0.25% | 100% | 100% |
| Overall | 0.05% | 0.45% | 0.48% | 100% | 100% |
| Class D | 0.06% | 0.15% | 0.22% | 100% | 100% |
| Class F | 0.15% | 0.68% | 0.91% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.76% | 0.59% | 100% | 99% |
| Class A2 | -0.02% | 0.32% | 0.17% | 100% | 99% |
| Class B | 0.00% | 0.53% | 0.41% | 100% | 99% |
| Class C | 0.07% | 0.46% | 0.85% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.51% | 0.51% | 100% | 99% |
| Class D | 0.02% | 0.04% | 0.35% | 99% | 99% |
| Class F | 0.07% | 0.48% | 0.38% | 101% | 101% |

Table 5. Experimental results for test CE3-2.5.4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | 0.20% | 0.10% | 100% | 102% |
| Class A2 | -0.20% | -0.74% | -0.47% | 100% | 101% |
| Class B | -0.10% | -0.79% | -0.87% | 100% | 102% |
| Class C | -0.07% | -0.50% | -0.54% | 100% | 100% |
| Class E | -0.06% | -0.53% | -0.54% | 100% | 101% |
| Overall | -0.14% | -0.51% | -0.51% | 100% | 101% |
| Class D | -0.04% | -0.39% | -0.40% | 100% | 100% |
| Class F | -0.03% | -0.09% | 0.33% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.37% | -0.72% | 100% | 99% |
| Class A2 | -0.06% | -0.50% | -0.15% | 100% | 99% |
| Class B | -0.04% | -0.52% | -0.80% | 100% | 99% |
| Class C | -0.01% | -0.23% | -0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.41% | -0.46% | 100% | 99% |
| Class D | -0.02% | -0.54% | -0.73% | 100% | 99% |
| Class F | -0.02% | 0.00% | 0.10% | 101% | 101% |

# Conclusion

We propose to adopt this proposal into VTM and the VVC spec draft.

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, JVET-L1023, 12th JVET Meeting, Macao, China, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th JVET Meeting, Macao, China, October 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).