| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0403 |
| --- | --- |

| Title: | CE4: Generic Vector Coding of Motion Vector Difference (Tests 4.4.1.a and 4.4.1.b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Mehdi Salehifar Seung-Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email: | 1-858-635-5300 seethal.paluri@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

Abstract

This contribution presents the simulation results of the two CE tests for Generic Vector Coding technique for motion vector difference (MVD) (CE4, Tests 4.4.1.a and 4.4.1.b). Modifications to the algorithm have been carried out to reduce the worst case use of context coded bins from 24 to 6 by coding MVDx and MVDy components together. MVD (x,y) is coded using a combination of Layer and index information, wherein the index identifies the MVD(x,y) combination within a group of MVDs (i.e., a Layer). Both the tests differ in their groupings of the layers. In the former, all the layers greater than 0 are grouped together, while in the later test, layers greater than or equal to 3 are grouped together while layers 1 and 2 are grouped separately. The BD-rate changes for RA and LDB are reported for CE4.4.1.a and CE4.4.1.b as (0.01/0.02/0.02), (0.03/-0.09/-0.11) and (0.00/0.04/0.02), (-0.01/0.01/-0.02) respectively.

# Background

The existing coding of MVD x-y components rely on coding the x and y components independently. Decoding of each MVD component is done by first checking if the received MVD component is greater than zero and if true, if it is greater than one. It should be noted that these two flags are context coded bins. Following this, the MVD component’s absolute value is decoded and the sign information is also decoded. The process is then repeated for the second MVD component. Therefore, a decoded block uses 4 context coded bins during decoding, which can increase the computational complexity. Further to this the sign bit needs to also be coded. In a scenario where a 6-parameter Affine-Bi-directional MVDs are to be coded, 24 context coded bins are to be used.

# Proposed method

## Layer and index concept

Figure 1.0 below shows the results achieved by grouping MVD(x,y) values with similar frequency distributions. The center cell represents a MVD(x,y) combination of (0,0), i.e., MVDx = 0 and MVDy = 0. The center point (0,0) also corresponds to the most frequently occuring MVD combination in the data set. Further to this, the blocks adjacent to the center point correspond to either an increment or decrement of the MVD values along the horizontal or vertical axes. Frequency analysis of the data suggests, that certain combinations of MVD values occur with similar probabilities such that it can be combined into groups. Consequently, we define a Layer to represent MVD combinations that occur with similar probabilities. The figure below can also be expanded to incorporate several other layers. We define groups of MVD combinations that occur with similar likelihood as a Layer. Figure 1.0 illustrates a rhombus like structure with five layers. As the Layers grow, so too does the magnitude of the MVDs. Additionally, within each Layer, an index identifies the MVD combination being considered. The proposed scheme codes, the Layer and subsequently its index to indicate the combination of the MVD(x,y) being used. This proposal has advantages over the traditional MVD coding technique, as the proposed scheme (i) codes both the MVD x and y components together, (ii) sign information is not sent separately as it is deciphered using the Layer and index, and (iii) uses fewer context bins.

Figure 1.0: Shows the MVD(x,y) Layers.

Figures 2.0 (a) below shows Layers 0 and 1 and Figure 2.0 (b) shows the characteristics of MVD in each quadrant. It should be noted that while the horizontal axis represents MVD x and the vertical axis represents the MVD y components, the proposed scheme uses a flipped vertical axis to lower the computational complexity of the indexing mechanism.

Figures 2.0 (a –b): Show ths specifics of the MVD characteristics.

Additionally, the Figure below shows the MVD combinations that belong to Layer 2.

Figure 3.0: Shows the the index combinations in Layer 2.

The figure above demonstrates the MVD indices of Layer 2. Each point on the MVD x-y grid represents an MVD increment or decrement of one. For example, the index 0 in Layer 2 represents an MVD(x,y) combination of (0,2), an index of 1 represents an MVD values of (1,-1) and so on. It should be noted that due to the symmerty of the structure, indices 1 and 7; 6 and 2; 5 and 3 differ only in their horizontal MVD component.

## Proposed Coding Structures

### Test 4.1.1.a

Figure 4.0 Shows the proposed MVD decoder stucture.

The figure above shows a new structure for MVD decoding. Here, a flag i.e., is Layer greater than zero (GRT0) is used. This flag is context coded and is first parsed. With 1 context coded bin for each MVD set, in the worst case scenario where it is a 6-parameter Affine Bi-directional case would have 6 sets of MVD and with each set using only 1 context coded bin. Therefore, the worst case scenario would use 6 context coded bins. The status of the flag is first checked. If the value of the flag is false, this implies that the MVD(x,y) values are (0,0). If the flag is indeed true, then it implies that, the Layer is greater than zero with a minimum value of 1. Following this, the layer information is parsed and decoded. Exponential Golomb Codes with Golomb parameter equal to 1 are used during this decoding. Following this, the horizontal and vertical MVD that corresponds to the Decode MVD(x,y) values in the figure above are deciphered as follows:

MVDx=(idx≤Lyr ?idx :(idx≤Lyr≪1+Lyr) ?Lyr≪1-idx :idx-(Lyr≪2))

MVDy =(idx≤Lyr≪1 ?Lyr- idx :idx- Lyr≪1+Lyr)

Eqns (1-2)

### Test 4.4.1.b

In this structure, we present another possible approach toward MVD decoding. This method relies on grouping some layers together while other layers may be grouped together. The figure below illustrates this approach to this technique.

Figure 5.0 Shows an MVD decoder structure.

Here, a context coded flag is first parsed. This flag can assume the states of true or false and is used to indicate if the Layer is greater than zero (i.e., Layer GRT0). If this flag is false, then it implies that the MVD values in the horizontal and vertical directions are both (0,0). If this flag is true, then another equiprobable bin is used to decipher layers that have been grouped together. Here, as a case in point, Layers 1 and 2 may be grouped together while the remaining layers may be grouped together. In order to differentiate between Layers 1 and 2, an additional equiprobable bin is used. Finally, the MVD information can be infered once the Layer and Index information is decoded. If the Layer is not 1 or 2, then the minimum Layer under consideration is 3. Here, Exponential Golomb codes with parameter 1 are used.

The following equations below may be used:

MVD1(x,y) may be deciphered as follows

MVDx=idx≤Lyr?idx :Lyr≪1-idx

MVDy=(idx≤Lyr?  !idx :(idx==2 ? -1 :0))

Eqns.3-4

MVD2(x,y) and MVD3(x,y) may be deciphered as follows:

MVDx=(idx≤Lyr?idx :(idx≤(Lyr≪1+Lyr ?Lyr≪1-idx :idx-Lyr≪2 ))

MVDy=idx≤Lyr≪1?  Lyr-idx :idx-(Lyr≪1+Lyr))

Eqns. 5-6

# 3	Experimental results

The proposed method has been implemented on VTM 3.0 reference software and evaluated experimentally according to common test conditions defined in JVET-J1010 [1]. The results were generated for all the sequences.

Results for Test 4.4.1.a are as follows:

Table 1: Experimental results for random access test condition; anchor is VTM 3.0.

|  | Over VTM 3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01 | -0.14 | 0.05 | 100 | 99 |
| Class A2 | 0.04 | 0.08 | 0.06 | 100 | 99 |
| Class B | 0.00 | 0.08 | -0.04 | 100 | 103 |
| Class C | -0.01 | 0.03 | 0.03 | 100 | 101 |
| Class E |  |  |  |  |  |
| Overall | 0.01 | 0.02 | 0.02 | 100 | 101 |
| Class D | 0.07 | 0.06 | -0.06 | 100 | 99 |

Table 2: Experimental results for low delay B test condition; anchor is VTM 3.0.

|  | Over VTM 3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02 | -0.19 | -0.06 | 100 | 99 |
| Class C | 0.04 | 0.14 | 0.20 | 100 | 101 |
| Class E |  |  |  |  |  |
| Overall | 0.03 | -0.09 | -0.11 | 100 | 100 |
| Class D | 0.15 | 0.28 | 0.39 | 99 | 103 |

Results for Test 4.4.1.b are as follows:

Table 3: Experimental results for random access test condition; anchor is VTM 3.0.

|  | Over VTM 3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02 | -0.02 | -0.01 | 100 | 98 |
| Class A2 | 0.04 | 0.11 | 0.07 | 100 | 98 |
| Class B | -0.02 | 0.11 | -0.03 | 101 | 103 |
| Class C | -0.01 | -0.03 | 0.09 | 101 | 101 |
| Class E |  |  |  |  |  |
| Overall | 0.00 | 0.04 | 0.02 | 100 | 101 |
| Class D | 0.00 | -0.08 | -0.05 | 101 | 102 |

Table 4: Experimental results for low delay B test condition; anchor is VTM 3.0.

|  | Over VTM 3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00 | -0.15 | 0.02 | 99 | 96 |
| Class C | -0.02 | 0.20 | 0.17 | 101 | 100 |
| Class E |  |  |  |  |  |
| Overall | -0.01 | 0.01 | -0.02 | 100 | 99 |
| Class D | 0.03 | -0.26 | -0.10 | 101 | 107 |

# Specification Text Changes

The specification table shown below is to replace the existing Motion vector difference syntax table. The table below demonstrates the changes for Test 4.4.1.a.

7.3.4.9 Motion vector difference syntax

| mvd_coding( x0, y0, refList ,cpIdx ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag | ae(v) |
| if( abs_mvd_greater0_flag ) { |  |
| mvd_layer_minus1 | ae(v) |
| mvd_idx | tb(v) |
| } |  |
| } |  |

7.4.5.9 Motion vector symantics

abs_mvd_greater0_flag equals to 1 specifies that the sum of the absolute values of the x and y components of the motion vector difference is greater than 0.

mvd_layer_minus1 plus 1 specifies the absolute value of layer. The value of mvd_layer_minus1 shall be in the range of 0 to maxLayer inclusive.

When mvd_layer_minus1 is not present, it is inferred to be equal to 0.

The maximum value of maxLayer shall be in the range of 0 to 216 + 2, inclusive.

mvd_idx specifies the index of the signalled MVD within a mvd_layer_minus1. When mvd_idx is not present it is inferred to be equal to 0.

The maximum value cMax of the truncated binary binarization tb(v) is set to (4* mvd_layer_minus1).

The horizontal and vertical MVD values, i.e., (horMvd and verMvd) are derived as follows:

horMvd = (mvd_idx <= mvd_layer_minus1 ? mvd_idx : (mvd_idx <= ((mvd_layer_minus1 << 1) + mvd_layer_minus1) ? (mvd_later_minus1 << 1) –mvd_idx : mvd_idx –mvd_layer_minus1  <<2)))

verMvd = ((mvd_idx <= (mvd_layer_minus1<<1)) ? mvd_layer_minus1 –mvd_idx : mvd_idx – ((mvd_layer_minus1 << 1) + mvd_layer))

Table 9-4 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |
|  | mvd_layer_minus1 | EG1 | - |
|  | mvd_idx | TB | cMax = ( 4*mvd_layer_minus1) |

Table 9-10 Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| abs_mvd_greater0_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_minus2[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |
| mvd_layer_minus1 | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_idx | 0 | bypass | bypass | bypass | bypass | na |

# Conclusion

We observed that the proposed method is a simplified and logical way to combine the MVD coding of x and y components and it reduces the number of context coded bins used from 24 to just 6 in the worst case scenario. Also, the sign information need not be coded. We recommend that the proposed Generic Vector Coding for Motion Vector Difference be adopted.

# Reference

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010, 12th Meeting: Macao, CN, 08–12 Oct. 2018.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).