| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0409-v1 |
| --- | --- |

| Title: | Non-CE8: Mismatch between text specification and reference software on ATMVP candidate derivation when CPR is enabled |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li and Shan Liu (Tencent) Wei-Jung Chien and Marta Karczewicz (Qualcomm) | Tel: Email: | xiaozhongxu@tencent.com wchien@qti.qualcomm.com |
| Source: | Tencent, Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a specification aligned CPR software implementation regarding ATMVP CU level MV offset derivation. In a previous VVC specification, when performing alternative temporal motion vector prediction (ATMVP), a motion offset at CU level is obtained from the motion vector of the first available spatial neighboring merge candidate that has the collocated picture as its reference picture. In BMS2.1, if a neighboring merge candidate is coded in CPR mode, it is considered as not available. This implementation is inherited into VTM3.0 as part of CPR mode. In the current VVC specification, only the first available spatial candidate in the merge list will be checked. If the candidate doesn’t satisfy the condition of ATMVP neighboring block requirement, zero motion vector will be used to derive the collocated block in the collocated picture. Therefore, there is a mismatch between the specification and software implementation. Simulation results report that BD rate changes are xxx%/xxx%/ xxx% in AI/RA/LB configurations, respectively, when the provided specification aligned implementation is compared to VTM-3.0 anchor (CPR=1).

# Introduction

In current VVC WD [1], CPR mode is enabled by using the current picture is as a reference picture. When deriving the CU level motion vector offset for ATMVP mode, the following apply:

In current VTM-3.0 implementation, the equivalent specification text will be:

The variable availableFlagN is set equal to FALSE, and the following applies:

- When availableFlagA0 is equal to 1 and the reference picture refIdxLXA0 corresponding to is not current picture, the following applies:

  - availableFlagN is set equal to TRUE,

  - refIdxLXN is set equal to refIdxLXA0 and mvLXN is set equal to mvLXA0, for X being replaced by 0 and 1.

- When availableFlagN is equal to FALSE and availableFlagLB0 is equal to 1 and the reference picture refIdxLXB0 corresponding to is not current picture, the following applies:

  - availableFlagN is set equal to TRUE,

  - refIdxLXN is set equal to refIdxLXB0 and mvLXN is set equal to mvLXB0, for X being replaced by 0 and 1.

- When availableFlagN is equal to FALSE and availableFlagB1 is equal to 1 and the reference picture refIdxLXB1 corresponding to is not current picture, the following applies:

  - availableFlagN is set equal to TRUE.

  - refIdxLXN is set equal to refIdxLXB1 and mvLXN is set equal to mvLXB1, for X being replaced by 0 and 1.

- When availableFlagN is equal to FALSE and availableFlagA1is equal to 1 and the reference picture refIdxLXA1 corresponding to is not current picture, the following applies:

  - availableFlagN is set equal to TRUE.

  - refIdxLXN is set equal to refIdxLXA1 and mvLXN is set equal to mvLXA1, for X being replaced by 0 and 1.

When availableFlagN is equal to TRUE, the following applies:

- If all of the following conditions are true, tempMV is set equal to mvL1N:

  - predFlagL1N is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList1[refIdxL1N]) is equal to 0,

  - DiffPicOrderCnt(aPic, currPic) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice,

  - slice_type is equal to B,

  - collocated_from_l0_flag is equal to 0.

- Otherwise if all of the following conditions are true, tempMV is set equal to mvL0N:

  - predFlagL0N is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList0[refIdxL0N]) is equal to 0.

# Proposed Solutions

In this proposal, a current specification aligned implementation on top of VTM-3.0 is provided as follows.

The variable availableFlagN is set equal to FALSE, and the following applies:

- When availableFlagA0 is equal to 1 and the reference picture refIdxLXA0 corresponding to is not current picture, the following applies:

  - availableFlagN is set equal to TRUE,

  - refIdxLXN is set equal to refIdxLXA0 and mvLXN is set equal to mvLXA0, for X being replaced by 0 and 1.

- When availableFlagN is equal to FALSE and availableFlagLB0 is equal to 1 and the reference picture refIdxLXB0 corresponding to is not current picture, the following applies:

  - availableFlagN is set equal to TRUE,

  - refIdxLXN is set equal to refIdxLXB0 and mvLXN is set equal to mvLXB0, for X being replaced by 0 and 1.

- When availableFlagN is equal to FALSE and availableFlagB1 is equal to 1 and the reference picture refIdxLXB1 corresponding to is not current picture, the following applies:

  - availableFlagN is set equal to TRUE.

  - refIdxLXN is set equal to refIdxLXB1 and mvLXN is set equal to mvLXB1, for X being replaced by 0 and 1.

- When availableFlagN is equal to FALSE and availableFlagA1is equal to 1 and the reference picture refIdxLXA1 corresponding to is not current picture, the following applies:

  - availableFlagN is set equal to TRUE.

  - refIdxLXN is set equal to refIdxLXA1 and mvLXN is set equal to mvLXA1, for X being replaced by 0 and 1.

When availableFlagN is equal to TRUE, the following applies:

- If all of the following conditions are true, tempMV is set equal to mvL1N:

  - predFlagL1N is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList1[refIdxL1N]) is equal to 0,

  - DiffPicOrderCnt(aPic, currPic) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice,

  - slice_type is equal to B,

  - collocated_from_l0_flag is equal to 0.

- Otherwise if all of the following conditions are true, tempMV is set equal to mvL0N:

  - predFlagL0N is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList0[refIdxL0N]) is equal to 0.

# Experimental results

Tests are carried out by using the VTM3.0 software, and the common test conditions (CTC) defined in [2] and common test conditions in CE8 [3] are followed.

Table 1: Summary results of proposed method vs. VTM3.0+CPR=0 and VTM3.0+CPR=1

# Recommendation

In this contribution, it is proposed to align the software implementation of AMTVP motion vector offset derivation to that is described in the specification, when CPR mode is used.

The authors would like to thank Alibaba for cross-checking.

# References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, the 12th JVET meeting, the 12th JVET meeting, Oct. 2018, Macao, CN.

[2] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

[3] X. Xu, Y.-C. Sun, Y.-H. Chao, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding”, JVET-L1028, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).